3GPP TSG RAN WG1 Meeting #84bis                                                                   R1-162989

Busan, Korea 11th - 15th April 2016

Source:	ITRI, MediaTek Inc.

Title:	38GHz Measurement Results for UMi Street Canyon Scenario

Agenda Item:	8.2.1

Document for: 	Discussion and Decision

Introduction

In RAN1 #AH Channel Model, the 3GPP compatible LOS pathloss model for UMa, UMi street canyon, InH office and InH shopping mall scenarios are agreed as working assumption in the following table [1]. For NLOS pathloss model, the decision of ABG or CI/CIF models will be further discussed at the RAN1 #84bis meeting.

| Scenario | Pathloss [dB], f is in GHz and d is in meters | Shadow fading std [dB]7) | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.1 σSF=4.1 | 10m < d2D < d‘BP 1) d'BP < d2D <5000m |
| UMi - Street Canyon LOS |  | σSF=3.76 σSF=3.76 | 10m < d2D < d‘BP 1) d'BP < d2D <5000m |
| InH - Office LOS |  | σSF=3.02 | 1-73m |
| InH - Shopping mall LOS |  | σSF=2.01 | 0.5-149m |

In this contribution, we update and modify the CI and ABG modeling parameters in our RAN1#84 contribution [2]. In addition, we have compared the current working assumptions of LOS/NLOS pathloss models with our 38GHz measurement data set.  The object of this contribution is to show the differences between our data fitted model and the working assumption model.

Measurement Scenario

In this contribution, we provide the 38GHz LOS/NLOS channel measurement results for street canyon scenario. The inter-site distance for street canyon is typically less than 200 m. The cell radii is 100 m and base stations (BSs) are usually mounted below rooftops at a height of 7-20 m. The height of outdoor UEs is 1.5-2.5 m.

Our 38GHz channel measurement campaign locates at the street between the second engineering and apprentice buildings in Taiwan national Chung-Cheng University (CCU). Such allocation of buildings and street is similar to UMi street canyon environment. Fig. 1 shows CCU campus map and the relative locations of transmitter and receiver. In this measurement campaign, the transmitter locates at Tx1, Tx2 and Tx3 with heights of 11.6 m, 7 m and 7 m. The corresponding receiver locations of Tx1 is Rx16 to Rx25, Tx2 is Rx1 to Rx9 and Tx3 is Rx10 to Rx15. Table 2 in Annex describes all the distances of Tx/Rx and the corresponding LOS/NLOS conditions.

Figure 1: CCU campus map for 38GHz UMi channel measurement and the relative locations of Tx and Rx.

38GHz Measurement Results

Path Loss and Shadow Fading

The measurement results of large scale parameter include path loss (PL) and shadow fading (SF). Two common PL models are analyzed in this contribution, i.e., Alpha-Beta-Gamma (ABG) and Close-in (CI) free space reference distance models. The CI PL model is given as:

PLddB=FSPLd0+10nlog10dd0+Xσ,

where d0 is the free space reference distance, n is the path loss exponent (PLE) and Xσ is the SF term in dB. The FSPL of d0 can be given by

PLdBd0=20log10(4πd0λ),

where λ is the wave length. The reference distance d0 in the following analysis is 1m.

The ABG PL model is given as:

PLdB=10∙αlog10d+β+10∙γlog10f+Xσ,

where α represents the PL increasing slope as Tx-Rx distance increases, β is the floating offset value in dB, γ induces the frequency dependent PL. Currently, the PL model adopted in 3GPP 3D-SCM is a modified ABG model with dual-slope to fit the near/far PLs. Based on the measurement results, the parameters of both CI and ABG models are summarized in Table 1.

Table 1 also summarizes the PL parameters in working assumption WF [1] and the revised 5G channel model [3].  It can be seen that the PLE of LOS is 2.29 that is close to the theoretical free space value. For LOS case, the CI PLE provided by [3] is 2.1, which is identical to the ABG α value in [1]. Thus in Fig. 2, the curve of CI model in [3] coincides with ABG model in [1]. Fig. 2 also shows the 38GHz LOS measurement data and its corresponding fitted models (CI and ABG models). The slope of CI model in [1] is very close to the measurement model.

For NLOS case, the PLE of CI model [3] is 3.17 which is also close to our curve fitting result of 3.09. The 38GHz NLOS measurement data and its corresponding fitted models (CI and ABG models) are shown in Figure 3. Within the range of data set, the ABG model [3] is closer to our data fitted models compared with the CI model [3]. Since the values of α and β are get by the measurement data, ABG model could provide more accurate within the range of measurement data.

Based the measurement results and observations, we have the following proposal:

Proposal 1: For LOS case, the working assumption model [1] is quite close to our 38GHz data fitted model.

Proposal 2: For NLOS case, the ABG model can provide better modeling accuracy than CI model.

Proposal 3: The measurement results and fitted parameters can be captured in the TR.

Table 1. CI and ABG PL Model Parameters

| Scenario | LOS/NLOS | Model | Source | PLE/ | [dB] |  | Xσ [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UMi Street Canyon | LOS | CI | This work | 2.29 | - | - | 2.3 |
|  |  |  | [3] | 2.1 | - | - | 3.76 |
|  |  | ABG | This work | 2.36 | 62.78 | - | 2.92 |
|  |  |  | R1-161744 [1] | 2.1 | 32.4 | 2 | 3.76 |
|  | NLOS | CI | This work | 3.09 | - | - | 5.57 |
|  |  |  | [3] | 3.17 | - | - | 8.09 |
|  |  | ABG | This work | 4.02 | 48.5 | - | 4.3 |
|  |  |  | [3] | 3.53 | 22.4 | 2.13 | 7.82 |

Figure 2. LOS measurement results of 38 GHz pathloss for UMi street canyon scenario

Figure 3. NLOS measurement results of 38 GHz pathloss for UMi street canyon scenario

Summary

In this contribution, we present the 38GHz channel measurement results for the scenario of UMi street canyon. Based on the results and observations, we made the following proposals.

Proposal 1: For LOS case, the working assumption model [1] is quite close to our 38GHz data fitted model.

Proposal 2: For NLOS case, the ABG model can provide better modeling accuracy than CI model.

Proposal 3: The measurement results and fitted parameters can be captured in the TR.

- References

- 3GPP, R1-161744, “WF on LOS Pathloss Modeling”, Huawei, HiSilicon, Samsung, Ericsson, DOCOMO, CMCC, Keysight, Fraunhofer HHI, Ljubljana, Slovenia, 14-16 March, 2016

- 3GPP, RP-160662, “38GHz Measurement Results of UMi Scenario”, ITRI, Malta, Feb. 14-18, 2016.

- Aalto University, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz – Revised version”, Mar. 2016 (http://www.5gworkshops.com/5GCM.html).

- Acknowledgment

ITRI would like to thank National Chung-Cheng University (CCU) for their support of the channel measurement campaign.

- Annex

Table 2. Distance and LOS/NLOS condition of Tx-Rx Links

| Transmitter | Receiver | Distance | LOS/NLOS |
| --- | --- | --- | --- |
| Tx1 | Rx16 | 62.27 | LOS |
|  | Rx17 | 71.96 | LOS |
|  | Rx18 | 81.72 | LOS |
|  | Rx19 | 91.53 | LOS |
|  | Rx20 | 101.38 | LOS |
|  | Rx21 | 111.25 | LOS |
|  | Rx22 | 131.06 | LOS |
|  | Rx23 | 52.71 | LOS |
|  | Rx24 | 81.72 | LOS |
|  | Rx25 | 101.38 | LOS |
| Tx2 | Rx1 | 49.26 | LOS |
|  | Rx2 | 39.83 | LOS |
|  | Rx3 | 30.77 | LOS |
|  | Rx4 | 27.89 | NLOS |
|  | Rx5 | 37.65 | NLOS |
|  | Rx6 | 52.29 | NLOS |
|  | Rx7 | 57.45 | NLOS |
|  | Rx8 | 58.57 | NLOS |
|  | Rx9 | 53.52 | NLOS |
| Tx3 | Rx10 | 53.52 | NLOS |
|  | Rx11 | 58.57 | NLOS |
|  | Rx12 | 57.45 | NLOS |
|  | Rx13 | 52.29 | NLOS |
|  | Rx14 | 39.84 | NLOS |
|  | Rx15 | 49.26 | NLOS |