3GPPRAN1#84-BIS	R1-163017

Busan, Korea

11th - 15th April 2016

Agenda item:    7.2.1.2.3

Source:             Qualcomm Incorporated

Title:                  Remaining details of UL control design

Document for:  Discussion and Decision

- Introduction

In RAN1 adhoc meeting #2 on NB-IOT, the following agreement has been made:

- For ACK/NACK only transmission of NB-PDSCH in NB-IoT:

  - Only single tone transmission is supported

- For ACK/NACK only transmission of NB-PDSCH in NB-IoT:

  - ∏ /2-BPSK modulation is used for single tone transmission

- Number of repetitions of A/N resource unit is semi-statically configured by RRC signaling at least if the associated NB-PDSCH is after Msg4

In addition, we have the following working assumption.

- Support 3 DM-RS symbols per 7 symbol period

- The DM-RS sequence is obtained as

  - Use  single tone DM-RS PUSCH spread by length 3 OCC sequence defined for PUCCH

    - The OCC ID is pseudo-randomly selected (from existing OCCs) according to

In this contribution, we provide our views on PUCCH design for NB-IOT.

- UL ACK channel design

For UL ACK channel, 3 DM-RS symbol is current working assumption. We have run simulations to confirm that 3 DM-RS design gives significant performance benefit compared to 1 DM-RS as shown in Figure 1. to Figure 3. In all simulations, TU1 channel model is used and DTX to ACK error rate is fixed at 10^-2.  Figure 1 shows the UE with MCL 164dB with different bundling size. It can be observed that to reach the same performance target, roughly doubled bundling size is required for 1 DM-RS. With 3 DM-RS we can save half of the resources. Figure 2. Shows that when there is other cell PUSCH random data channel interference fixed at 1dB SNR, then close to 5dB performance gain is achieved with 3 DM-RS, which translated to at least 2/3 of resource savings as shown in Figure 3. These simulations show significant gain of 3 DM-RS for ACK/NACK channel. We therefore propose to confirm the working assumption:

Figure 1.  UE with MCL 164dB in TU1 channel

Figure 2. UE with bundling size 100 subframes and varied SNR in TU1 Channel with inter-cell interferer fixed at 1dB SNR

Figure 3. UE with MCL 164dB and varied bundling size with interferer fixed at 1dB SNR

Proposal 1: Confirm the working assumption:  Support 3 DM-RS symbols per 7 symbol period for ACK/NACK channel

Furthermore, a similar spreading for data symbols of ACK/NACK channel should be applied similar to DM-RS symbols to suppress inter-cell interference. We therefore have the following proposals:

Proposal 2: Use length 4 OCC sequence defined for PUCCH on data symbols of ACK/NACK channel

- The OCC ID is pseudo-randomly selected (from existing OCCs) according to

Figure 4. HARQ-ACK structure with slot-based frequency hopping

Proposal 3: Consider slot-based frequency hopping within the narrow band for UL ACK/NACK channel transmission to allow time tracking.

- Summary

In this contribution we presented our views on the PUCCH design. We make the following proposals

Proposal 1: Confirm the working assumption:  Support 3 DM-RS symbols per 7 symbol period for ACK/NACK channel

Proposal 2: Use length 4 OCC sequence defined for PUCCH on data symbols of ACK/NACK channel

- The OCC ID is pseudo-randomly selected (from existing OCCs) according to

Proposal 3: Consider slot-based frequency hopping within the narrow band for UL ACK/NACK channel transmission to allow time tracking.

References

[1]	GP-151621, “New Work Item: NarrowBand IOT (NB-IOT),” source Qualcomm Inc., RAN #69.

[2]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418),” source VODAFONE Group Plc. GERAN#62.

[3]	3GPP TR 45.820, “Technical Specification Group GSM/EDGE Radio Access Network; Cellular System; Support for Ultra Low Complexity and Low Throughput Internet of Things; (Release 13),” www.3gpp.org.