3GPP TSG RAN WG1 Meeting #84bis		R1-163077

Busan, Korea 11th - 15th April 2016

Agenda item:	8.2.3.2

Source:	InterDigital, Intel Corporation

Title:	An Alternative Interpolation Formula for Spatial Consistency

Document for:		Discussion and Decision

# 1	Introduction

A list of additional features for above 6 GHz channel model was identified and agreed in RAN1 Ad Hoc meeting on channel model [1]. One of the additional features is spatial consistency. There are three alternative proposals on modeling spatial consistency [2], where the first proposal was decided as working assumption [3]. This proposal is based on spatially consistent random variables. Specifically, the spatially consistent random variables will be applied in generating small scale parameters, as well as LOS/NLOS and indoor/outdoor states, for nearby UE’s.

There are many methods of generating spatially consistent random variables. In summary, these methods could be categorized by three types: 1). Interpolation of independently and identically distributed (i.i.d.) random variables (e.g., variant 1 in [2], [4], [5]); 2). Exponential decay filtering (e.g., [6]); 3). Mixture of interpolation and exponential decay filtering (e.g., variant 2 in [2]).

It was pointed out in [5] that within the “Interpolation of i.i.d. random variables” category, the proposed interpolation formulas still have the issue of not satisfying the correlation property.

This contribution proposes an alternative interpolation formula, which is shown to satisfy the 1-dimensional correlation property within the interpolation area.

# 2     Discussion

The method of generating spatially consistent random variables by interpolating independent random variables is as shown in Figure 1. Let Y0,0, Y0,1,Y1,0,Y1,1 be i.i.d. random variables. These values are assigned to the four corner points in the figure. The distance between two nearby corner points is the correlation distance dcorr. For any point (x,y) within the rectangular area, its corresponding value Yx,y is a function of Y0,0, Y0,1,Y1,0,Y1,1.

In [2], the random variables Y0,0, Y0,1,Y1,0,Y1,1 are Gaussian distributed and the formula to determine Yx,y is given by

|  | Yx,y=1-y/dcorr1-x/dcorrY0,0+x/dcorrY1,0+y/dcorr1-x/dcorrY0,1+x/dcorrY1,1. | (1) |
| --- | --- | --- |

It is shown in [5] that for the 1-dimensional example of Figure 2, the correlation property is not satisfied when using the interpolation formula (1), i.e.,

|  | E[A∙C]≠E[C∙D]. | (2) |
| --- | --- | --- |

Though an alternative formula is provided in [5], it does not satisfy the correlation property in 2-dimensional domain. A similar conclusion can be obtained for the formula in [4].

Figure 1: Example of generating one spatially consistent random variable [2]

Figure 2: A one dimensional example of correlation between locations within a grid

In this contribution, we are proposing an alternative formula to determine Yx,y by Y0,0, Y0,1,Y1,0,Y1,1. The formula is as follows:

|  | Yx,y=cosπ∙y2∙dcorrcosπ∙x2∙dcorrY0,0+sinπ∙x2∙dcorrY1,0+sinπ∙y2∙dcorrcosπ∙x2∙dcorrY0,1+sinπ∙x2∙dcorrY1,1⁡. | (3) |
| --- | --- | --- |

The random values generated by this formula satisfy the 1-dimensional correlation property within the rectangular interpolation area. The correlation error in 2-dimensional domain is limited by 0.1971. The detailed analysis is provided in Appendix. Consider the same example as in Figure 2. It can be derived from the interpolation formula (3) that

EA∙C=E[Y0,02]∙cosπ6,

EC∙D=E[Y0,02]∙cosπ3cosπ6+sinπ3sin⁡(π6).

It follows from the property cosx-y=cosxcosy+sinxsiny that the correlation property (2) holds.

In Figure 3, we show the simulation results on the spatially consistent LOS/NLOS states. In the simulations, we consider a 400 meters by 400 meters area and the BS is placed in the center of the area. The correlation distance is chosen to be 25 meters. The left figure shows the LOS/NLOS state resulting from the interpolation formula (1), while the right figure shows the LOS/NLOS state resulting from the interpolation formula (3). Similar spatial consistency scenario is observed from both figures.

Due to the retention of the 1-dimensional correlation property and small errors in 2-dimensional correlation, we propose to use the interpolation formula (3), in case the method of interpolating independent random variables is adopted.

Proposal 1: If the interpolation of i.i.d. random variables is to be used for spatial consistency, then consider the interpolation formula in (3).

Figure 3: Left: Spatially consistent LOS/NLOS with interpolation formula in (1); Right: Spatially consistent LOS/NLOS with interpolation formula in (3).

# 3	Conclusion

In this contribution, we present an alternative interpolation formula to be used for spatial consistency. Our proposal is as follows:

Proposal 1: If the interpolation of i.i.d. random variables is to be used for spatial consistency, then consider the interpolation formula in (3).

# References

- 3GPP, R1-161631, NEC, “Refinement on above 6GHz channel modelling,” Mar. 2016.

- 3GPP, R1-161622, Intel, et.al., “Spatial consistency modeling in drop based model,” Mar. 2016.

- 3GPP, R1-161726, “WF on spatial consistency,” Mar. 2016.

- 3GPP, R1-161646, ZTE, “To maintain spatial consistency in LoS/NLoS decision,” Mar. 2016.

- 3GPP, R1-161708, Samsung, “Discussion on spatial consistency,” Mar. 2016.

- 3GPP, R1-161651, InterDigital, “Spatial consistency for LOS/NLSO state,” Mar. 2016.

- 3GPP, TR36.873 (v12.2.0), “Study on 3D channel model for LTE,” July 2015.

# Appendix: Performance Analysis of the Interpolation Formula

Consider a point (x,y) in the square area of Figure 1. Suppose xdcorr=i and ydcorr=j. Then we have

Yx,y=cosπ∙j2cosπ∙i2Y0,0+sinπ∙i2Y1,0+sinπ∙j2cosπ∙i2Y0,1+sinπ∙i2Y1,1.

Similarly, for a different point (x',y') in the square area with x'dcorr=i' and y'dcorr=j', we have

Yx',y'=cosπ∙j'2cosπ∙i'2Y0,0+sinπ∙i'2Y1,0+sinπ∙j'2cosπ∙i'2Y0,1+sinπ∙i'2Y1,1.

Then, the correlation between these two points is calculated as

EYx,y∙Yx',y' =EY0,02∙cosπ∙j2cosπ∙i2cosπ∙j'2cosπ∙i'2+cosπ∙j2sinπ∙i2cosπ∙j'2sinπ∙i'2+sinπ∙j2cosπ∙i2sinπ∙j'2cosπ∙i'2+sinπ∙j2sinπ∙i2sinπ∙j'2sinπ∙i'2
=EY0,02cosπ∙(i'-i)2cosπ∙(j'-j)2.

It is clear that for 1-dimensional condition, i.e., either i=i' or j=j', the correlation depends only on the distance between two points, rather than their locations. Hence, the correlation property is satisfied.

For 2-dimensional condition, the correlation property is not generally satisfied. For a fixed distance d between two points, we have d=x-x'2+y-y'2. Hence,

EYx,y∙Yx',y' =EY0,02cosπ∙(i'-i)2cosπ∙d/dcorr2-i'-i22⁡.

Since 0≤i'-i≤1 and d≤dcorr, the above correlation is upper bounded by 0.1971. The upper bound is reached when d=dcorrand i'-i=22.

Here, the correlation distance dcorr has a different definition as the one used in [7].