3GPP TSG-RAN WG1#84bis	R1-163087

Busan, Korea 11th - 15th April 2016

Source:	Ericsson

Title:	Rank 5-8 Class A FD-MIMO Codebook Alternatives and Performance

Agenda Item:	7.1.4

Document for:	Discussion and Decision

# Introduction

In RAN1#84, the rank 5-8 Class A codebooks were discussed and the following conclusions were captured in the RAN1 chairman’s notes [1].

Conclusions:

- Companies are requested to evaluate rank 5 – 8  codebook design for class A based on R1-161355, R1-160850, and R1-160613 until the next meeting

  - Companies should take into acount the correction of R1-161355

- RAN1 will decide a final conclusion related rank 5 – 8 codebook design issues for class A based on evaluation results and technical analysis before or in the next meeting

- RAN1 also will decide a final conclusion related to  R1-161204 before or in the next meeting

In this contribution, we provide simulation results comparing the two alternatives for Class A rank 5-8 codebook design,  i.e. the current Rel-13 codebooks defined in 36.213 [2] and the ones described in [4].  We address the main concerns raised by the companies opposing the current Rel-13 rank 5-8 codebook that is presently specified in [2] (with further corrections captured in [3]).  We also present system level results demonstrating the performance benefits of the Rel-13 rank 5-8 codebook.

# Discussion

Companies opposing the Rel-13 rank 5-8 codebooks raised various issues with the codebook.  We discuss the main concerns raised below:

- Complexity

In our understanding, there are no concerns on the rank 5-8 codebook performance, just that its complexity may be more than is merited by the gains it has over the Rel-10 codebook.  However, it’s not clear to us yet that the complexity is excessive in Rel-13 when we consider that an UE with 8 Rx antennas is already complex, and take into account e.g. the ability to reuse CSI computations from up to rank 4 for ranks 5-8 and that i2 is not used for ranks 5-8.

- Application scenario

It was pointed out that 8 layer operation is not well established within 3GPP yet, and that new use cases such as wireless backhaul can be considered for 8 layer operation.  We don’t see how the application scenario for 8 layers is relevant, since whether to support 8 layers is not under discussion.  It is our understanding that RAN1 have agreed to specify 8 layer transmission to UEs configured for FD-MIMO.  That being said, new use cases such as wireless backhaul may be interesting, and such use cases as well as corresponding codebook enhancements can be considered in new releases.

- Reporting i2 for the Class B K=1 codebook

A concern was raised that it is not clear how i2 is reported for Class B K=1 codebook for ranks 5-8 on PUCCH.  It is not clear to us at this stage where the lack of clarity is in the specification, but such a problem seems like it could be straightforwardly corrected with the normal CR process.

- Codebook subset restriction for ranks 5-8

It was asserted that it is not reasonable to apply the same codebook subset restriction on both higher rank and lower rank (by restricting a beam direction across multiple ranks).  However, we don’t see why codebook subset restriction should operate differently for ranks 5-8 than for ranks 1-4.  CBSR improves performance by allowing accurate CSI feedback when interference is not radiated in selected directions, and this is true for higher as well as lower ranks.

# Simulation Results

In order to compare the performance of the 8-port ranks 5-8 codebooks with the 16-port ranks 5-8 codebooks, we performed system simulations with a 2x8 array and 2x1 subarray virtualization.  The simulation assumptions are summarized in the Appendix.  A comparison of rank distribution between the 8-port codebooks and the 16-port codebooks is shown in Figure 1 for 3D-UMi and 3D-UMa.  In the figure, the red bars represent the 16-port codebooks and the blue bars represent the 8-port codebooks.  The resource utilization corresponding to 3D-UMi and 3D-UMa in Figure 1 are approximately 4.7% and 6.3%, respectively.  As can be seen in the figure, the 16-port codebooks provide a higher rank distribution for ranks 5-8 than the 8-port codebooks.  For instance, in the 3D-UMi case, the 16-port codebooks achieve ranks 5-8 58.9% of the time while the 8-port codebooks achieve ranks 5-8 47.5% of the time.  This amounts to a 24% increase in transmissions involving ranks 5-8 for the 16-port codebook in 3D-UMi.  The corresponding increase in transmissions involving ranks 5-8 in 3D-UMa is 29% for the 16-port codebook.

Observations:

- A 24% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

- A 32% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

| - 3D-UMi | - 3D-UMa |
| --- | --- |

Figure 1.  Comparison of rank distribution between 8-port codebooks and 16-port codebooks.

The throughput results of the performance comparison are given in Table 1.  As can be seen from the table, the 16-port codebooks achieve 3.5% to  6.3% mean user throughput gains over the 8-port codebooks.  Hence, these results indicate that the 16-port codebooks still have notable performance benefits over the alternative 8-port codebooks.

Observation:  the 16-port ranks 5-8 codebooks yields 3.5% to 6.3% mean user throughput over the alternative 8-port ranks 5-8 codebooks.

Hence, we make the following proposal:

Proposal:  The current Rel-13 codebooks for CLASS A rank 5 to 8 should be kept.

Table 1.  Throughput comparison between 8-port codebooks and 16-port codebooks

|  | 3D-UMi |  | 3D-UMa |  |
| --- | --- | --- | --- | --- |
|  | 8-port | 16-port | 8-port | 16-port |
| Resource Utilization | ~5% | ~5% | ~6% | ~6% |
| Mean Throughput | 110.85 Mbps | 114.73 Mbps | 90.25 Mbps | 95.95 Mbps |
| Mean Throughput Gain | - | 3.5% | - | 6.3% |

# Conclusion

In this contribution, we provide simulation results comparing  Rel-13 Class A rank 5-8 codebooks  with the alternative design in [4].  We address the main concerns raised by the companies opposing the  Rel-13 rank 5-8 codebook.  Based on our results, we make the following observations and proposal:

Observations:

- A 24% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

- A 32% increase in transmissions involving ranks 5-8 is observed in 3D-UMi with the 16-port rank 5-8 codebooks when compared to the Rel-10 8-port rank 5-8 codebooks.

- The 16-port ranks 5-8 codebooks yields 3.5% to 6.3% mean user throughput over the alternative 8-port ranks 5-8 codebooks.

Proposal:

- The current Rel-13 codebooks for CLASS A rank 5 to 8 should be kept.

# References

RAN1 Chairman’s Notes, 3GPP TSG RAN1 Meeting #84, St Julian’s, Malta, 15th - 19th February 2016

TS36.213, v13.1.1

R1-161355, “Correction to rank 5-8 FD-MIMO CSI feedback”, Ericsson, et al

R1-160850, “ Codebook Design for Rank 5 - 8”,  ZTE

R1-160613, “Discussion on rank 5-8 codebook for FD-MIMO”,  LG Electronics

R1-161204, “Draft CR on FD-MIMO codebooks (36.213)”, Samsung, et al.

# Appendix

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMa 500m ISD 3D UMi 200m ISD |
| Antenna Configurations | 2x8 with 2x1 virt., UMa (122° tilt) & UMi (130° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi), 46 dBm (UMa) For Rel-10 8 port codebook, transmit power is reduced by 3 dB. |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| Overhead | 1-8 DMRS ports and 2 CRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |