3GPP TSG RAN WG1 Meeting #84 bis                                               	R1-163315

Busan, 11th – 15th April 2016

Source: 	Ericsson

Title:	Link evaluation for PDCCH for short TTI

Agenda Item:	7.3.10.1

Document for:	Approval

# Introduction

In this contribution we provide link simulation results for the PDCCH for shortened TTI in LTE downlink.

# Discussion

In [1], a shortened PDCCH (sPDCCH) is presented, to be able to schedule short TTIs more often than once per subframe. This sPDCCH carries UL and DL DCIs, and is proposed to be coded in a similar way as PDCCH and transmitted in the first symbol every short TTI.

As discussed in [1], the sPDCCH can be transmitted in the first symbol of each short TTI, where a number of resource elements are used for sPDCCH, and the remaining resources are used for data. The mapping of sPDCCH to resource elements is done in a similar way as for PDCCH to increase frequency diversity. As for PDCCH, the sPDCCH can be transmitted with different CCE aggregations, where each CCE uses 36 RE.

In this paper demodulation of the CRS-based sPDCCH is simulated.

# Evaluation results

This section presents link-level simulation performance for sPDCCH transmission based on the WF on evaluation methodology [2],[3].

## Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Short TTI bandwidth | 50 RB (10 MHz) |
| TTI length | 2 |
| Channel model | EPA, EVA |
| UE speed | 3km/h (5.56 Hz), 60 km/h (111 Hz) |
| Antenna configuration | 2Tx(eNB), 2Rx(UE) |
| CP length | Normal |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Performance metrics | BLER |

## Simulation results for 1, 2 or 4 CCE

Figure 1 to Figure 3 show BLER for sPDCCH (solid), for different payloads and aggregation level of 1, 2 or 4 CCEs. For comparison, the figures also show BLER for PDCCH (dashed). The results for PDCCH and sPDCCH are very similar, as they are designed in a very similar way. The placement of sPDCCH in frequency is done in a similar way as PDCCH, selecting groups of resource elements.

The sPDCCH can be designed with same payloads usage as PDCCH.

The sPDCCH can be designed with decoding performance comparable to PDCCH.

The sPDCCH can be transmitted in first symbol of every short TTI, using rest of resources for data.

Map sPDCCH to resource elements similarly as PDCCH, using CCEs and resource element groups, to improve frequency diversity.

For all simulations, a TTI length of 2 OFDM symbols is used, but this does not affect the performance since sPDCCH is placed only in the first symbol.

In Table 1, required SNR levels for achieving sPDCCH BLER of 1% are given for different code rates. As discussed in [4], the payloads may be around 30-50 bits for the DCI formats.

Figure 1. BLER for sPDCCH (solid) and PDCCH (dashed). EPA LOW, 3 km/h, 50PRB allocation, 1 CCE

Figure 4 to Figure 6 show simulated performance for an EVA LOW channel at 60 km/h. Tabulated performance for 1% BLER is given in Table 2.

Figure 2. BLER for sPDCCH (solid) and PDCCH (dashed). EPA LOW, 3 km/h, 50PRB allocation, 2 CCE

Figure 3. BLER for sPDCCH (solid) and PDCCH (dashed). EPA LOW, 3 km/h, 50PRB allocation, 4 CCE

Table 1: sPDCCH SNR coverage for 1% BLER for different payload and number of CCEs.
EPA LOW, 3 km/h, 50PRB allocation.

| Payload including CRC | 1 CCE | 2 CCE | 4 CCE |
| --- | --- | --- | --- |
| 30 | 2.2948 | -1.1127 | -3.9754 |
| 40 | 4.0844 | 0.24589 | -2.963 |
| 50 | 5.8727 | 1.2751 | -1.7851 |
| 60 | 8.0323 | 2.3714 | -0.96339 |
| 70 | 11.9942 | 3.3731 | 0.003019 |
| 80 |  | 4.2717 | 0.58479 |

Figure 4. BLER for sPDCCH (solid) and PDCCH (dashed). EVA LOW, 60 km/h, 50PRB allocation, 1 CCE

Figure 5. BLER for sPDCCH (solid) and PDCCH (dashed). EVA LOW, 60 km/h, 50PRB allocation, 2 CCE

Figure 6. BLER for sPDCCH (solid) and PDCCH (dashed). EVA LOW, 60 km/h, 50PRB allocation, 4 CCE

Table 2: sPDCCH SNR coverage for 1% BLER for different payload and number of CCEs.
EVA LOW, 60 km/h, 25RB allocation.

| Payload including CRC | 1 CCE | 2 CCE | 4 CCE |
| --- | --- | --- | --- |
| 30 | 1.1129 | -2.0879 | -4.7001 |
| 40 | 2.8008 | -0.96088 | -3.5526 |
| 50 | 4.5225 | 0.10141 | -2.7542 |
| 60 | 6.8983 | 1.1805 | -1.9977 |
| 70 | 12.1822 | 2.0945 | -1.3566 |
| 80 |  | 2.9563 | -0.66955 |

# Summary

In this contribution we discussed link level performance for sPDCCH transmissions. The above discussion is summarized with the following observations and proposals:

Observation 1	The sPDCCH can be designed with same payloads usage as PDCCH.

Observation 2	The sPDCCH can be designed with decoding performance comparable to PDCCH.

Proposal 1	The sPDCCH can be transmitted in first symbol of every short TTI, using rest of resources for data.

Proposal 2	Map sPDCCH to resource elements similarly as PDCCH, using CCEs and resource element groups, to improve frequency diversity.

# References

R1-163319, Physical layer aspects for PDCCH for short TTI, Ericsson, RAN1#84bis, April 2016

R1-157806, WF on evaluation methodology for latency reduction, Ericsson et al., RAN1#83, November 2015

R1-161422, Way forward on simulation results and methodology, Ericsson., RAN1#84, February 2016

R1-163323, Definition of DCI bit fields for short TTI, Ericsson, RAN1#84bis, April 2016

R1-163322, Downlink control signaling design for short TTI, Ericsson, RAN1#84bis, April 2016