3GPP TSG RAN WG1 Meeting #84bis 	R1-163349

Busan, Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0xxxx | rev | x | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Channel interleaver correction for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Channel interleaver description in TS36.212 currently states that for subframes where cell-specific SRS symbols can exist, that rate matching (#symbols is reduced) is applied. This is inconsistent with what is stated in TS36.211 section 5.3.4 for BL/CE UEs in CEModeB: “For BL/CE UEs in CEModeB, resource elements in the last SC-FDMA symbol in a subframe configured with cell-specific SRS shall be counted in the PUSCH mapping but not used for transmission of the PUSCH.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | The channel interleaver is modified to NOT reduce the number of SCFDMA symbols because of SRS multiplexing when a BL/CE UE is configured in CEModeB. This is implicity stated by placing an additional condition on when is permitted to equal 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cross-tti data symbol combining not possible for CEModeB since rate matching and interleaving of data symbol transmission not identical within a bundle of 4 subframes. This specifically applies when SRS subframe configuration is different from 0 and 15 (FDD case) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.6 	Channel coding of control information

Control data arrives at the coding unit in the form of channel quality information (CQI and/or PMI), HARQ-ACK and rank indication, and CSI-RS resource indication (CRI). Different coding rates for the control information are achieved by allocating different number of coded symbols for its transmission. When control data are transmitted in the PUSCH, the channel coding for HARQ-ACK, rank indication (including RI only, joint report of RI/i1, joint report of CRI/RI, joint report of CRI/RI/ i1,joint report of CRI/RI/PTI, and joint report of RI/PTI), CRI and channel quality information  is done independently.

...

For UL-SCH data information , where

-	 is the number of layers the corresponding UL-SCH transport block is mapped onto, and

-	 is the scheduled bandwidth for PUSCH transmission in the current sub-frame for the transport block, and

-	 is the number of SC-FDMA symbols in the current PUSCH transmission sub-frame given by , where

-	 is equal to 1 for non-BL/CE UEs and BL/CE UEs in CEModeA

-	if UE configured with one UL cell is configured to send PUSCH and SRS in the same subframe for initial transmission, or

-	if UE transmits PUSCH and SRS in the same subframe for the current subframe in the same serving cell, or

-	if the PUSCH resource allocation for the current subframe even partially overlaps with the cell-specific SRS subframe and bandwidth configuration defined in section 5.5.3 of [2], or

-	if the current subframe in the same serving cell is a UE-specific type-1 SRS subframe as defined in Section 8.2 of [3], or

-	if the current subframe in the same serving cell is a UE-specific type-0 SRS subframe as defined in section 8.2 of [3] and the UE is configured with multiple TAGs.

-	Otherwise  is equal to 0.