3GPP TSG RAN WG1 Meeting #84bis 	R1-163350

Busan, Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0xxxx | rev | x | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MDPCCH candidate overflow monitoring correction for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | MDPCCH candidate overflow monitoring correction for eMTC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the RAN1#84 agreement: Agreement: • A MPDCCH SS with repetitions that crosses over (SFN=0), is not monitored by the UE. • At each (SFN=0), the UE shall reset the starting SF used for the SS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The current “other than” text could be interpreted as saying that the first valid MPDCCH candidate starting in nf=0 should be ignored because it has ECCEs in nf=1, even though it may not have any ECCEs in nf=-1 (i.e. the previous SFN range) 2. If the UE is monitoring in parallel 2 search spaces on 2 different NBs, the current text could be interpreted as saying that the UE drops both candidates at the overflow point even though the overlap only needs to be avoided on one of the narrowbands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.1.5	MPDCCH assignment procedure

A BL/CE UE shall monitor a set of MPDCCH candidates on one or more Narrowbands (described in subclause 5.2.4 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the MPDCCHs in the set according to all the monitored DCI formats. The Narrowband in a subframe used for MPDCCH monitoring is determined as described in [3].

...

For MPDCCH UE-specific search space, Type0-common search space, and Type2-common search space locations of starting subframe  are given by where is the th consecutive BL/CE DL subframe from subframe , and , and , and , where

-	subframe  is a subframe satisfying the condition , where

-	For MPDCCH UE-specific search space, Type0-common search space is given by the higher layer parameter mPDCCH-startSF-UESS,

-	For Type2-common search space, is given by the higher layer parameter mPDCCH-startSF-CSS-RA-r13

-	is given by higher layer parameter mPDCCH-NumRepetition, and

-	,, , are given in Table 9.1.5-3.

For Type1-common search space, = and is determined from locations of paging opportunity subframes,

If SystemInformationBlockType1-BR or other SI messages are transmitted in subframe , a BL/CE UE shall assume MPDCCH in the subframe  is dropped.The BL/CE UE is not required to monitor an MPDCCH candidate in a narrowband if any ECCEs corresponding to the MPDCCH candidate occur within a frame before and also occur within frame.