3GPP TSG RAN WG1 Meeting #84bis 	R1-163401

Busan, Korea, 11th - 15th April 2016

Source:	Sharp

Title:	Link level evaluation results of shortened TTI for PUSCH

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

Introduction

In the RAN1#84 meeting, the following agreements were made on PUSCH DM-RS for the latency reduction study item [1].

Agreements: - Following aspects are further studied in the next RAN1 meeting - Note: But the study is not limited to them. - Design of sPUSCH DM-RS - Alt.1: DM-RS symbol shared by multiple short-TTIs within the same subframe - Alt.2: DM-RS contained in each sPUSCH - HARQ for sPUSCH - Whether/how to realize asynchronous and/or synchronous HARQ - sTTI operation for Pcell and/or SCells by (e)CA in addition to non-(e)CA case

In this contribution, we show the evaluation results of design of sPUSCH DM-RS.

Performance evaluations

sPUSCH design

In [1], it was agreed that at least DM-RS design and HARQ operation for sPUSCH design are further studied. In this contribution we focus on the DM-RS design for sPUSCH. We assume slot TTI, 4-symbol TTI, and 2-symbol TTI as sTTI.

Figure 1 shows an example of slot TTI, 4-symbol TTI, and 2-symbol TTI of sPUSCH design. As shown in this figure, in slot TTI case as shown in Figure 1 (a), the same DM-RS pattern as legacy TTI can be used. In Figure 1 (b), this design is mentioned in [2], the same DM-RS pattern as legacy TTI is used and each DM-RS symbol is shared by two UEs as shown in this figure. In the 2-symbol TTI shown in Figure 1 (c), this has already been mentioned in [3], and this is aligned with Alt. 2.

Figure 1. An example of sPUSCH structure

In the 2-symbol TTI case, since DM-RS symbol is contained in each TTI, the maximum throughput degrades due to the DM-RS overhead. At the same time, shorter HARQ cycles may improve user throughput when packet size is very small. Therefore, user throughput performance of the 2-symbol TTI can be evaluated with system level simulations.

Evaluation results

We evaluated the performances of BLER and throughput. Detailed evaluation assumptions are listed in Annex and it is aligned with the link level evaluation assumptions for sPUSCH based on the agreed assumptions in [4]. The number of PRBs for sPUSCH is 25 in this evaluation.

Low mobility case

Figure 2 shows BLER performance for QPSK with code rate 1/3 and 16QAM with code rate 3/4 in low mobility case. In low mobility case, BLER performance is almost same as the legacy TTI case.

Observation 1:

- In low mobility case, similar performances are achieved from BLER perspective irrespective of TTI length.

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure 2. BLER performances in low mobility case |  |

Table 1 and Table 2 show the SINR at 10% BLER using the format in [5] to capture the TR.

Table 1. SINR at 10% BLER EPA 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.05 dB |  | 1.95 dB | 2.27 dB | 2.19 dB |
| 16QAM 3/4 |  | 13.72 dB |  | 13.72 dB | 13.69 dB | 13.53 dB |

Table 2. SINR at 10% BLER ETU 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.06 dB |  | 0.11 dB | - 0.12 dB | 0.22 dB |
| 16QAM 3/4 |  | 12.68 dB |  | 12.59 dB | 12.58 dB | 12.56 dB |

High mobility case

Figure 3 shows BLER performance for QPSK with code rate 1/3 and 16QAM with code rate 3/4 in high mobility case. As shown in the figures, the BLER performance difference between legacy TTI and various sTTIs are negligible at low SNR with QPSK. However, at higher SNR with 16QAM, the performance of sTTIs, esp. 4-symbol TTI, is slightly worse, because channel variance in time domain due to high Doppler cannot be compensated within TTI because only one DM-RS symbol is contained within 4 SC-FDMA symbols.

Observation 2:

- In high mobility case, sTTIs has comparable performance to legacy TTI based PUSCH under low SNR with low order modulation, sTTI PUSCH, esp. 4-symbol sTTI performance becomes slightly worse (around -1 dB) under high SNR with higher order modulation.

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure 3. BLER performances in high mobility case |  |

Table 3 and Table 4 show the SINR at 10% BLER using the format in [5] to capture the TR.

Table 3. SINR at 10% BLER EVA 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.33 dB |  | 0.39 dB | 0.3 dB | 0.51 dB |
| 16QAM 3/4 |  | 12.87 dB |  | 13.42 dB | 13.19 dB | 12.66 dB |

Table 4. SINR at 10% BLER ETU 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 0.2 dB |  | 0.17 dB | 0.16 dB | 0.17 dB |
| 16QAM 3/4 |  | 13.03 dB |  | 13.29 dB | 13.01 dB | 12.65 dB |

Conclusion

In this contribution, we have the following observations:

Observation 1:

- In low mobility case, similar performances are achieved from BLER perspective irrespective of TTI length.

Observation 2:

- In high mobility case, sTTIs has comparable performance to legacy TTI based PUSCH under low SNR with low order modulation, sTTI PUSCH, esp. 4-symbol TTI performance becomes slightly worse (around -1 dB) under high SNR with higher order modulation.

References

3GPP RAN1#84 chairman’s notes, January 2016.

R1-160294, “Short TTI for UL transmissions”, RAN1#84, February 2016.

R1-160940, “Link level evaluation of PUSCH for short TTI”, RAN1#84, February 2016.

R1-157806, “WF on evaluation methodology for latency reduction”, RAN1#83, November 2015.

R1-161422, “Way forward on simulation results and methodology”, RAN1#84, February 2016.

Annex

Evaluation assumptions

Table 5. Evaluation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 10MHz |
| TTI length | 2/4/7 symbols |
| Allocated bandwidth | 25 PRBs |
| Channel model | EPA 3km/h, ETU 3km/h, EVA 60km/h, ETU 60 km/h |
| Antenna configuration | 1Tx (UE), 2Rx (eNB) |
| CP length | Normal |
| Transmission mode | TM1 |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Link adaptation | Disabled |
| Modulation and code rate | QPSK 1/3, 16QAM 3/4 |
| HARQ retransmission | Disabled |
| Performance metrics | BLER, throughput |

Throughput performances to capture TR

Table 6 to 9 show the throughput performance at 10% BLER using the format in [5] to capture the TR.

Table 6. Throughput at 10% BLER EPA 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.09 Mbps |  | 1.82 Mbps | 2.09 Mbps | 2.13 Mbps |
| 16QAM 3/4 |  | 5.39 Mbps |  | 8.05 Mbps | 9.52 Mbps | 9.57 Mbps |

Table 7. Throughput at 10% BLER ETU 3 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.09 Mbps |  | 1.80 Mbps | 2.15 Mbps | 2.10 Mbps |
| 16QAM 3/4 |  | 5.18 Mbps |  | 8.13 Mbps | 9.35 Mbps | 9.39 Mbps |

Table 8. Throughput at 10% BLER EVA 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.08 Mbps |  | 1.77 Mbps | 2.10 Mbps | 2.07 Mbps |
| 16QAM 3/4 |  | 5.28 Mbps |  | 7.76 Mbps | 8.82 Mbps | 9.43 Mbps |

Table 9. Throughput at 10% BLER ETU 60 km/h

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.06 Mbps |  | 1.77 Mbps | 2.09 Mbps | 2.11 Mbps |
| 16QAM 3/4 |  | 5.13 Mbps |  | 7.61 Mbps | 8.91 Mbps | 9.32 Mbps |

Figure 4 and Figure 5 shows the throughput performance.

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure 4. Throughput performances in low mobility case |  |
| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| Figure 5. Throughput performances in high mobility case |  |