3GPP TSG RAN WG1 Meeting #84bis 	R1-163472

Busan, Korea, April 11-15, 2016

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI, CATT

Title:	Large-scale and small-scale channel modeling for indoor hotspot

Document for:	Discussion and decision

Introduction

In the Channel Modeling Ad hoc meeting, the large-scale parameters (LSP) and small-scale parameters (SSP) modeling has been discussed. In the two working assumptions [1] and [2] from the meeting, most of the LSPs and SSPs for UMi and UMa have been addressed respectively. And also the frequency dependency for part of the LSPs and SSPs has been included. So far, the LSP and SSP modeling for indoor hotspot are still under discussion.

Based on the latest published LSP and SSP research results in Appendix A, the LSPs and SSPs for InH has been further investigated in this contribution.

- 2	Indoor hotspot LSP and SSP modeling

In the recently published white paper from both industry and academy researches in [3], the fast fading modelling for InH has been investigated based on both measurements and ray-tracing. The indoor office environments have been investigated at frequencies including 2.44GHz, 2.9 GHz, 3.5 GHz, 5.8GHz, 6 GHz, 14 GHz, 14.8GHz, 15 GHz, 20 GHz, 26GHz, 28 GHz, 29 GHz, 38GHz, 39.5GHz, 58.7GH, 60 GHz, and 73 GHz. Lots of valuable results for different frequencies have been summarized from Table 7 to Table 10 in Appendix A. Based on these valuable data results, a set of LSPs and SSPs that can be applicable for frequencies up to 100GHz has been summarized in this section.

For LSPs, they are modeled by a log-normal distribution with mean  and standard variation . For some LSPs, based on the data results for different frequencies, frequency dependency has been investigated by checking the change of parameters with frequencies. For some LSPs, frequency dependency can be observed. But for some other LSPs, no strong frequency dependency is observed. The results are summarized from Table 1 to Table 4. The LSPs for IMT-A InH were also listed as reference [4]. It should be noted that the results from IMT-A InH are based 2D modeling.

Table 1. LSP for indoor office LOS (part1)

| Scenarios |  | LOS |  |
| --- | --- | --- | --- |
|  |  | IMT-A | Recommended |
| Delay spread (DS) log10([s]) | DS | –7.70 | [-0.01*log10(1+f)-7.79] |
|  | DS | 0.18 | [-0.16*log10(1+f)+0.50] |
| AoD spread (σASD) log10([]) | ASD | 1.60 | 1.60 |
|  | ASD | 0.18 | 0.18 |
| AoA spread (σASA) log10([]) | ASA | 1.62 | [-0.19*log10(1+f)+1.86] |
|  | ASA | 0.22 | [0.12*log10(1+f)] |
| ZoA spread (σZSA) log10([]) | ZSA | N/A | [-0.26*log10(1+f)+1.21] |
|  | ZSA | N/A | [-0.04*log10(1+f)+0.17] |
| ZoD spread (σZSD) log10([]) | ZSD | N/A | [-1.43*log10(1+f)+2.25] |
|  | ZSD | N/A | [0.13*log10(1+f)+0.15] |
| K-factor (K) [dB] | ZSA | 7 | [0.84*log10(1+f)+2.12] |
|  | ZSA | 4 | [-0.58*log10(1+f)+6.19] |
| XPR [dB] |  | 11 | 15 |
|  |  | N/A | 3 |
| Correlation distance in the horizontal plane [m] | DS | 8 | 8 |
|  | ASD | 7 | 7 |
|  | ASA | 5 | 5 |
|  | SF | 10 | 10 |
|  | K | 4 | 4 |
|  | ZSA | N/A | 3 |
|  | ZSD | N/A | 3 |
| Delay distribution |  | Exp | Exp |
| AoD and AoA distribution |  | Laplacian | Wrapped Gaussian |
| ZoD and ZoA distribution |  | N/A | Laplacian |
| Delay scaling parameter r |  | 3.6 | 2.15 |

Table 2. LSP for indoor office LOS (part2)

| Scenarios |  | LOS |  |
| --- | --- | --- | --- |
|  |  | IMT-A | Recommended |
| Cross-Correlations | ASD vs DS | 0.6 | 0.6 |
|  | ASA vs DS | 0.8 | 0.8 |
|  | ASA vs SF | –0.5 | –0.5 |
|  | ASD vs SF | –0.4 | –0.4 |
|  | DS vs SF | –0.8 | –0.8 |
|  | ASD vs ASA | 0.4 | 0.4 |
|  | ASD vs K | 0 | 0 |
|  | ASA vs K | 0 | 0 |
|  | DS vs K | –0.5 | -0.5 |
|  | SF vs K | 0.5 | 0.5 |
|  | ZSD vs SF | N/A | 0.1 |
|  | ZSA vs SF | N/A | -0.3 |
|  | ZSD vs K | N/A | 0 |
|  | ZSA vs K | N/A | 0.1 |
|  | ZSD vs DS | N/A | 0.2 |
|  | ZSA vs DS | N/A | 0.1 |
|  | ZSD vs ASD | N/A | 0.3 |
|  | ZSA vs ASD | N/A | 0.1 |
|  | ZSD vs ASA | N/A | 0.3 |
|  | ZSA vs ASA | N/A | 0.3 |
|  | ZSD vs ZSA | N/A | 0.3 |

Table 3. LSP for indoor office NLOS (part1)

| Scenarios |  | NLOS |  |
| --- | --- | --- | --- |
|  |  | IMT-A | Recommended |
| Delay spread (DS) log10([s]) | DS | –7.41 | [-0.28*log10(1+f)-7.29] |
|  | DS | 0.14 | [0.10*log10(1+f)+0.11] |
| AoD spread (σASD) log10([]) | ASD | 1.62 | 1.49 |
|  | ASD | 0.25 | 0.17 |
| AoA spread (σASA) log10([]) | ASA | 1.77 | [-0.11*log10(1+f)+1.80] |
|  | ASA | 0.16 | [0.12*log10(1+f)] |
| ZoA spread (σZSA) log10([]) | ZSA | N/A | [-0.15*log10(1+f)+1.04] |
|  | ZSA | N/A | [-0.09*log10(1+f)+0.24] |
| ZoD spread (σZSD) log10([]) | ZSD | N/A | 1.37 |
|  | ZSD | N/A | 0.38 |
| K-factor (K) [dB] | ZSA | N/A | N/A |
|  | ZSA | N/A | N/A |
| XPR [dB] |  | 10 | 12 |
|  |  | N/A | 7 |
| Correlation distance in the horizontal plane [m] | DS | 5 | 5 |
|  | ASD | 3 | 3 |
|  | ASA | 3 | 3 |
|  | SF | 6 | 6 |
|  | K | N/A | N/A |
|  | ZSA | N/A | 3 |
|  | ZSD | N/A | 3 |
| Delay distribution |  | Exp | Exp |
| AoD and AoA distribution |  | Laplacian | Wrapped Gaussian |
| ZoD and ZoA distribution |  | N/A | Laplacian |
| Delay scaling parameter r |  | 3 | 1.84 |

Table 4. LSP for indoor office NLOS (part2)

| Scenarios |  | NLOS |  |
| --- | --- | --- | --- |
|  |  | IMT-A | Recommended |
| Cross-Correlations | ASD vs DS | 0.4 | 0.4 |
|  | ASA vs DS | 0 | 0 |
|  | ASA vs SF | –0.4 | –0.4 |
|  | ASD vs SF | 0 | 0 |
|  | DS vs SF | –0.5 | –0.5 |
|  | ASD vs ASA | 0 | 0 |
|  | ASD vs K | N/A | N/A |
|  | ASA vs K | N/A | N/A |
|  | DS vs K | N/A | N/A |
|  | SF vs K | N/A | N/A |
|  | ZSD vs SF | N/A | N/A |
|  | ZSA vs SF | N/A | -0.1 |
|  | ZSD vs K | N/A | N/A |
|  | ZSA vs K | N/A | N/A |
|  | ZSD vs DS | N/A | -0.1 |
|  | ZSA vs DS | N/A | -0.1 |
|  | ZSD vs ASD | N/A | 0.3 |
|  | ZSA vs ASD | N/A | 0.2 |
|  | ZSD vs ASA | N/A | 0.1 |
|  | ZSA vs ASA | N/A | -0.04 |
|  | ZSD vs ZSA | N/A | 0.4 |

For SSPs, clustering results are based on joint spatial and temporal analysis. The frequency dependency of SSP was also investigated. The results are summarized in Table 5 and Table 6. The SSPs for IMT-A InH were also listed as reference [4]. It should be noted that the results from IMT-A InH are based 2D modeling.

Table 5. SSP for indoor office LOS

| Scenarios | LOS |  |
| --- | --- | --- |
|  | IMT-A | Recommended |
| Number of cluster | 15 | [4] |
| Number of rays per cluster | 20 | [20] |
| Cluster ASD | 5 | 7 |
| Cluster ASA | 8 |  |
| Cluster ZSA | N/A |  |
| Cluster ZSD | N/A | 5 |
| Per cluster shadowing std [dB] | 6 | 6 |

Table 6. SSP for indoor office NLOS

| Scenarios | NLOS |  |
| --- | --- | --- |
|  | IMT-A | Recommended |
| Number of cluster | 19 | [4] |
| Number of rays per cluster | 20 | [20] |
| Cluster ASD | 5 | 3 |
| Cluster ASA | 11 |  |
| Cluster ZSA | N/A |  |
| Cluster ZSD | N/A | 6 |
| Per cluster shadowing std [dB] | 3 | 3 |

- 3	Conclusion

In this contribution, the LSP and SSP have been investigated for indoor indoor office. Based on the results, the following is proposed:

Proposal 1:.for indoor office, the correlation distance and cross correlation of LSP in IMT-A can be reused for azimuth domain channel characteristics. The recommended LSP from Table 1 to Table 4 should be adopted for large-scale channel modeling of this SI.

Proposal 2:.for indoor office, the number of rays per cluster and per cluster shadowing in IMT-A can be reused. The recommended SSP in Table 5 and Table 6 should be adopted for cluster-level small–scale channel modeling of this SI.

- References

R1-161661, “Joint proposal on large-scale and small-scale parameters for >6GHz channel model”, Samsung, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, Nokia Networks, NTT DOCOMO, Qualcomm, ETRI, Ljubljana, March. 14-16, 2016.

R1-161661, “Joint proposal on large-scale parameters for UMa scenario”, Nokia Networks, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm and Samsung, Ljubljana, March. 14-16, 2016.

Aalto University, AT&T, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz (v2.0)”, Mar. 2016 (http://www.5gworkshops.com/5GCM.html).

ITU-R M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”, Dec. 2009.

- Appendix A:	InH Large-scale channel characteristics

Table 7. Preliminary Indoor office large-scale channel characteristics (part 1)

|  |  | 3.5GHz1 |  | 6GHz1 |  | 20 GHz2 | 28GHz3 | 73GHz4 | 3.GHz5 |  | 8GHz5 |  | 73GHz5 |  | 28GHz5 |  | 38GHz6 |  | 26GHz7 |  | 39.5GHz7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenarios |  | LOS | NLOS | LOS | NLOS | LOS | LOS | Hybrid | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Delay spread () log10(seconds) | Median(ns) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 17.72 | 28.77 | 10.48 | 31.28 | 14.53 | 23.06 | 10.58 | 23.28 | TBD | TBD | TBD | TBD | TBD | TBD |
|  | DS | TBD | TBD | TBD | TBD | -7.33 | -7.75 | -8.1 | -8.08 | -7.58 | -8.19 | -7.62 | -7.85 | -7.67 | -7.97 | -7.7 | -7.83 | -7.64 | -7.63 | -7.61 | -8.12 | -8.58 |
|  | DS | TBD | TBD | TBD | TBD | 0.1 | 0.13 | 0.4 | 0.53 | 0.15 | 0.63 | 0.29 | 0.18 | 0.2 | 0.25 | 0.2 | 0.21 | 0.19 | 0.32 | 0.27 | 0.32 | 0.56 |
| Delay distribution |  | TBD | TBD | TBD | TBD | TBD | Exponential | Exponential | Exponential |  |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD |
| AoA spread (ASA) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 54.04 | 64.75 | 51.5 | 65.65 | 30.48 | 37.46 | 28.24 | 47.51 | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASA | 3GPP | 3GPP | 1.72 | 1.49 | TBD | 1.57 | 1.6 | 1.72 | 1.81 | 1.71 | 1.8 | 1.46 | 1.54 | 1.43 | 1.66 | 1.76 | 1.87 | 1.60 | 1.61 | 1.36 | 1.46 |
|  | ASA | 3GPP | 3GPP | 0.14 | 0.23 | TBD | 0.22 | 0.37 | 0.05 | 0.04 | 0.09 | 0.06 | 0.13 | 0.19 | 0.14 | 0.12 | 0.17 | 0.09 | 0.20 | 0.12 | 0.16 | 0.20 |
| AoD spread (ASD) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD | 3GPP | 3GPP | 1.34 | 1.49 | 1.8 | 1.08 | 1.5 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASD | 3GPP | 3GPP | 0.15 | 0.08 | 0.09 | 0.23 | 0.26 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| ZoA spread (ZSA) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 6.99 | 6.98 | 6.73 | 6.9 | 4.37 | 5.71 | 9.9 | 6.24 | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA | 1.02 | 1.08 | 1.33 | 0.95 | TBD | 0.67 | -0.025d+1.18 | 0.84 | 0.84 | 0.83 | 0.84 | 0.62 | 0.76 | 1 | 0.79 | 0.86 | 0.87 | 0.90 | TBD | 0.83 | TBD |
|  | ZSA | 0.41 | 0.36 | 0.07 | 0.28 | TBD | 0.17 | 0.30 | 0 | 0 | 0.01 | 0.01 | 0.14 | 0.08 | 0.05 | 0.08 | 0.04 | 0.05 | 0.14 | TBD | 0.20 | TBD |
| ZoD spread (ZSD) log10(degrees) | Median(deg) | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD | 1.22 | 1.26 | 1.25 | 1.48 | 0 | 0.40 | -0.040d+1.45 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD | 0.23 | 0.67 | 0.25 | 0.09 | 0.48 | 0.23 | 0.33 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| AoD and AoA distribution |  | TBD | TBD | TBD | TBD | TBD | Laplacian | Uniform | Wrapped Gaussian |  |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD |
| ZoD and ZoA distribution |  | Gaussian |  | TBD | TBD | TBD | Laplacian | Laplacian | Laplacian |  |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD |
| Delay scaling parameter r |  | TBD | TBD | TBD | TBD | TBD | 1.95 | 2.4 | TBD | TBD | TBD | TBD | 2.03 | 1.35 | 2.23 | 1.78 | TBD | TBD | TBD | TBD | TBD | TBD |
| XPR (dB) | XPR | TBD | TBD | TBD | TBD | TBD | TBD | 15 | TBD | TBD | TBD | TBD | 10 | 6 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | XPR | TBD | TBD | TBD | TBD | TBD | TBD |  | TBD | TBD | TBD | TBD | 7.5 | 10 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| LOS Ricean K factor (dB) * | K | TBD | N/A | TBD | N/A | TBD | TBD | 8 | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A |
|  | K | TBD | N/A | TBD | N/A | TBD | TBD | 3 | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A | TBD | N/A |

- From BUPT based on measurement

- From DOCOMO based on measurement

- From KT based on measurement

- From Nokia/NYU based on ray-tracing

- From Huawei based on measurement

- From ETRI based on measurement

- From CATT based on measurement

Table 8. Preliminary Indoor office large-scale channel characteristics (part 2)

|  |  | 14GHz1 |  | 28GHz1 |  | 2.44 GHz2 | 5.8 GHz2 | 14.8 GHz2 | 15GHz2 |  | 58.7GHz2 | 2.9GHz3 |  | 29GHz3 |  | 28GHz4 |  | 73GHz4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenarios |  | LOS | NLOS | LOS | NLOS | NLOS | NLOS | NLOS | LOS | NLOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Delay spread () log10(seconds) | Median(ns) | 27.78 | 36.92 | 29.35 | 43.16 | 24 | 28 | 24 | 22.9 | 42.4 | 21 | 24 | 45 | 25 | 36 | TBD | TBD | TBD | TBD |
|  | DS | -7.60 | -7.47 | -7.55 | -7.41 | -7.59 | -7.55 | -7.62 | -7.65 | -7.43 | -7.64 | -7.7 | -7.4 | -7.6 | -7.5 | TBD | TBD | TBD | TBD |
|  | DS | 0.27 | 0.22 | 0.15 | 0.17 | 0.25 | 0.17 | 0.20 | 0.2 | 0.17 | 0.30 | 0.29 | 0.14 | 0.37 | 0.18 | TBD | TBD | TBD | TBD |
| Delay distribution |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | Exponential |  |  |  | TBD | TBD | TBD | TBD |
| XPR (dB) | XPR | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 14 | 10.4 | 22.8 | 15.4 |
|  | XPR | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 1.5 | 9.7 | 2.4 | 8 |

- From CMCC&BUPT based on measurement

- From Ericsson based on measurement

- From Qualcomm based on measurement

- From NYU based on measurement

Table 9. Preliminary Indoor office large-scale channel characteristics (part 3)

| Parameters |  | # 3.5GHz1 |  | # 20GHz2 | # 28GHz3 | # 73GHz4 | # 28GHz5 |  | # 73GHz5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | LOS | Hybrid | LOS | NLOS | LOS | NLOS |
| Correlation distance (m) | DS [ns] | TBD | TBD | TBD | TBD | TBD | 2.26 | 2.47 | 1.85 | 1.69 |
|  | ASD [deg] | TBD | TBD | TBD | TBD | TBD | 2.01 | 1.83 | 1.96 | 0.67 |
|  | ASA [deg] | TBD | TBD | TBD | TBD | TBD | 2.07 | 2.61 | 2.06 | 1.62 |
|  | ZSD [deg] | 4 | 4 | TBD | TBD | TBD | 3.01 | 2.23 | 3.03 | 2.02 |
|  | ZSA [deg] | 4 | 4 | TBD | TBD | TBD | 2.65 | 2.54 | 2.09 | 0.83 |
|  | SF [dB] | TBD | TBD | TBD | TBD | TBD | 1.64 | 1.14 | 0.28 | 2.45 |
|  | K [dB] | TBD | TBD | TBD | TBD | TBD | 1.62 | 1.13 | 0.28 | 2.41 |
| Cross Correlation | ASD [deg] vs DS [s] | 3GPP | 3GPP | 0.26 | 0.481 | 0.47 | TBD | TBD | TBD | TBD |
|  | ASA [deg] vs DS [s] | 3GPP | 3GPP | TBD | 0.5433 | 0.28 | 0.83 | -0.03 | 0.69 | -0.1 |
|  | ASA [deg] vs SF [dB] | TBD | TBD | TBD | 0.0454 | TBD | 0.07 | -0.55 | -0.41 | -0.13 |
|  | ASD [deg] vs SF [dB] | TBD | TBD | -0.68 | 0.3111 | TBD | TBD | TBD | TBD | TBD |
|  | DS[s] vs SF[dB] | TBD | TBD | -0.33 | 0.3318 | TBD | 0.44 | 0.35 | -0.24 | 0.6 |
|  | ASD [deg] vs ASA [deg] | 3GPP | 3GPP | TBD | 0.6236 | 0.15 | TBD | TBD | TBD | TBD |
|  | ASD [deg] vs K [dB] | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ASA [deg] vs K [dB] | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | DS [s] vs K [dB] | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | SF [dB] vs K [dB] | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs SF [dB] | 0.18 | 0 | 0.02 | 0.3749 | TBD | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs SF [dB] | 0.34 | 0 | TBD | 0.3313 | TBD | -0.52 | -0.01 | -0.68 | -0.27 |
|  | ZSD [deg] vs K [dB] | 0 | N/A | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs K [dB] | 0.1 | N/A | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs DS [s] | 0 | -0.27 | 0.24 | 0.1752 | 0.07 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs DS [s] | 0 | -0.06 | TBD | 0.4963 | -0.01 | -0.2 | -0.12 | 0.41 | -0.24 |
|  | ZSD [deg] vs ASD [deg] | 0.65 | 0.35 | 0.1 | -0.3449 | 0.21 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs ASD [deg] | 0 | 0.23 | TBD | 0.5116 | 0.1 | TBD | TBD | TBD | TBD |
|  | ZSD [deg] vs ASA [deg] | 0 | -0.08 | TBD | -0.0418 | 0.22 | TBD | TBD | TBD | TBD |
|  | ZSA [deg] vs ASA [deg] | 0.62 | 0.43 | TBD | 0.6429 | 0.06 | -0.03 | -0.28 | 0.61 | -0.37 |
|  | ZSD [deg] vs ZSA [deg] | 0.05 | 0.42 | TBD | 0.0975 | 0.44 | TBD | TBD | TBD | TBD |

- From BUPT based on measurement

- From DOCOMO based on measurement

- From KT based on measurement

- From Nokia/NYU based on ray-tracing

- From Huawei based on measurement

Table 10. Preliminary Indoor office small large-scale channel characteristics

| Parameters | # 73GHz1 | # 28GHz2 | # 3.5GHz3 |  | # 8GHz3 |  | # 28GHz3 |  | # 73GHz3 |  | # 3.5GHz4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hybrid | LOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Number of clusters | 6 | 7 | 4 | 3 | 4 | 3 | 3.271 | 3.5 | 3.9 | 4.3 | TBD | TBD |
| Number of rays per cluster | 10 | 4 | 20 | 20 | 20 | 20 | 52.9 | 41.82 | 38.47 | 35.75 | TBD | TBD |
| Cluster ASD [deg] | 3.2 | 9.75 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| Cluster ASA[deg] | 3.7 | 13.22 | 12.75 | 20.89 | 9.81 | 19.95 | 4.39 | 10.71 | 5.51 | 8.7 | TBD | TBD |
| Cluster ZSD[deg] | 10^(-0.043d+0.09) | 3.61 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | 6 | 6 |
| Cluster ZSA[deg] | 10^(-0.036d-0.18) | 5.16 | 6.92 | 6.91 | 6.53 | 6.76 | 3.12 | 3.89 | 3.76 | 3.89 | 9 | 9 |
| Per cluster shadowing std [dB] | 12 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

- From Nokia/NYU based on ray-tracing

- From KT based on measurement

- From Huawei based on measurement

- From CMCC based on measurement