3GPP TSG RAN WG1 Meeting #84bis                                                                                 R1-163519

Busan, 11th – 15th April 2016

Source: 	Ericsson

Title:	Text proposal on findings from the system level evaluation of latency reduction

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

# Introduction

During RAN1#84 and RAN1#84bis several companies provided system level evaluation results for latency reduction based on shortened TTI. In this TP, the observations obtained from the results provided according to the agreed format [1] are added to the latency reduction TR [2].

# Proposal

Incorporate the following in the text proposal in the Latency reduction TR.

---------------------------------- Start of Text Proposal ------------------------------------------------------

## 9.4	Findings from system evaluations on TTI reduction and reduced processing time

Findings from the results in Annex B for the scenario described in Annex A with FTP over TCP traffic are summarized below.

The results of all sources reveal that shortening TTI length can have great benefits. One source shows gains with shortening TTI for all UEs at all loads and for all file sizes. In all other sources benefits are not always present but certain situations are found to be favourable to observe gains with TTI shortening. With small file sizes (100kbit, 100kB), shortening TTI length improves significantly the performance of all UEs at low to medium load, while mostly good UEs and cell-center UEs benefit from TTI shortening at high load. With medium file size (500kB), cell-edge UEs have little to no benefit in all load points, but good UEs and cell center UEs benefit for most load points. With large file size (1MB), only cell center UEs benefit from shortening TTI length.

All sources with results for different core network delays show that gains when operating with shorter TTI reduce with larger core network delay but are still very high at low load.

Most sources obtained smaller gains from shortening TTI with increased resource utilization (RU) in most simulated scenarios. A less strong correlation between RU and gains with shortened TTI was observed by two sources.

Generally, in the favourable situations for TTI shortening, gains are larger with shorter TTI length such as 2 OFDM symbols (OS) than with 7 OS. However, it was found by two sources that 7 OS TTI length may provide gains in more scenarios than even shorter TTI. Regarding TTI length shorter than 2OS, one source shows better performance with 1 OS TTI than all other TTI lengths in all simulated scenarios, while four other sources have more contrasted results. One of them shows that performance gains with 1 OS TTI are smaller than with 2 OS TTI in most cases. Another source shows that the performance benefit of 1 OS TTI over 2 OS TTI is only visible for cell center UEs. Two other sources show that in most scenarios 1 OS TTI is less beneficial than 2 OS TTI.

It is understood that the additional control overhead for shortened TTI that is more frequent for 1 OS and 2 OS TTI is the limiting factor of very short TTI length. Three sources highlight that in average less than 2 UEs are scheduled in the same shortened TTI at low load in their simulations. One of these sources shows that the required size of the control channel region for scheduling shortened TTI is in average less than 2 CCEs at low load. Another of these sources shows that multiplexing more UEs in a shortened TTI may have benefits at medium to high load. One source shows that shortened TTI gains increase with larger system BW as the relative control channel overhead for shortened TTI decreases.

Four sources evaluated the usage of shortened TTI for obtaining faster update of CSI measurements. One of the source shows minor performance improvement with fast CSI. Two of the sources show some benefit of using shortened TTI for including CSI update, especially at high load and for cell edge UEs. Some benefit of having fast CSI was observed in the fourth source mostly due to the effect of a smaller CQI report delay rather than a smaller CQI report period. Two sources evaluated TTI shortening with fast CSI for different UE speeds. One of the sources shows that short TTI benefits are higher when UE speed increases from 3 km/h to 120 km/h. The other source shows moderate impact on shortened TTI gain when UE speed increases from 3 km/h to 30km/h.

One source shows benefit of shortened TTI with UL FTP traffic at low load. The largest gains were observed with 2OS TTI and dynamic DMRS insertion.

----------------------------------------------- End of Text Proposal ------------------------------------------------

# References

R1-161422, Way forward on simulation results and methodology, Ericsson, RAN1#84, February 2016

TR36.881v0.6.0, Study on latency reduction techniques for LTE