3GPP TSG RAN WG1 Meeting #84bis 	R1-163526

# Busan, Korea 11th - 15th April 2016

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 623 | rev | 1 | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Simultaneous HARQ-ACK and P-CSI in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5c |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Editorial corrections for missing format 3 in 7.3.2.1 - The fallback to PUCCH format 2/2a/2b has prerequisite of no positive SR. - The fallback condition has been captured by“if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK and SR(if any) according to subclause 10.1.2.2.3 and there is no PUCCH format 5 determined to transmit the HARQ-ACK and SR(if any) according to subclause 10.1.2.2.4” in the main bullet. - Periodic CSI dropping when colliding with positive SR was not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe UE behavior for HARQ-ACK and P-CSI multiplexing when falls back to Rel-12 behavor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior for HARQ-ACK and P-CSI multiplexing is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1, 10.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.3.2.1	TDD HARQ-ACK reporting procedure for same UL/DL configuration

[...]

For TDD and a UE configured with PUCCH format 4 or PUCCH format 5, and if the UE has HARQ-ACK/SR and periodic CSI reports to transmit in a subframe,

- if a PUCCH format 3 is determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.3 or 10.1.3.2.4, the UE shall use the determined PUCCH format 3 for transmission of the HARQ-ACK/SR and  periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR.

- if a PUCCH format 4 is determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.3 or a PUCCH format 5 is determined to transmit the HARQ-ACK/SR according to 10.1.3.2.4, the UE shall use the determined PUCCH format 4 or PUCCH format 5 for transmission of the HARQ-ACK/SR and periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

- if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.4 and there are more than one periodic CSI reports in the subframe,

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a single PUCCH format 4 resource  according to higher layer parameter format4-MultiCSI-resourceConfiguration, the PUCCH format 4 resource  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a PUCCH format 5 resource according to higher layer parameter format5-MultiCSI-resourceConfiguration, the PUCCH format 5 resource is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with two PUCCH format 4 resources  and according to higher layer parameter format4-MultiCSI-resourceConfiguration, if , the PUCCH format 4 resource with the smaller  between  and  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s); otherwise, the PUCCH format 4 resource with the larger  between  and  is used for transmission of the HARQ-ACK/SR periodic CSI report(s), where

    - is the total number of HARQ-ACK bits in the subframe;

    - if there no scheduling request bit in the subframe and  otherwise

    - is the total number of CSI report bits in the subframe;

    - , , is the number of PRBs for  and respectively, according to higher layer parameter numberOfPRB-format4-r13 according to Table 10.1.1-2;

    - if shortened PUCCH format 4 is used in the subframe and  otherwise; and

    - is the code rate given by higher layer parameter maximumPayloadCoderate-r13 according to Table 10.1.1-1.

  - otherwise, the UE shall drop the periodic CSI reports and transmit only HARQ-ACK/SR.

- if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to subclause 10.1.3.2.4 and there is only one periodic CSI report in the subframe,

  - if there is no positive SR and the parameter simultaneousAckNackAndCQI is set TRUE and if the UE receives,

    - a single PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH/EPDCCH in subframe , where , and the counter DAI value in the PDCCH/EPDCCH is equal to '1' (defined in Table 7.3-X), or a PDCCH/EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where , and the counter DAI value in the PDCCH/EPDCCH is equal to '1', or

    - a single PDSCH transmission only on the primary cell where there is not a corresponding PDCCH/EPDCCH detected within subframe(s) , where  and no PDCCH/EPDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where ,

then the UE shall transmit the CSI and HARQ-ACK using PUCCH format 2/2a/2b according to subclause 5.2.3.4 in [4];

  - else,

the UE shall drop the CSI and transmit the HARQ-ACK according to subclause 10.1.3.2.3 or 10.1.3.2.4 when UE shall transmit  HARQ-ACK only or UE shall drop the CSI and transmit the HARQ-ACK and SR according to the procedure for PUCCH format 1b with channel selection in TDD when there is positive SR.

[...]

10.1.1	PUCCH format information

[…]

For FDD or for FDD-TDD and primary cell frame structure type 1, for a UE configured with PUCCH format 4 or PUCCH format 5, and if the UE has HARQ-ACK/SR and periodic CSI reports to transmit in a subframe,

- if a PUCCH format 3 is determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.3 or 10.1.2.2.4, the UE shall use the determined PUCCH format 3 for transmission of the HARQ-ACK/SR and  periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

- if a PUCCH format 4 is determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.3 or a PUCCH format 5 is determined to transmit the HARQ-ACK/SR according to 10.1.2.2.4, the UE shall used the determined PUCCH format 4 or PUCCH format 5 for transmission of the HARQ-ACK/SR and periodic CSI report(s) if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE; otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR;

- if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.4 and there are more than one periodic CSI report(s) in the subframe,

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a single PUCCH format 4 resource  according to higher layer parameter format4-MultiCSI-resourceConfiguration, the PUCCH format 4 resource  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with a PUCCH format 5 resource according to higher layer parameter format5-MultiCSI-resourceConfiguration, the PUCCH format 5 resource is used for transmission of the HARQ-ACK/SR and periodic CSI report(s);

  - if the parameter simultaneousAckNackAndCQI-Format4-Format5-r13 provided by higher layers is set TRUE and if the UE is configured with two PUCCH format 4 resources  and according to higher layer parameter format4-MultiCSI-resourceConfiguration, if , the PUCCH format 4 resource with the smaller  between  and  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s); otherwise, the PUCCH format 4 resource with the larger  between  and  is used for transmission of the HARQ-ACK/SR and periodic CSI report(s), where

    - is the total number of HARQ-ACK bits in the subframe;

    - if there no scheduling request bit in the subframe and  otherwise

    - is the total number of CSI report bits in the subframe;

    - , , is the number of PRBs for  and respectively, according to higher layer parameter numberOfPRB-format4-r13 according to Table 10.1.1-2;

    - if shortened PUCCH format 4 is used in the subframe and  otherwise; and

    - is the code rate given by higher layer parameter maximumPayloadCoderate-r13 according to Table 10.1.1-2.

  - otherwise, the UE shall drop the periodic CSI report(s) and transmit only HARQ-ACK/SR.

- if there is no PUCCH format 3 or 4 determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.3 and there is no PUCCH format 3 or 5 determined to transmit the HARQ-ACK/SR according to subclause 10.1.2.2.4 and there are only one periodic CSI report in the subframe,

  - if there is no positive SR and the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE and if the HARQ-ACK corresponds to a PDSCH transmission or PDCCH/EPDCCH indicating downlink SPS release only on the primary cell, then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using PUCCH format 2/2a/2b

  - else, the UE shall drop the CSI and transmit the HARQ-ACK according to subclause 10.1.2.2.3 or 10.1.2.2.4 when UE shall transmit HARQ-ACK only or UE shall drop the CSI and transmit the HARQ-ACK and SR according to the procedure for FDD with PUCCH format 1a/1b when there is positive SR.

[...]