3GPP TSG RAN WG1 Meeting #84bis	R1-163596

Busan, Korea, April 11-15, 2016

Agenda Item:	7.2.1.1.4

Source:	Huawei, HiSilicon

Title:	NB-SSS Evaluations

Document for:	Discussion and decision

# Introduction

In RAN1 meeting #84, it is agreed on NB-SSS design that [1]:

Agreements:

- NB-SSS is transmitted in subframe 9

- Number of symbols for NB-SSS: 11

- NB-SSS base sequence is constructed from one or more ZC sequences

  - Length FFS

  - FFS whether multiple root sequences are used or a binary scrambling code

It has been further agreed at RAN1#84bis that:

Agreements:

- NB-SSS periodicity is 20ms

- The following information is indicated by the NB-SSS:

  - One of 504 PCIDs

  - 80ms boundary (4 values)

- NB-SSS sequence is composed of a length-131 frequency-domain ZC sequence and a binary scrambling sequence

  - Root indices {3..128}

- PCID is indicated by combination of ZC root index and a binary scrambling sequence

- 80ms boundary is indicated by the one of 4 time-domain cyclic shifts, {0, 33, 66, 99}

- The binary scrambling sequence is a Hadamard sequence

  - Working assumption (from R1-163442), to be confirmed on Wed if no significant problems are identified:

In this contribution, the cross-correlation property of the updated NB-SSS design with scrambling sequences based on Hadamard matrix is evaluated. On this basis, we propose the final details needed for NB-SSS design.

# Evaluation of the cross-correlation property of the agreed NB-SSS design

## NB-SSS sequence design

According to the RAN1 agreements, the NB-SSS signal is generated by the multiplication of a cyclic shifted 131-length ZC sequence ZCu,m(n) and a binary scrambling code bn:

NB-SSSu,mn= ZCu,m(n)×bqn

where:

ZCu,mn=exp-jπumodn+m,NSSSmodn+m,NSSS+1NSSS

bqn=Hadamard2q-1128x128modn,128, q=0,1,2,3

n=0, 1,… , 130

u=3,… , 128

m=0, 39, 78, 117

q=0, 1, 2, 3

u is the root index of the ZC sequence, m is the cyclic shift applied to the original ZC sequence, and bqn is the q-th scrambling sequence generated based on 128x128 Hadamard matrix.

## NB-SSS generation procedure

The procedure of NB-SSS generation is similar with the procedure given in [2]. Firstly a multiplication operation is applied between the ZC sequence, with a certain root index u and a certain cyclic shift m, and the corresponding scrambling sequence. Secondly, add zeros to the result sequence to become a 132-length sequence. And then get 11 sub-sequences each with length of 12. After this, do resource mapping to the 12 subcarriers. Finally, do IFFT and add CP for each subsequence.

Figure 1 Generation procedure of NB-SSS

## Cross-correlation property of the NB-SSS

Figure 2 to Figure 4 and Table 1 to Table 3 show the cross-correlation property of the NB-SSS sequences considering synchronized BSs. The timing error ∆τ in the figures denotes the assumed offset between the received NB-SSS signals from two adjacent BSs. It is seen that the cross-correlation values between the proposed NB-SSS sequences is less than 0.23 in 99% cases

Figure 2 Cross-correlation property of the Hadamard based scrambling sequences (no sync error)

Table 1 Cross-correlation property of the Hadamard based scrambling sequences (no sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation | 0.077 | 0.128 | 0.146 | 0.179 |

Figure 3 Cross-correlation property of the Hadamard based scrambling sequences (small sync error)

Table 2 Cross-correlation property of the Hadamard based scrambling sequences (small sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation | 0.080 | 0.137 | 0.155 | 0.191 |

Figure 4 Cross-correlation property of the Hadamard based scrambling sequences (large sync error)

Table 3 Cross-correlation property of the Hadamard based scrambling sequences (large sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation | 0.095 | 0.166 | 0.187 | 0.230 |

Observation 1: The working assumption on scrambling sequences based on 128x128 Hadamard matrix can keep cross-correlation between different NB-SSS sequences less than 0.23 in 99% cases considering synchronized BSs.

Figure 5 and Table 4 show the cross-correlation property of the NB-SSS sequences considering unsynchronized BSs. Since BSs are unsynchronized with each other, the timing error between the two received NB-SSS signal can be assumed from 0 ms to the length of the NB-SSS signal. In this case, the maximum cross-correlation value among those corresponding to all the possible timing errors is recorded as the cross-correlation value for each pair of NB-SSS sequences. The sampling rate is 1.92 MHz  in the simulations.

Figure 5 Cross-correlation property of the Hadamard based scrambling sequences (traversing timing error)

Table 4 Cross-correlation property of the Hadamard based scrambling sequences (large sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation for the long sequence with time domain cyclic shift | 0.163 | 0.192 | 0.204 | 0.238 |

Observation 2: The working assumption on scrambling sequences based on 128x128 Hadamard matrix can keep cross-correlation between different NB-SSS sequences less than 0.24 in 99% cases considering unsynchronized BSs.

We have shown that the scrambling sequences based on the working assumption performs very well, so the working assumption should be confirmed.

Proposal 1: Confirm the working assumption that the scrambling sequence is generated based on 128x128 Hadamard matrix. The q-th  scrambling sequence is generated as

bqn=Hadamard2q-1128x128modn,128, q=0,1,2,3

# Mapping from PCI to NB-SSS sequence

The mapping function from NB-IoT cell ID to the root index and the index of the scrambling sequence needs to be agreed. It can be:

u=modPCI,126+3

q=PCI126

This mapping function can be simply realized in the UE.

Proposal 1: The mapping function from NB-IoT cell ID to the root index and the index of the scrambling sequence can follow the rules as:

u=modPCI,126+3

q=PCI126

# Mapping from SFN to NB-SSS sequence

The mapping from SFN to NB-SSS sequence needs to be agreed. The agreed set of cyclic shifts is C = {0, 33, 66, 99}. Ci (0≤i≤3) is applied to the NB-SSS signal in the (8*k + 2*i)-th frame, where 0≤k≤127.

Proposal 2: Cyclic shift Ci (0≤i≤3) is applied to the NB-SSS signal in the (8*k + 2*i)-th frame, where 0≤k≤127.

# Conclusions

In this contribution, we provide some evaluation results for the agreed NB-SSS design. The following proposals were made.

Proposal 1: Confirm the working assumption that the scrambling sequence is generated based on Hadamard matrix. The qth scrambling sequence is generated as

bqn=Hadamard2q-1128x128modn,128, q=0,1,2,3

Proposal 2: The mapping function from NB-IoT cell ID to the root index and the index of the scrambling sequence can follow the rules as:

u=modPCI,126+3

q=PCI126

Proposal 3: Cyclic shift Ci (0≤i≤3) is applied to the NB-SSS signal in the (8*k + 2*i)-th frame, where 0≤k≤127.

Observation 1: The proposed scrambling sequences based on 128x128 Hadamard matrix can keep cross-correlation between different NB-SSS sequences less than 0.23 in 99% cases considering synchronized BSs.

Observation 2: The proposed scrambling sequences based on 128x128 Hadamard matrix can keep cross-correlation between different NB-SSS sequences less than 0.24 in 99% cases considering unsynchronized BSs.

# References

R1-161806, “RAN1 Chairman’s Notes”, 3GPP RAN1 84#, Malta, February 2016.

# Annex

In this annex, for completeness, we show a comparison between the agreed long sequence and the short sequence from R1-163407.

Figure 6 Cross-correlation property of NB-SSSsequences (no sync error)

Table 5 Cross-correlation property of NB-SSSsequences (no sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation of Long-sequence | 0.077 | 0.128 | 0.146 | 0.179 |
| Cross-correlation of Short-sequence | 0.104 | 0.214 | 0.240 | 0.283 |

Figure 7 Cross-correlation property of NB-SSSsequences (small sync error)

Table 6 Cross-correlation property of NB-SSSsequences (small sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation of Long-sequence | 0.080 | 0.137 | 0.155 | 0.191 |
| Cross-correlation of Short-sequence | 0.105 | 0.213 | 0.241 | 0.287 |

Figure 4 Cross-correlation property of NB-SSSsequences (large sync error)

Table 3 Cross-correlation property of NB-SSSsequences (large sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation of Long-sequence | 0.095 | 0.166 | 0.187 | 0.230 |
| Cross-correlation of Short-sequence | 0.115 | 0.223 | 0.254 | 0.309 |

Figure 5 Cross-correlation property of NB-SSSsequences (traversing timing error)

Table 4 Cross-correlation property of NB-SSSsequences (large sync error)

| CDF | 50% | 90% | 95% | 99% |
| --- | --- | --- | --- | --- |
| Cross-correlation of Long-sequence with time domain cyclic shift | 0.163 | 0.192 | 0.204 | 0.238 |
| Cross-correlation of Short-sequence | 0.228 | 0.290 | 0.312 | 0.368 |