R1-163599

3GPP TSG RAN WG1 Meeting #84bis

Busan, Korea, April 11–15, 2016

Agenda item 7.2.1.2.5

WF on UL Narrowband DMRS update of R1-163591

Background

Some agreements for multi-tone:

DMRS symbol locations for multitone are the same as for LTE

For 3-tone case, 16 base sequences are provided (new sequences, not truncated versions of the 6-tone sequences)

For 6-tone case, 16 base sequences are provided

Define 3 cyclic shifts for the 3-tone case and 4 for the 6-tone case

No CSs for 12-tone case

Cyclic shifts are indicated by SIB and not indicated in DCI

No OCC

Sequences are QPSK in frequency domain

Proposal 1

16 base sequences for 3-tone case are defined as 𝑟_𝑢 (𝑛)=𝑒^(𝑗𝜑(𝑛)𝜋/4), 0≤𝑛≤2 with

| m | (0),(1),(2) |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 1 |
| 3 | 1 | -3 | 3 |
| 4 | 1 | -1 | -3 |
| 5 | 1 | -1 | -1 |
| 6 | 1 | -1 | 1 |
| 7 | 1 | -1 | 3 |
| 8 | 1 | 1 | -3 |
| 9 | 1 | 1 | -1 |
| 10 | 1 | 1 | 1 |
| 11 | 1 | 1 | 3 |
| 12 | 1 | 3 | -3 |
| 13 | 1 | 3 | -1 |
| 14 | 1 | 3 | 1 |
| 15 | 1 | 3 | 3 |

Proposal 2

For each of the 3-tone base DMRS sequence, three cyclic shifts are defined as 𝑟 ̅(𝑛)=𝑒^(𝑗 2𝜋/3 𝛼𝑛) 𝑟(𝑛) where 𝑟(𝑛) is a base sequence, 0≤𝑛<3 and 𝛼 is the cyclic shift 0≤𝛼<3.

Proposal 3

For each of the 6-tone base DMRS sequence, three cyclic shifts are defined as 𝑟 ̅(𝑛)=𝑒^(𝑗 2𝜋/6 𝛼𝑛) 𝑟(𝑛),  where r(n) is a base sequence, 0≤𝑛<5 and 𝛼 is the cyclic shift 𝛼={0, 1, 2, 4}

Proposed Working Assumption

16 base sequences for 6-tone case are defined as 𝑟_𝑢 (𝑛)=𝑒^(𝑗𝜑(𝑛)𝜋/4), 0≤𝑛≤5 with

Alt1

| m | (0), (1), (2), (3), (4), (5), |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| 1 | 1 | 1 | -1 | -3 | -3 | 3 |
| 2 | 1 | 1 | 3 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 | -1 | 1 | -3 |
| 4 | 1 | -1 | 3 | -3 | -1 | -1 |
| 5 | 1 | 3 | 1 | -1 | -1 | 3 |
| 6 | 1 | -3 | -3 | 1 | 3 | 1 |
| 7 | -1 | -1 | 1 | -3 | -3 | -1 |
| 8 | -1 | -1 | -1 | 3 | -3 | -1 |
| 9 | 3 | -1 | 1 | -3 | -3 | 3 |
| 10 | 3 | -1 | 3 | -3 | -1 | 1 |
| 11 | 3 | 3 | -3 | -3 | -3 | -1 |
| 12 | 3 | -3 | 3 | -1 | 3 | 3 |
| 13 | -3 | 1 | 3 | 1 | -3 | -1 |
| 14 | -3 | 1 | -3 | 3 | -3 | -1 |
| 15 | -3 | 3 | -3 | 1 | 1 | -3 |

Proposed Working Assumption

Alt2

Given in R1-163424

Proposed DM-RS multi-tone evaluation methods

First, prioritize cross-correlation

Preferred maximum pair-wise cross-correlation to be between 0.7 to 0.8

Second, check the cross-correlation between length-6 and length-3, 12 DM-RS sequences.

Third, check CM or PAPR according to

Alt1, Case 2 on next slide

Alt2, Case 4 on next slide compare to data only without DM-RS sequences

CM is calculated as

Cases of CM or PAPR evaluations