3GPP TSG RAN WG1 Meeting #84bis 	R1-163687

# Busan, Korea, 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | CRNum | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CSI-RS ID configuration for TM9 in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_EBF_FDMIMO-Core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In the current version of TS 36.331, multiple CSI-RS and CSI-IM resource configuration for TM9 is provided as per RAN1’s request. In contrast to the configuration, in section 7.2.5 of the current CR on TS 36.213, CSI-RS resource configuration identity is allowed only for the UE which is in TM10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of CSI-RS resource configuration identity for TM9 UE in class B with K>1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN1 agreements are not correctly captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.2.5	Channel-State Information – Reference Signal (CSI-RS) definition

For a serving cell and UE configured in transmission mode 9 and not configured with higher layer parameter eMIMO-Type, the UE can be configured with one CSI-RS resource configuration.

For a serving cell and UE configured in transmission mode 9 and configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to ‘CLASS A’, the UE can be configured with one CSI-RS resource configuration.

For a serving cell and UE configured in transmission mode 9 and configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to ‘CLASS B’, the UE can be configured with one or more CSI-RS resource configuration(s).

For a serving cell and UE configured in transmission mode 10, the UE can be configured with one or more CSI-RS resource configuration(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured via higher layer signaling for each CSI-RS resource configuration:

-	CSI-RS resource configuration identity, if the UE is configured in transmission mode 9 and configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to ‘CLASS B’, the UE is configured with more than one CSI-RS resource configurations, or if the UE is configured in transmission mode 10,

-	Number of CSI-RS ports.  The allowable values and port mapping are given in subclause 6.10.5 of [3].

-	CSI RS Configuration (see Table 6.10.5.2-1 and Table 6.10.5.2-2 in [3])

-	CSI RS subframe configuration . The allowable values are given in subclause 6.10.5.3 of [3].

-	UE assumption on reference PDSCH transmitted power for CSI feedback , if the UE is configured in transmission mode 9.

-	UE assumption on reference PDSCH transmitted power for CSI feedback  for each CSI process, if the UE is configured in transmission mode 10. If CSI subframe sets  and  are configured by higher layers for a CSI process, is configured for each CSI subframe set of the CSI process.

-	Pseudo-random sequence generator parameter, . The allowable values are given in [11].

-	CDM type parameter, if the UE is configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to ‘CLASS A’ for a CSI process. The allowable values are given in subclause 6.10.5.3 of [3].

-	Higher layer parameter qcl-CRS-Info-r11 for Quasi co-location type B UE assumption of CRS antenna ports and CSI-RS antenna ports with the following parameters, if the UE is configured in transmission mode 10:

-	qcl-ScramblingIdentity-r11.

-	crs-PortsCount-r11.

-	mbsfn-SubframeConfigList-r11.

…