3GPP TSG RAN WG1 Meeting #84bis 	R1-163705

# Busan, Korea 11th - 15th April 2016

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0241 | rev |  | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RE mapping in MBSFN subframe for BL/CE UEs in CEModeB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Panasonic, Intel, Sony, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-04-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For PDSCH transmission in TM9 and MPDCCH transmission in CE Mode B, UE assumes CRS presence in non-MBSFN subframes while no CRS in MBSFN subframes for rate matching. Therefore, when the PDSCH/MPDCCH repetition cross MBSFN and non-MBSFN subframes, I/Q combing cannot be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Specify that in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals shall not be counted in the PDSCH mapping and not used for transmission of the PDSCH for BL/CE UEs in CEModeB configured in transmission mode 9, in MBSFN subframe(s). 2. Specify that in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals shall not be counted in the MPDCCH mapping and not used for transmission of the MPDCCH for BL/CE UEs in CEModeB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | I/Q combining cannot supported for TM9 and MPDCCH in case the repetition occur in non-MBSFN subframe(s) and MBSFN subframe(s) for BL/CE UEs in CEModeB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6,8B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------- 1st change ------------------------------------------------------------------------

6.4.1	Physical downlink shared channel for BL/CE UEs

For BL/CE UEs, the following additions and exceptions hold in addition to those in clause 6.4:

-	For BL/CE UEs resource elements occupied by CSI reference signals shall be counted in the PDCSH mapping but not used for transmission of the PDSCH.

- For BL/CE UEs resource elements within PRBs containing PBCH repetitions shall be counted in the PDSCH mapping but not used for transmission of the PDSCH.

- For BL/CE UEs in CEModeB configured in transmission mode 9, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals shall not be counted in the PDSCH mapping and not used for transmission of the PDSCH.

--------------------------------------------------------- 2nd change ------------------------------------------------------------------------

6.8B.5	Mapping to resource elements

Mapping to resource elements shall be done according to Clause 6.8A.5with the following exceptions:

-	The term EPDCCH shall be replaced by MPDCCH.

-	The mapping shall be repeated across each of the  valid downlink subframes.

-	 is the number of ECCEs used for this MPDCCH in the first of the  subframes.

-	For an MPDCCH associated with a 2+4 PRB set as defined in [4], the mapping to resource elements  on antenna port  shall be in increasing order of first the index  and then the index over the 6 PRBs for MPDCCH format 5 and over the 2 or 4 PRBs for the other MPDCCH formats.

-	Resource elements occupied by CSI reference signals shall be counted in the MPDCCH mapping but not used for transmission of the MPDCCH. The UE may assume there is no MPDCCH transmission which uses overlapping sets of subframes as PDSCH transmissions to that UE, where the MPDCCH is located at a different narrowband than the PDSCH.

- For BL/CE UEs in CEModeB, in MBSFN subframe(s), resource elements that correspond to the positions of cell-specific reference signals shall not be counted in the MPDCCH mapping and not used for transmission of the MPDCCH.

--------------------------------------------------------- End of change --------------------------------------------------------------------