3GPP TSG RAN WG1 Meeting #84bis                                                                                 R1-163719

Seoul, Korea, 11th - 15th April 2016

Source: 	Ericsson

Title:	Text proposal on link evaluation results

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

# Introduction

This TP collects the link evaluation results for sPDCCH, sPDSCH, sPUSCH and sPUCCH.

# Proposal

Incorporate the following in the text proposal in the Latency reduction TR.

---------------------------------- Start of Text Proposal ------------------------------------------------------

Annex C:

Link-level evaluation results

In this Annex the link results are provided for sPDSCH, sPDCCH, sPUSCH and sPUCCH. The results are generated by the simulation assumptions available in Annex A if nothing else is described.

C1	Link-level evaluation sPDSCH

## C1.1	CRS based demodulation

Table C1.1-1 EPA, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 1.02 | 0.65 |  | 0.59 | 0.49 | 0.52 |
| 16QAM r3/4 | 13.72 | 13.50 |  | 13.87 | 13.71 | 13.62 |
| 64QAM r5/6 | 22.50 | 22.02 |  | 22.14 | 22.72 | 22.38 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.59 |  | 5.18 | 3.8 | 3.56 |
| 16QAM r3/4 |  | 15.52 |  | 16.55 | 14.86 | 14.31 |
| 64QAM r5/6 |  | 22.94 |  | 23.59 | 22.03 | 21.24 |
| R1-163399/ Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.05 |  | 4.53 | 3.93 | 3.56 |
| 16QAM r3/4 |  | 15.07 |  | 15.29 | 14.85 | 14.31 |
| 64QAM r5/6 |  | 21.97 |  | 22.73 | 21.91 | 21.24 |
| R1-163399/ Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.41 |  | 3.41 | 3.82 | 3.56 |
| 16QAM r3/4 |  | 14.46 |  | 14.43 | 14.85 | 14.31 |
| 64QAM r5/6 |  | 21.37 |  | 21.54 | 21.91 | 21.24 |
| R1-163399/ Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.4 |  |  | 1.3 | 0.7 |
| 16QAM r3/4 |  | 13.3 |  |  | 13.5 | 13 |
| 64QAM r5/6 |  | 20.3 |  |  | 21.1 | 20.4 |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-2 EPA, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 6.81 | 6.32 |  | 6.27 | 6.17 | 6.16 |
| 16QAM r3/4 | 23.13 | 22.79 |  | 23.31 | 23.07 | 22.93 |
| 64QAM r5/6 | 32.81 | 32.15 |  | 32.39 | 33.01 | 32.53 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 7 | 6.8 | -- | -- | 6.8 | 6.25 |
| 16QAM r3/4 | 23.5 | 23.2 | -- | -- | 23.1 | 23.2 |
| 64QAM r5/6 | 34.5 | 33.8 | -- | -- | 35.5 | 34 |
| R1-163511 / Qualcomm 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-3 EPA, 3 km/h, rank 4

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.55 | 3.44 |  | 3.42 | 3.32 | 3.46 |
| 16QAM r3/4 | 23.47 | 23.22 |  | 23.53 | 23.77 | 23.78 |
| 64QAM r5/6 | 35.46 | 34.78 |  | 34.93 | 34.75 | 35.10 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-4 EPA, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 1.12 | 0.69 |  | 0.50 | 0.38 | 0.15 |
| 16QAM r3/4 | 14.21 | 13.58 |  | 13.29 | 13.10 | 13.33 |
| 64QAM r5/6 | 22.98 | 22.04 |  | 21.53 | 22.19 | 22.21 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-5 EPA, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 6.66 | 6.10 |  | 5.86 | 5.67 | 5.45 |
| 16QAM r3/4 | 23.14 | 22.48 |  | 22.68 | 22.57 | 23.18 |
| 64QAM r5/6 | >36 | 35.03 |  | 32.76 | 34.26 | 34.68 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 7.5 | 7.5 | -- | -- | 7.5 | 6.25 |
| 16QAM r3/4 | >25 | >25 | -- | -- | >25 | >25 |
| 64QAM r5/6 | -- | -- | -- | -- | -- | -- |
| R1-163511 / Qualcomm 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-6 EVA, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.73 | -0.99 |  | -0.98 | -1.06 | -1.07 |
| 16QAM r3/4 | 12.02 | 11.92 |  | 12.16 | 12.02 | 12.06 |
| 64QAM r5/6 | 20.76 | 20.39 |  | 20.45 | 20.78 | 20.67 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-7 EVA, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.91 | 3.52 |  | 3.56 | 3.45 | 3.43 |
| 16QAM r3/4 | 20.13 | 20.00 |  | 20.36 | 20.08 | 20.08 |
| 64QAM r5/6 | 31.99 | 31.04 |  | 31.25 | 31.70 | 31.38 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-8 EVA, 3 km/h, rank 4

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 2.94 | 2.90 |  | 2.90 | 2.80 | 2.95 |
| 16QAM r3/4 | 22.17 | 22.06 |  | 22.16 | 22.16 | 22.10 |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-9 EVA, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.06 | -0.40 |  | -0.55 | -0.69 | -0.84 |
| 16QAM r3/4 | 12.74 | 12.41 |  | 12.27 | 12.09 | 12.15 |
| 64QAM r5/6 | 22.27 | 21.21 |  | 20.70 | 21.16 | 20.83 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.79 |  | 4.03 | 2.35 | 1.79 |
| 16QAM r3/4 |  | 14.61 |  | 15.2 | 13.41 | 13.03 |
| 64QAM r5/6 |  | 21.95 |  | 22.79 | 20.65 | 20.28 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.72 |  | 3.08 | 2.17 | 1.79 |
| 16QAM r3/4 |  | 14.02 |  | 14.35 | 13.65 | 13.03 |
| 64QAM r5/6 |  | 21.78 |  | 22.76 | 20.71 | 20.28 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.93 |  | 2.09 | 2.17 | 1.79 |
| 16QAM r3/4 |  | 13.12 |  | 13.65 | 13.65 | 13.03 |
| 64QAM r5/6 |  | 21.67 |  | 22.64 | 20.71 | 20.28 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |

Table C1.1-10 EVA, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.56 | 4.21 |  | 4.10 | 3.97 | 3.79 |
| 16QAM r3/4 | 21.11 | 20.59 |  | 20.70 | 20.42 | 20.47 |
| 64QAM r5/6 | >36 | 35.40 |  | 32.29 | 33.55 | 33.08 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-11 ETU, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | -0.71 | -1.04 |  | -1.06 | -1.13 | -1.18 |
| 16QAM r3/4 | 12.08 | 11.90 |  | 12.19 | 12.00 | 12.01 |
| 64QAM r5/6 | 23.52 | 22.79 |  | 22.93 | 23.62 | 23.30 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.26 |  | 3.42 | 1.71 | 1.3 |
| 16QAM r3/4 |  | 14.46 |  | 15.13 | 13.08 | 12.9 |
| 64QAM r5/6 |  | 21.66 |  | 22.36 | 20.59 | 20.14 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.26 |  | 2.61 | 1.77 | 1.3 |
| 16QAM r3/4 |  | 13.66 |  | 14.16 | 13.02 | 12.9 |
| 64QAM r5/6 |  | 20.59 |  | 21.36 | 20.5 | 20.14 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.37 |  | 1.48 | 1.77 | 1.3 |
| 16QAM r3/4 |  | 12.67 |  | 12.64 | 13.02 | 12.9 |
| 64QAM r5/6 |  | 19.94 |  | 20.06 | 20.5 | 20.14 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |

Table C1.1-12 ETU, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.20 | 3.80 |  | 3.79 | 3.65 | 3.60 |
| 16QAM r3/4 | 23.28 | 22.83 |  | 23.62 | 22.99 | 23.00 |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 4.4 | 4.3 | -- | -- | 4.2 | 3.75 |
| 16QAM r3/4 | 20.2 | 20 | -- | -- | 19.9 | 19.9 |
| 64QAM r5/6 | 29.8 | 29.8 | -- | -- | 31 | 30.5 |
| R1-163511 / Qualcomm 25 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |

Table C1.1-13 ETU, 3 km/h, rank 4

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 3.11 | 3.01 |  | 2.99 | 2.88 | 3.01 |
| 16QAM r3/4 | 27.26 | 27.08 |  | 27.99 | 28.62 | 29.97 |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |

Table C1.1-14 ETU, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 0.08 | -0.30 |  | -0.47 | -0.60 | -0.77 |
| 16QAM r3/4 | 12.88 | 12.57 |  | 12.46 | 12.22 | 12.21 |
| 64QAM r5/6 | 24.59 | 23.00 |  | 22.44 | 23.09 | 22.27 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.84 |  | 3.34 | 1.24 | 1.75 |
| 16QAM r3/4 |  | 14.42 |  | 14.97 | 13.1 | 12.85 |
| 64QAM r5/6 |  | 21.92 |  | 22.56 | 20.65 | 20.24 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.28 |  | 2.65 | 1.58 | 1.75 |
| 16QAM r3/4 |  | 13.54 |  | 14.24 | 13.03 | 12.85 |
| 64QAM r5/6 |  | 21.42 |  | 22.23 | 20.54 | 20.24 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.5 |  | 1.35 | 1.58 | 1.75 |
| 16QAM r3/4 |  | 12.81 |  | 13.06 | 13.03 | 12.85 |
| 64QAM r5/6 |  | 21.23 |  | 21.83 | 20.54 | 20.24 |
| R1-163399 / Sharp 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme if CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI. |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 |  |  | -0.2 | -0.4 |
| 16QAM r3/4 |  | 14.5 |  |  | 13.5 | 13.3 |
| 64QAM r5/6 |  |  |  |  | 21.8 | 21.5 |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | 22.3 |  |  |  | 21.2 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth. Channel estimation based on CRS Scheme for sTTI, the most recent four CRS symbols are used for channel estimation for 1ms TTI, CRS symbols in current subframe are used for channel estimation |  |  |  |  |  |  |

Table C1.1-15 ETU, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 | 4.62 | 4.25 |  | 4.13 | 4.00 | 3.72 |
| 16QAM r3/4 | 22.71 | 21.92 |  | 22.02 | 21.55 | 21.37 |
| 64QAM r5/6 | >36 | >36 |  | >36 | >36 | >36 |
| R1-163313 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 | 4.8 | 4.8 | -- | -- | 4.7 | 3.9 |
| 16QAM r3/4 | >25 | >25 | -- | -- | >25 | >25 |
| 64QAM r5/6 | -- | -- | -- | -- | -- | -- |
| R1-163511 / Qualcomm 25 PRB, 10 MHz system bandwidth, |  |  |  |  |  |  |

C1.2	DMRS based demodulation

Table C1.2-1 EPA, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 2.04 |  | 1.99 | 1.91 | 1.30 |
| 16QAM r3/4 |  | 13.95 |  | 13.88 | 14.21 | 13.95 |
| 64QAM r5/6 |  | 23.18 |  | 23.06 | 22.98 | 22.83 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.43 |  | 2.53 | 2.58 | 2.68 |
| 16QAM r3/4 |  | 13.81 |  | 13.77 | 13.81 | 14.02 |
| 64QAM r5/6 |  | 21.2 |  | 20.77 | 20.85 | 21.19 |
| R1-163400 / Sharp 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.01 |  |  |  |  |
| 16QAM r3/4 |  | 14.00 |  |  |  |  |
| 64QAM r5/6 |  | 23.63 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.25 |  | 1.16 | 1.04 | 0.66 |
| 16QAM r3/4 |  | 13.58 |  | 13.56 | 13.88 | 13.70 |
| 64QAM r5/6 |  | 22.62 |  | 22.55 | 22.45 | 22.41 |
| R1-163410 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.20 |  |  |  |  |
| 16QAM r3/4 |  | 13.63 |  |  |  |  |
| 64QAM r5/6 |  | 22.94 |  |  |  |  |
| R1-163410/ Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.6 |  |  | 1.5 | 0.7 |
| 16QAM r3/4 |  | 13.4 |  |  | 13.7 | 13 |
| 64QAM r5/6 |  | 21.5 |  |  | 23.8 | 20.2 |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth DM-RS available in every TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.5 |  |  |  |  |
| 16QAM r3/4 |  | 13.4 |  |  |  |  |
| 64QAM r5/6 |  | 21.5 |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth DMRS density 6RE per PRB in sTTI /12RE per PRB in TTI DMRS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.8 |  |  |  |  |
| 16QAM r3/4 |  | 13.8 |  |  |  |  |
| 64QAM r5/6 |  | 26.8 |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS available in every TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.8 |  |  |  |  |
| 16QAM r3/4 |  | 13.8 |  |  |  |  |
| 64QAM r5/6 |  | 25.8 |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.1 |  |  |  | 0.0 |
| 16QAM r3/4 |  | 12.2 |  |  |  | 12.9 |
| 64QAM r5/6 |  | 20.9 |  |  |  | 20.9 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  | -0.5 (DMRS: 2RE/RB) -1.3 (DMRS: 4RE/RB) -1.6 (DMRS: 6RE/RB) -1.3 (DMRS: 8RE/RB) |  | -0.4 (DMRS: 2RE/RB) -1.3 (DMRS: 4RE/RB) -1.7 (DMRS: 6RE/RB) -1.7 (DMRS: 8RE/RB) |  |
| 16QAM r3/4 |  |  | 8.3 (DMRS: 2RE/RB) 8.3 (DMRS: 4RE/RB) 8.8 (DMRS: 6RE/RB) 9.5 (DMRS: 8RE/RB) |  | 9.8 (DMRS: 2RE/RB) 9.3 (DMRS: 4RE/RB) 9.4 (DMRS: 6RE/RB) 9.7 (DMRS: 8RE/RB) |  |
| 64QAM r5/6 |  |  | 14.2 (DMRS: 2RE/RB) 14.4 (DMRS: 4RE/RB) 15.0 (DMRS: 6RE/RB) 16.4 (DMRS: 8RE/RB) |  | 16.3 (DMRS: 2RE/RB) 16.1 (DMRS: 4RE/RB) 16.3 (DMRS: 6RE/RB) 16.8 (DMRS: 8RE/RB) |  |
| R1-162505 / LG Electronics 25 PRB, 10 MHz system bandwidth No short PDCCH region within sTTI Precoding matrix determination based on CSI feedback TB sizes for evaluation are obtained by targeting 1/3, 3/4, and 5/6 code rate when the maximum RS overhead is assumed (i.e., CRS and DMRS with 8 REs/PRB) |  |  |  |  |  |  |

Table C1.2-2 EPA, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 8.27 |  | 8.17 | 8.01 | 7.06 |
| 16QAM r3/4 |  | 23.97 |  | 23.89 | 24.34 | 23.98 |
| 64QAM r5/6 |  | 34.79 |  | 34.68 | 34.55 | 35.11 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 8.22 |  |  |  |  |
| 16QAM r3/4 |  | 24.05 |  |  |  |  |
| 64QAM r5/6 |  | 35.36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.13 |  | 7.00 | 6.85 | 6.42 |
| 16QAM r3/4 |  | 23.30 |  | 23.20 | 23.72 | 23.41 |
| 64QAM r5/6 |  | 33.42 |  | 33.39 | 33.27 | 33.45 |
| R1-163410/ Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.07 |  |  |  |  |
| 16QAM r3/4 |  | 23.37 |  |  |  |  |
| 64QAM r5/6 |  | 33.92 |  |  |  |  |
| R1-163410/ Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI PRBs bundling over three 3PBRs DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.9 |  |  |  | 5.6 |
| 16QAM r3/4 |  | 22.1 |  | 22.1 |  | 22.2 |
| 64QAM r5/6 |  | 32.5 |  |  |  | 32.7 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-3 EPA, 3 km/h, rank 4

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  |  |  |  |  |
| 16QAM r3/4 |  | 12.4 |  | 12.4 |  | 12.7 |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 10 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-4 EPA, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.46 |  | 1.41 | 1.32 | 0.84 |
| 16QAM r3/4 |  | 13.20 |  | 13.38 | 14.04 | 14.10 |
| 64QAM r5/6 |  | 22.45 |  | 29.25 | >36 | 23.67 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.46 |  |  |  |  |
| 16QAM r3/4 |  | 13.46 |  |  |  |  |
| 64QAM r5/6 |  | 25.74 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.2 |  |  |  | 0.3 |
| 16QAM r3/4 |  | 13.0 |  |  |  | 14.2 |
| 64QAM r5/6 |  | 24.2 |  |  |  | >28 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-5 EPA, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 7.49 |  | 7.49 | 7.34 | 6.65 |
| 16QAM r3/4 |  | 23.11 |  | 26.02 | >36 | 24.92 |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 7.52 |  |  |  |  |
| 16QAM r3/4 |  | 25.87 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.1 |  |  |  | 6.2 |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  | >36 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-6 EVA, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.35 |  | 0.37 | 0.33 | -0.35 |
| 16QAM r3/4 |  | 12.37 |  | 12.39 | 12.70 | 12.27 |
| 64QAM r5/6 |  | 21.72 |  | 21.48 | 21.52 | 21.51 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.34 |  |  |  |  |
| 16QAM r3/4 |  | 12.43 |  |  |  |  |
| 64QAM r5/6 |  | 22.06 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.3 |  |  |  | -0.7 |
| 16QAM r3/4 |  | 11.8 |  |  |  | 11.6 |
| 64QAM r5/6 |  | 20.6 |  |  |  | 20.0 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-7 EVA, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 5.33 |  | 5.34 | 5.23 | 4.43 |
| 16QAM r3/4 |  | 21.35 |  | 21.41 | 21.91 | 21.41 |
| 64QAM r5/6 |  | >36 |  | 34.95 | 35.11 | >36 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.29 |  |  |  |  |
| 16QAM r3/4 |  | 21.49 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |

Table C1.2-8 EVA, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.25 |  | 0.27 | 0.21 | -0.35 |
| 16QAM r3/4 |  | 12.30 |  | 12.46 | 12.97 | 12.92 |
| 64QAM r5/6 |  | 22.07 |  | 25.65 | >36 | 22.98 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.86 |  | 0.97 | 0.92 | 0.96 |
| 16QAM r3/4 |  | 12.48 |  | 12.5 | 12.98 | 12.59 |
| 64QAM r5/6 |  | 19.87 |  | 20.71 | - | 19.85 |
| R1-163400 / Sharp 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.24 |  |  |  |  |
| 16QAM r3/4 |  | 12.51 |  |  |  |  |
| 64QAM r5/6 |  | 25.34 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.4 |  |  |  | -0.4 |
| 16QAM r3/4 |  | 12.1 |  |  |  | 13.0 |
| 64QAM r5/6 |  | 23.6 |  |  |  | >28 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-9 EVA, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 5.54 |  | 5.64 | 5.52 | 4.77 |
| 16QAM r3/4 |  | 21.71 |  | 23.32 | 26.84 | 22.89 |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.55 |  |  |  |  |
| 16QAM r3/4 |  | 23.61 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |

Table C1.2-10 ETU, 3 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.42 |  | 0.39 | 0.31 | -0.51 |
| 16QAM r3/4 |  | 12.98 |  | 12.95 | 13.27 | 12.65 |
| 64QAM r5/6 |  | 25.86 |  | 25.33 | 25.35 | 24.52 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.38 |  | 0.41 | 0.44 | 0.56 |
| 16QAM r3/4 |  | 12.1 |  | 12.07 | 12.1 | 12.3 |
| 64QAM r5/6 |  | 19.22 |  | 19.27 | 19.34 | 19.5 |
| R1-163400/ Sharp 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.39 |  |  |  |  |
| 16QAM r3/4 |  | 13.04 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.5 |  |  |  | -0.9 |
| 16QAM r3/4 |  | 11.8 |  | 11.8 |  | 11.5 |
| 64QAM r5/6 |  | 20 |  |  |  | 19.8 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-11 ETU, 3 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 6.14 |  | 6.12 | 6.03 | 4.75 |
| 16QAM r3/4 |  | 25.46 |  | 25.17 | 25.87 | 24.79 |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.09 |  |  |  |  |
| 16QAM r3/4 |  | 26.45 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5 |  |  |  | 4.8 |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  | >36 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

Table C1.2-12 ETU, 60 km/h, rank 1

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.39 |  | 0.39 | 0.34 | -0.29 |
| 16QAM r3/4 |  | 12.93 |  | 13.10 | 13.66 | 13.44 |
| 64QAM r5/6 |  | 26.17 |  | >36 | >36 | 28.17 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.4 |  | 0.41 | 0.64 | 0.43 |
| 16QAM r3/4 |  | 12 |  | 12.13 | 12.52 | 12.13 |
| 64QAM r5/6 |  | 19.38 |  | 20.03 | - | 19.53 |
| R1-163400/ Sharp 50 PRB, 10 MHz system bandwidth |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.38 |  |  |  |  |
| 16QAM r3/4 |  | 13.19 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 |  |  | -0.2 | -0.4 |
| 16QAM r3/4 |  | 11.9 |  |  | 13.1 | 13.3 |
| 64QAM r5/6 |  | 19.8 |  |  |  | 21.5 |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth DM-RS available in every TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0 |  |  |  |  |
| 16QAM r3/4 |  | 12.5 |  |  |  |  |
| 64QAM r5/6 |  | 23.5 |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth DMRS only available in every second TTI DMRS density 6RE per PRB in sTTI/12RE per PRB in TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.5 |  |  |  |  |
| 16QAM r3/4 |  | 12.5 |  |  |  |  |
| 64QAM r5/6 |  | 21.9 |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DMRS available in every TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.5 |  |  |  |  |
| 16QAM r3/4 |  | 13 |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163264 / Nokia 50 PRB, 10 MHz system bandwidth Bundled pattern over 3 PRBs (total 12RE per 3PRB) DM-RS only available in every second TTI Channel estimation performed with 3PRB granularity |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | -0.6 |  |  |  | -0.6 |
| 16QAM r3/4 |  | 11.6 |  |  |  | 12.3 |
| 64QAM r5/6 |  | 23 |  |  |  | >28 |
| R1-163774 / Huawei 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 5 PRBs bundling for sTTI, 3 PRBs bundling for 1ms TTI. |  |  |  |  |  |  |

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  |  | 2.1 (DMRS: 2RE/RB) 0.5 (DMRS: 4RE/RB) 0.1 (DMRS: 6RE/RB) 0.1 (DMRS: 8RE/RB) |  | 2.6 (DMRS: 2RE/RB) 0.8 (DMRS: 4RE/RB) 0.2 (DMRS: 6RE/RB) 0.1 (DMRS: 8RE/RB) |  |
| 16QAM r3/4 |  |  | >25.0 (DMRS: 2RE/RB) 10.5 (DMRS: 4RE/RB) 10.6 (DMRS: 6RE/RB) 11.5 (DMRS: 8RE/RB) |  | >25.0 (DMRS: 2RE/RB) 12.2 (DMRS: 4RE/RB) 12.0 (DMRS: 6RE/RB) 12.1 (DMRS: 8RE/RB) |  |
| 64QAM r5/6 |  |  | >30.0 (DMRS: 2RE/RB) 17.3 (DMRS: 4RE/RB) 17.3 (DMRS: 6RE/RB) 20.0 (DMRS: 8RE/RB) |  | >30.0 (DMRS: 2RE/RB) 24.6 (DMRS: 4RE/RB) 22.8 (DMRS: 6RE/RB) 25.4 (DMRS: 8RE/RB) |  |
| R1-162505 / LG Electronics 25 PRB, 10 MHz system bandwidth No short PDCCH region within sTTI Precoding matrix determination based on CSI feedback TB sizes for evaluation are obtained by targeting 1/3, 3/4, and 5/6 code rate when the maximum RS overhead is assumed (i.e., CRS and DMRS with 8 REs/PRB) |  |  |  |  |  |  |

Table C1.2-13 ETU, 60 km/h, rank 2

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 6.03 |  | 6.14 | 6.03 | 6.07 |
| 16QAM r3/4 |  | 25.38 |  | 28.61 | >36 | >36 |
| 64QAM r5/6 |  | >36 |  | >36 | >36 | >36 |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 6.03 |  |  |  |  |
| 16QAM r3/4 |  | >36 |  |  |  |  |
| 64QAM r5/6 |  | >36 |  |  |  |  |
| R1-163314 / Ericsson 50 PRB, 10 MHz system bandwidth, RX imperfections and 6% TX EVM, 2CCE (72RE) reserved for short PDCCH transmission each short TTI DM-RS only available in every second TTI |  |  |  |  |  |  |

C2	Link-level evaluation sPDCCH

Table C2-1 EPA 3km/h, 50 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 4.0844 | 0.24589 | -2.963 |  |
| 70 bits | 11.9942 | 3.3731 | 0.003019 |  |
| R1-163305 / Ericsson |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 4.6 | 1.2 | -1.8 | -4.7 |
| 70 bits | 12.6 | 4 | 1.4 | -2.1 |
| R1-162401 / ZTE |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 | -0.7 | -3.6 | -6.3 |
| R1-162506 / LG Electronics distributed PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.2 | 1.6 | -1.8 | -5.5 |
| R1-162506 / LG Electronics localized PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.3 | 1.8 | -1.3 | -4.6 |
| R1-162506 / LG Electronics localized PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 2.9 | -0.7 | -3.6 | -6.3 |
| R1-162506 / LG Electronics distributed PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.4 | 1.8 | -1.0 | -4.0 |
| R1-162506 / LG Electronics localized PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 | -0.7 | -3.7 | -6.3 |
| R1-162506 / LG Electronics distributed PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.5 | 2.1 | -0.8 | -3.4 |
| R1-162506 / LG Electronics localized PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 2.9 | -0.7 | -3.6 | -6.2 |
| R1-162506 / LG Electronics distributed PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.0 (DMRS: 2RE/RB) >30 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | 0.6 (DMRS: 2RE/RB) 1.2 (DMRS: 4RE/RB) >30 (DMRS: 6RE/RB) | -1.7 (DMRS: 2RE/RB) -1.8 (DMRS: 4RE/RB) 0.6 (DMRS: 6RE/RB) | -3.8 (DMRS: 2RE/RB) -4.2 (DMRS: 4RE/RB) -2.6 (DMRS: 6RE/RB) |
| R1-162504 / LG Electronics sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) DMRS, distributed PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.6 (DMRS: 2RE/RB) 4.7 (DMRS: 4RE/RB) 6.5 (DMRS: 6RE/RB) | 0.4 (DMRS: 2RE/RB) -0.3 (DMRS: 4RE/RB) 0.1 (DMRS: 6RE/RB) | -1.8 (DMRS: 2RE/RB) -2.8 (DMRS: 4RE/RB) -2.8 (DMRS: 6RE/RB) | -3.9 (DMRS: 2RE/RB) -5.0 (DMRS: 4RE/RB) -5.3 (DMRS: 6RE/RB) |
| R1-162504 / LG Electronics sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.0 (DMRS: 2RE/RB) 1.6 (DMRS: 4RE/RB) 2.1 (DMRS: 6RE/RB) | 0.2 (DMRS: 2RE/RB) -1.0 (DMRS: 4RE/RB) -1.4 (DMRS: 6RE/RB) | -1.6 (DMRS: 2RE/RB) -3.0 (DMRS: 4RE/RB) -3.6 (DMRS: 6RE/RB) | -3.6 (DMRS: 2RE/RB) -5.2 (DMRS: 4RE/RB) -5.9 (DMRS: 6RE/RB) |
| R1-162504 / LG Electronics sPDCCH transmission using 36xAL REs except CRS resource (including DMRS resource) distributed PRBs, 7 symbol sPDCCH |  |  |  |  |

Table C2-2 EVA 60km/h, 25 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 2.8008 | -0.96088 | -3.5526 |  |
| 70 bits | 12.1822 | 2.0945 | -1.3566 |  |
| R1-163305 / Ericsson |  |  |  |  |

Table C2-2 ETU 60km/h, 50 PRBs, CRS

|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| --- | --- | --- | --- | --- |
| 40 bits | 4.6 | 1 | -2 | -5.1 |
| 70 bits | 13.8 | 3.6 | 0.9 | -2.5 |
| R1-162401 / ZTE |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.2 | 1.2 | -1.8 | -4.6 |
| R1-162506 / LG Electronics localized PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.2 | -2.5 | -4.8 |
| R1-162506 / LG Electronics distributed PRBs, 1 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 5.8 | 1.8 | -0.9 | -4.0 |
| R1-162506 / LG Electronics localized PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.3 | -2.3 | -4.6 |
| R1-162506 / LG Electronics distributed PRBs, 2 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.4 | 2.4 | -0.5 | -3.4 |
| R1-162506 / LG Electronics localized PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.2 | -2.5 | -4.8 |
| R1-162506 / LG Electronics distributed PRBs, 3 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 6.3 | 3.2 | 0.2 | -2.5 |
| R1-162506 / LG Electronics localized PRBs, 7 symbol sPDCCH |  |  |  |  |
|  | 1 CCE | 2 CCEs | 4 CCEs | 8 CCEs |
| 40 bits | 3.8 | 0.5 | -1.8 | -3.9 |
| R1-162506 / LG Electronics distributed PRBs, 7 symbol sPDCCH |  |  |  |  |

C3	Link-level evaluation sPUSCH

Table C3-1 EPA, 3 km/h, 25 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 2.69 | 2.06 | 1.89 | 1.84 | 1.61 |
| 16QAM r3/4 |  | 13.97 | 13.79 | 13.86 | 13.66 | 13.63 |
| 64QAM r5/6 |  | 20.94 | 20.81 | 20.80 | 20.94 | 20.78 |
| R1-163316 / Ericsson DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.43 |  |  |  |  |
| 16QAM r3/4 |  | 13.84 |  |  |  |  |
| 64QAM r5/6 |  | 20.88 |  |  |  |  |
| R1-163316 / Ericsson DM-RS included in every second sTTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.29 |  |  |  |  |
| 16QAM r3/4 |  | 13.83 |  |  |  |  |
| 64QAM r5/6 |  | 20.88 |  |  |  |  |
| R1-163316 / Ericsson DM-RS included in third sTTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.72 | 1.22 | 1.72 | 0.76 | 0.97 |
| 16QAM r3/4 |  | 13.90 | 13.02 | 13.46 | 12.68 | 13.01 |
| 64QAM r5/6 |  | 19.76 | 19.14 | 20.3 | 19.61 | 19.68 |
| R1-162402 / ZTE |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.05 |  | 1.95 | 2.27 | 2.19 |
| 16QAM r3/4 |  | 13.72 |  | 13.72 | 13.69 | 13.53 |
| 64QAM r5/6 |  |  |  |  |  |  |
| 163401/ Sharp |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 2.69 |  |  |  |  |
| 16QAM r3/4 |  | 13.97 |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Ericsson Two UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.67 |  |  |  |  |
| 16QAM r3/4 |  | 24.10 |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Ericsson Three UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 4.67 |  |  |  |  |
| 16QAM r3/4 |  | - |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Ericsson Four UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 5.26 |  |  |  |  |
| 16QAM r3/4 |  | - |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163411 / Ericsson Five UEs share the same DM-RS |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.35 | 0.56 | 0.57 | 0.56 | 0.78 |
| 16QAM r3/4 |  | 13.11 | 13.30 | 13.25 | 13.12 | 13.60 |
| 64QAM r5/6 |  | 21.02 | 21.00 | 20.69 | 20.84 | 21.65 |
| R1-162509 / LGE DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | 0.581 |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162509 / LGE Two UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-2 EPA, 3 km/h, 50 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 3.9 |  |  | 4.2 | 3.7 |
| 16QAM r3/4 |  | 15.4 |  |  | 15.9 | 15.7 |
| 64QAM r5/6 |  | 22.3 |  |  | 23.1 | 22.9 |
| R1-160779 / Nokia DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.9 |  |  |  |  |
| 16QAM r3/4 |  | 15.5 |  |  |  |  |
| 64QAM r5/6 |  | 22.3 |  |  |  |  |
| R1-160779 / Nokia DM-RS included in every third sTTI |  |  |  |  |  |  |

Table C3-3 EVA, 60 km/h, 25 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.48 | 1.20 | 1.10 | 0.94 | 0.46 |
| 16QAM r3/4 |  | 13.64 | 13.47 | 13.67 | 13.63 | 13.47 |
| 64QAM r5/6 |  | 21.22 | 21.92 | 24.30 | 25.07 | 24.60 |
| R1-163316 / Ericsson |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.33 |  | 0.39 | 0.3 | 0.51 |
| 16QAM r3/4 |  | 12.87 |  | 13.42 | 13.19 | 12.66 |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401/ Sharp |  |  |  |  |  |  |

Table C3-4 ETU, 3 km/h, 25 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.06 |  | 0.11 | - 0.12 | 0.22 |
| 16QAM r3/4 |  | 12.68 |  | 12.59 | 12.58 | 12.56 |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401 / Sharp |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.693 |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162509 / LGE DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.682 |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162509 / LGE Two UEs share the same DM-RS |  |  |  |  |  |  |

Table C3-5 ETU, 60 km/h, 25 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 0.91 | 1.17 | 1.24 | 1.18 | 1.01 |
| 16QAM r3/4 |  | 12.5 | 13 | 13.2 | 13.05 | 13.3 |
| 64QAM r5/6 |  | 19.30 | 20.25 | 23.74 | - | 20.50 |
| R1-162402 / ZTE |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 0.2 |  | 0.17 | 0.16 | 0.17 |
| 16QAM r3/4 |  | 13.03 |  | 13.29 | 13.01 | 12.65 |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163401 / Sharp |  |  |  |  |  |  |

Table C3-6 ETU, 60 km/h, 50 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 3.5 |  |  | 3.3 | 3.2 |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-160779 / Nokia DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 3.5 |  |  |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-160779 / Nokia DM-RS included in every third sTTI |  |  |  |  |  |  |

Table C3-7 ETU, 120 km/h, 25 PRB

| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK r1/3 |  | 1.84 | 1.67 | 1.65 | 1.53 | 1.15 |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-163316 / Ericsson |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  | 1.13 | 1.26 | 1.46 | 1.40 | 1.12 |
| 16QAM r3/4 |  | 13.02 | 14.04 | 15.62 | 15.24 | 13.78 |
| 64QAM r5/6 |  | 21.58 | - | - | - | 22.11 |
| R1-162402 / ZTE |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.566 |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162509 / LGE DM-RS included in every TTI |  |  |  |  |  |  |
| SNR at 10% BLER | TTI length=1 | TTI length=2 | TTI length=3 | TTI length=4 | TTI length=7 | TTI length=14 |
| QPSK r1/3 |  |  |  | -0.561 |  |  |
| 16QAM r3/4 |  |  |  |  |  |  |
| 64QAM r5/6 |  |  |  |  |  |  |
| R1-162509 / LGE Two UEs share the same DM-RS |  |  |  |  |  |  |

C4	Link-level evaluation sPUCCH

Table C4-1 EPA, 1bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | 0 | -2.3 | -8 |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-2 EPA, 1bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | 0 | -2.3 | -8 |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-3 EPA, 2bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | 2 | -0.3 |  |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 4 symbols PUCCH format 1a/1b without frequency hopping 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  | 5.88 |  | 2.92 | 0.14 | -5.46 |
| R1-163402 / Sharp Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | 0.19 | -2.32 | -5.46 |
| R1-163402 / Sharp With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |

Table C4-4 EPA, 2bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | 2 | -0.3 |  |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 4 symbols PUCCH format 1a/1b without frequency hopping 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-5 ETU, 1bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -0.8 | -2.9 | -7.8 |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| 6.1 |  | -0.6 | -2.5 |  |  |
| R1-163520 / LG Electronics Sequence-based PUCCH details in R1-163520 |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  | 1.7 | -2.5 |  |  |
| R1-163520 / LG Electronics sPUCCH (similar to legacy) details in R1-163520 |  |  |  |  |  |

Table C4-6 ETU, 1bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0 |  | -0.8 | -2.9 | -7.8 |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |

Table C4-7 ETU, 2bit, 3km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | 1.7 | -0.7 |  |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  | 4.31 |  | 1.23 | -1.4 |  |
| R1-163402 / Sharp Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | -0.56 | -2.98 | -6.34 |
| R1-163402 / Sharp With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |

Table C4-8 ETU, 2bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.5 |  | 1.8 | -0.4 |  |
| R1-162399 / ZTE 2 symbols PUCCH details in R1-162403 7 symbols PUCCH format 1a/1b without frequency hopping |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  | 4.17 |  | 1.13 | -1.5 |  |
| R1-163402 / Sharp Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | -0.6 | -2.28 | -5.96 |
| R1-163402 / Sharp With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |

Table C4-9 EVA, 2bit, 60 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 5.38 |  | 2.53 | -0.37 |  |
| R1-163402 / Sharp Without frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |
| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
|  |  |  | 0.24 | -2.32 | -5.76 |
| R1-163402 / Sharp With frequency hopping Reuse legacy PUCCH structure |  |  |  |  |  |

Table C4-10 ETU, 2 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 0.9 | -0.5 |  |  |  |
| R1-162508 / LG Electronics sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

Table C4-11 ETU, 4 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 3.1 | 1.1 |  |  |  |
| R1-162508 / LG Electronics sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

Table C4-12 ETU, 7 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 5 | 2.8 |  |  |  |
| R1-162508 / LG Electronics sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

Table C4-14 ETU, 11 bits, 3 km/h

| 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- |
|  | 7.6 | 4.5 |  |  | -1.9 |
| R1-162508 / LG Electronics sPUCCH for CSI details in R1-162508 |  |  |  |  |  |

----------------------------------------------- End of Text Proposal ------------------------------------------------