3GPP TSG RAN WG1 Meeting #84bis                                                                                 R1-163720

Busan, 11th – 15th April 2016

Source: 	Ericsson

Title:	Text proposal on observation from link evaluations

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

# Introduction

This TP forms the observations based on the link evaluations results provided for shorter TTI operation.

# Proposal

Incorporate the following in the text proposal in the Latency reduction TR.

---------------------------------- Start of Text Proposal ------------------------------------------------------

## 9.5	Findings from link evaluations on TTI reduction and reduced processing time

In this section the findings from Annex C is outlined for the channels sPDCCH, sPDSCH, sPUSCH and sPUCCH.

## 9.5.1		sPDSCH

For the CRS based transmission mode, it is shown by five sources for rank one channel that the difference between the BLER performance of short TTI lengths and the one of a subframe long TTI is tolerable. Further evaluations are provide for rank two by two sources and rank four channels by one source , the results show a tolerable BLER performance difference.

For DMRS based transmission mode, it is shown by four sources for a rank one channel that the BLER performance difference for a shorter TTI operation is tolerable compared to subframe long TTI. Further two sources showed with a rank two channel that the BLER performance is tolerable when comparing shortened TTI and a subframe long TTI. One source provide results for rank four channel. Two sources evaluated for a TTI length of 2 OFDM symbols a scenario with increased PRB bundling size to 3 and a scenario where DM-RS is not available in adjacent multiple scheduled short TTIs. The results showed that the BLER performance could be maintained. Further one source evaluated increase of PRB bundling size to 5 and showed that the BLER performance is maintained. This will lead to higher data rate as the overhead is reduced. One source compared different DM-RS patterns and three sources assumed new DM-RS pattern for particular the shorter TTI lengths.

## 9.5.2		sPDCCH

Link evaluations for sPDCCH operation based on CRS demodulation has been performed by three sources and showed that BLER performance is tolerable. Further one source showed evaluation results for DM-RS based demodulation and showed that depending on the allocation scheme tolerable BLER performance can be achieved.

## 9.5.3		sPUSCH

Five sources showed that the BLER performance was tolerable with shorter TTI length compared a subframe long TTI. In some cases the results showed that the shorter the TTI is the better the BLER performance is. Two sources evaluated DMRS multiplexing. It was shown that at least two or three UEs can be multiplexed. Two sources evaluated short TTIs of length of 2 OFDM symbols with DM-RS not being available in multiple adjacent scheduled short TTIs, it is shown that having DM-RS in every second or third short TTI results in tolerable BLER performance.

## 9.5.4		sPUCCH

All sources with sPUCCH evaluation show that the performance of sPUCCH degrades generally with shorter TTI length due to the amount of energy transmitted becoming smaller and smaller. Further the results show that for HARQ-ACK payload size of 1 to 4 bits frequency hopping is beneficial. One sourced showed BLER performance for sPUCCH targeting CSI reporting.

----------------------------------------------- End of Text Proposal ------------------------------------------------