3GPP TSG RAN WG1 Meeting #84 bis	R1-163798

Busan, KR, 11th – 15th April 2016

Agenda Item:		7.3.10.1

Source:			MediaTek Inc.

Title:		System-level performance evaluation for TCP throughput enhancement with dynamic TTI tuning

Document for:		Discussion and decision

# Introduction

In RAN#67, the study item on latency reduction was approved [1] with the objective as

- Study enhancements to the E-UTRAN radio system in order to:

  - Significantly reduce the packet data latency over the LTE Uu air interface for an active UE

  - Significantly reduce the packet data transport round trip latency for UEs that have been inactive for a longer period (in connected state).

RAN2 has discussed L2 aspects in order to reduce the latency with evaluation results [2]. These evaluation results provide essential insight that latency reduction gain from shortened TTI heavily depends heavily on the evaluated scenarios, e.g. shortened TTI may bring little or even negative latency reduction gain when in the scenarios of a large file size and low Uu throughput.

. To guarantee positive latency reduction gain, we suggest initiating the study of dynamic TTI methods, which aim to adaptively configure TTI according to the applied scenario and accordingly adopt smart TTI selection.

In this contribution, we provide the motivation and rationale to have dynamic TTI schemes, along with evaluation results for fixed TTI schemes (using a fixed TTI, i.e., legacy TTI or shortened TTI) and the proposed dynamic TTI methods.

# Discussion

## L1/L2 control overhead for shortened TTI

Latency Reduction (LATRED) techniques has been agreed as a SI (RP-150465) in Release-14. To further reduce file transfer latency, the concept of shortened TTI is introduced to enhance the initial throughput of a TCP connection. The reason why shortened TTI can improve TCP throughput is that shortened TTI can reduce HARQ RTT and thus a shortened TCP round trip delay, causing a high cwnd growing speed and as a result a higher TCP throughput in the beginning of a TCP connection, e.g., in TCP slow start state.

Although shortened TTI has potential to reduce HARQ RTT, it has the cost of larger L1 overhead (i.e., for (s)PDCCH and new RS design) and L2 overhead (i.e., more overhead for packet segmentation and more number of  HARQ processes, as suggested in [3]). As a result, the delay reduction gain depends heavily on the considered scenarios and the assumption of L1/L2 overhead because a larger control overhead extends the latency to transmit file and thus reduces the desired delay reduction gain. Currently, according to [2] we have understood that shortened TTI brings about more delay gain when given suitable scenarios, e.g., high Uu throughput and smaller file size to be transfer, but it is still unclear which settings of L1/L2 overhead is reasonable for system-level latency reduction for shortened TTI because of unspecified sPDCCH design.

To have a correct evaluation on latency reduction gain of shortened TTI, we suggest RAN 1 to specify sPDCCH design and then accordingly investigate L1/L2 control overhead.

Proposal 1: RAN 1 decides sPDCCH design, and evaluates reasonable L1/L2 control overhead, so that latency reduction gain of shortened TTI for different scenarios can be correctly evaluated.

## Dynamic TTI Tuning

With the observation in [2] and previous system level evaluation for shortened TTI in RAN 1 #84, we can see legacy TTI and shortened TTI have their own suitable scenarios. In particularly, it seems possible to take both the advantages of both shortened TTI and legacy TTI by eNB’s configuring per-UE TTI according to the considered scenario. For example, when UE’s TCP connection is in slow start (SS) state, cwnd grows exponentially with time, so in this case using shortened TTI can boost cwnd growth and efficiently increase TCP throughput; in contrast, after the TCP congestion switches to congestion avoidance (CA) state, cwnd grows linearly with time, so using shortened TTI has limited gain in increasing cwnd, as observed in [4]. Thus, in CA state it is suitable to select long or legacy TTI to reduce overhead for higher resource utilization.

A smart TTI selection method may help further improve the latency reduction gain because it can select the best choice from legacy TTI and available TTI. At least, it has potential to avoid negative delay reduction from adopting shortened TTI in unsuitable scenarios.

Observation 1: Smart TTI selection methods might be useful to further improve latency gain, at least avoid negative latency gain from unsuitably applying shortened TTI.

To configure TTI dynamically or adaptively, we need the detailed frame structure of shortened TTI so that the modelling and evaluation of dynamic TTI selection would be more correct.

Proposal 2: RAN 1 define frame structure that facilitates shortened/legacy TTI, and specify the detailed switching mechanisms between shortened/legacy TTI.

# Evaluation Results

Our simulation results are in Annex B, which considering throughput and delay for different file size and system load. In each scenario, we consider fixed and dynamic TTI. For fixed TTI scenarios, a fixed TTI for 14 OS, 7 OS and 3/4 OS length are applied throughout the simulation; and for dynamic TTI scenario, TTI is dynamically configured per UE. See Annex A for detailed evaluation methodology.

| - CN delay =6 ms, RU =20% | (d) CN delay =0 ms, RU =20% |
| --- | --- |
| - CN delay =6 ms, RU =40% | (e) CN delay =0 ms, RU =40% |
| - CN delay =6 ms, RU =60% | (f) CN delay =0 ms, RU =60% |

Figure 1. Illustration of simulation results in Annex B

Figure 1 illustrates mean delay of different TTI under different scenarios, i.e. system load and core network delay. The number above each bar is the delay compared with the legacy TTI scheme (14 OS), i.e. delay gain = 1- delay (comparing scheme)/delay(14 OS scheme).

From Figure 1, we have several observations.

First, for a given TTI size, the delay gain from shortened TTI is decreased with the increase of  file arrival rate, (system load) and file size.

Observation 2: For a given TTI size, the delay gain from shortened TTI is decreased with the increase of  file arrival rate, (system load) and file size.

In addition, we find that the delay gain from shortened TTI is more sensitive when given a smaller backhaul delay. To be specific, when system load is not heavy (RU=20% or 40%), delay gain in CN delay 0 ms is better than the delay gain in CN delay 6 ms; however, when RU is 60%,  delay gain in CN delay 0 ms turns to be worse than the delay gain in CN delay 6 ms.

Observation 3. Given the same file size and a smaller backhaul delay, delay gain from shortened TTI is more sensitive to traffic load.

From heavy system load scenario (Figure 1(c) and 1(f)), we can see shortened TTI has worse delay gain when given smaller backhaul delay. This is because when given smaller backhaul delay, the performance of legacy TTI becomes better as well, so the room for further latency reduction is reduced.

Observation 4. A smaller backhaul delay does not always increase the delay reduction gain from shortened TTI.

Further, as we can observe that for fixed shortened TTI scheme, if traffic load is light, shortest TTI (3/4 OS) has the best performance. However, as the system load increases, the best fixed TTI scheme becomes 7 OS TTI.

Observation 5. The TTI size causing the best latency reduction gain among fixed TTI schemes increases with system load.

Finally, we can see dynamic TTI scheme has significant latency reduction gain over 7 OS and 3/4 OS scheme almost for all scenarios except for only the scenarios with very small file size and very low system load. With such a good property, we suggest RAN 1 to include the evaluation of dynamic TTI configuration to the technical report of RAN 1 LATED, i.e. TR 36.881.

Proposal 3. RAN 1 includes the evaluation of dynamic TTI configuration to the technical report of RAN 1 LATED, i.e. TR 36.881.

# Conclusions

In this contribution, we first introduced the motivation and advantages of dynamic TTI schemes, which suggest eNodeB’s adjusting per-UE TTI adaptively according to the applied scenario. We then provided system-level evaluation results to compare the latency performance of fixed TTI schemes (i.e., legacy TTI and shortened TTI) and dynamic TTI schemes. Our observations and proposals can be summarized as below.

Proposal 1: RAN 1 decides sPDCCH design, and evaluates reasonable L1/L2 control overhead, so that latency reduction gain of shortened TTI for different scenarios can be correctly evaluated.

Observation 2: For a given TTI size, the delay gain from shortened TTI is decreased with the increase of  file arrival rate, (system load) and file size.

Observation 3. Given the same file size and a smaller backhaul delay, delay gain from shortened TTI is more sensitive to traffic load.

Observation 4. A smaller backhaul delay does not always increase the delay reduction gain from shortened TTI.

Observation 5. The TTI size causing the best latency reduction gain among fixed TTI schemes increases with system load.

Proposal 1: RAN 1 decides sPDCCH design, and evaluates reasonable L1/L2 control overhead, so that latency reduction gain of shortened TTI for different scenarios can be correctly evaluated.

Proposal 2: RAN 1 evaluates the feasibility of dynamic TTI schemes so as to guarantee positive latency reduction gain for shortened-TTI enabled UE.

Proposal 3. RAN 1 includes the evaluation of dynamic TTI to the technical report of RAN 1 LATED, i.e. TR 36.881.

# Reference

- RP-150465, “Study on Latency reduction techniques for LTE,” RAN#67, Mar. 2015.

- 3GPP TR 36.881 V0.4.0, “Study on latency reduction techniques for LTE”, Oct. 2015.

- R2-156278, “L2 overhead for shortened TTI,” Nokia Networks, RAN2#92, Nov. 2015

- R1-160584, “Study on latency reduction due to TTI shortening,” RAN1#84. Feb. 2016.

- R1-160863, “System level evaluation of shortened TTI,” Intel Corporation, RAN1#84. Feb. 2016.

- R1-162946, “Control channel design for latency reduction,” MediaTek Inc., RAN1#84 bis, Apr. 2016

- R1-160583, “System-level performance evaluation of TTI shortening,” Samsung, RAN1#84. Feb. 2016.

- TR-36.889, “Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum”, RAN1 3GPP

# Annex A: Evaluation Methodology

Mechanism for Dynamic TTI selection: in this contribution to justify the potential advantage of dynamic TTI, we evaluate the dynamic TTI scheme whose DL TTI is decided by the TCP state. To be specific, we assume that there exists a method that enables the UE to instantly report the current TCP state, i.e. SS or others, of current TCP connections.  When receiving the TCP state report, TCP examines the states of TCP connections running on the UE. If there is at least one TCP connection is in CA state, eNB schedules the UE with legacy TTI; otherwise, UE always applies shortened TTI. For ease of analysis, we assume no latency in TCP state report and switch between legacy TTI and shortened TTI.

Available DL TTI and corresponding L1/L2 overhead: We consider 14, 7, and 3/4 symbol TTI as available DL TTI. 14 symbol TTI is the legacy TTI, and the others are shortened TTI. 3/4 symbol TTI is the repeated TTI pattern that 4 symbol TTI is followed by a 3 symbol TTI, i.e. on average the TTI is 3.5 symbol or 0.25 ms. As for the control overhead, we follow the assumption in [5], i.e.,

- For 14 symbol TTI, total L1/L2 overhead is 15%

- for 7 symbol TTI, total L1/L2 overhead is 17%

- For 3/4 symbol TTI, total L1/L2 overhead is 19%.

UL TTI for HARQ ACK/NACK transmission: To reduce latency for uplink TCP ACK transmission, we assume that in dynamic TTI scheme and shortened TTI scheme, shortened TTI enabled UE applies the minimal available TTI for UL transmission, i.e. 3/4 symbol TTI.

- Legacy TTI scheme : DL TTI = UL TTI = 14 symbol TTI

- Shortened TTI scheme: DL TTI = UL TTI = 3/4 symbol TTI

- Dynamic TTI scheme: DL TTI = dynamic tuning, UL TTI = 3/4 symbol TTI

Fast UL access: We assume that fast UL access method has been adopted and it takes only 4 TTI for UL access.

Scheduler: We further assume that eNodeB configure TTI for each UE, and TTI is configured through dynamic scheduling, e.g. configure TTI dynamically through (s)PDCCH as designed in [6]. For simplicity, we assume that when doing scheduling, the eNodeB first applies PF scheduler to allocate PRBs in 1 TTI to UEs.  Then eNodeB examines the UE status, decide whether to extend TTI by one of the proposed dynamic TTI schemes mentioned in section 3, and then update the quantity of allocated resources for the PF scheduler. For example, the eNodeB first allocates 20 and 30 DL PRB (fit in with the length of 0.25ms) to UE 1 and UE 2 respectively, and then decides to extend UE 1’s TTI to 1ms, and UE 2’s TTI to 0.5ms. As a result, UE 1 gets 20 PRB x 1 ms resources, and UE 2 gets 30 PRB x 0.5ms. Although the scheduling method is not optimal, we think in this stage using it to justify the concept of dynamic TTI is acceptable.

TCP and FTP setting:	As discussed in [2], we focus on TCP applications in this evaluation. The evaluation model for core network (CN) delay, HARQ RTT, SR/grant and TCP ACK delay follows that in [7]. We assume that it takes 3 milliseconds for the UE higher layer to prepare TCP acknowledgement after successfully receiving a TCP packet, i.e. ACK generation time. In order to simplify the TCP model, we assume that TCP ACK is always correctly transmitted to the App server without error.  TCP and FTP traffic model is shown in Table 1.

Table 1: TCP and FTP model

| Parameter | Assumption |
| --- | --- |
| Initial window size | 1460 Bytes |
| MSS | 1460 Bytes |
| TCP/IP overhead | 40 Bytes |
| ssthresh | 65535 Bytes |
| FTP traffic model | Model 3 (described in [8]) |
| File size / arrival rate λ | 12.5 KB (100kbits), 100 KB, 500 KB, 1MB Arrival rate λ is determined by RU (20, 40, 60%) |

The other evaluation parameters are provided in Annex. A.

Performance metrics: 	We define two performance metrics, i.e., throughput gain and delay gain, as below.

- Delay gain : given the same file size to be delivered, the ratio of reduced file transfer latency to the file transfer latency of baseline scheme, i.e.,

    - Delay gain = 1 – delay(proposed scheme)/ delay(baseline scheme)

- Throughput gain: given the same file transmission time, the percentage of increased number of transmitted bits over the transmitted bits of the baseline scheme, i.e.,

    - throughput gain = #bits(proposed scheme)/ #bits (baseline scheme) – 1

Notice that there is a positive correlation between delay gain and throughput gain, e.g. assuming that a proposed scheme needs transmission t ms to transmit M-byte file, then if the proposed scheme has a lager delay gain in transmitting a M-byte file, it then has a larger throughput gain when given t-ms file transmission time.

# Annex B: Evaluation Results

B.1 Evaluation results with DL traffic and non-ideal core network delay

Table B.1-1: Results with 100kbit file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 2.991 | 3.910 | 4.578 | 4.5566 | 2.707 | 3.408 | 3.932 | 3.6717 | 0.143 | 0.142 | 0.175 | 0.2048 |
|  | 50% | 3.034 | 4.015 | 4.755 | 4.6954 | 2.966 | 3.827 | 4.430 | 4.4086 | 2.721 | 3.405 | 3.897 | 3.9496 |
|  | 95% | 3.074 | 4.123 | 4.911 | 4.8244 | 3.084 | 4.104 | 4.897 | 4.8244 | 3.071 | 4.054 | 4.820 | 4.7840 |
|  | Mean | 3.035 | 4.018 | 4.748 | 4.6925 | 2.943 | 3.811 | 4.449 | 4.3894 | 2.133 | 2.682 | 3.032 | 3.2210 |
| DL: Delay CDF [s] | 5% | 0.036 | 0.027 | 0.022 | 0.0277 | 0.036 | 0.027 | 0.022 | 0.0226 | 0.036 | 0.027 | 0.023 | 0.0228 |
|  | 50% | 0.037 | 0.028 | 0.023 | 0.0235 | 0.038 | 0.029 | 0.025 | 0.0251 | 0.043 | 0.034 | 0.029 | 0.0289 |
|  | 95% | 0.038 | 0.029 | 0.024 | 0.0244 | 0.042 | 0.033 | 0.028 | 0.0310 | 1.252 | 1.274 | 1.412 | 0.9225 |
|  | Mean | 0.037 | 0.028 | 0.023 | 0.0236 | 0.038 | 0.029 | 0.025 | 0.0256 | 0.234 | 0.239 | 0.256 | 0.1681 |
| RU |  | 20.625 | 20.588 | 20.641 | 26.2859 | 45.431 | 45.836 | 46.0756 | 53.1883 | 66.149 | 66.625 | 67.315 | 71.4451 |
| 𝜆 |  | 40 |  |  |  | 80 |  |  |  | 120 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.1-2: Results with 100kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 12.004 | 15.697 | 17.780 | 17.9405 | 9.543 | 11.298 | 12.037 | 12.9405 | 0.905 | 1.068 | 1.231 | 1.4807 |
|  | 50% | 12.812 | 17.220 | 19.441 | 19.7725 | 12.407 | 15.306 | 17.103 | 17.6025 | 9.098 | 10.957 | 11.536 | 12.8216 |
|  | 95% | 13.302 | 18.385 | 20.620 | 21.0677 | 13.732 | 18.967 | 21.064 | 21.8362 | 13.715 | 18.318 | 20.289 | 21.2091 |
|  | Mean | 12.779 | 17.061 | 19.419 | 19.7579 | 12.112 | 15.304 | 16.891 | 17.6178 | 8.123 | 9.787 | 10.506 | 12.2418 |
| DL: Delay CDF [s] | 5% | 0.067 | 0.050 | 0.043 | 0.0429 | 0.065 | 0.047 | 0.042 | 0.0406 | 0.064 | 0.047 | 0.043 | 0.0406 |
|  | 50% | 0.072 | 0.054 | 0.046 | 0.0458 | 0.075 | 0.061 | 0.054 | 0.0527 | 0.122 | 0.106 | 0.110 | 0.0844 |
|  | 95% | 0.079 | 0.060 | 0.051 | 0.0503 | 0.105 | 0.091 | 0.084 | 0.0754 | 1.537 | 1.297 | 1.294 | 0.9594 |
|  | Mean | 0.073 | 0.054 | 0.047 | 0.0463 | 0.079 | 0.064 | 0.059 | 0.0549 | 0.333 | 0.329 | 0.336 | 0.2073 |
| RU |  | 19.702 | 19.603 | 19.824 | 19.9558 | 42.987 | 43.410 | 43.998 | 42.8668 | 65.017 | 65.609 | 66.183 | 62.6252 |
| 𝜆 |  | 5 |  |  |  | 10 |  |  |  | 15 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.1-3: Results with 500kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 14.946 | 19.763 | 23.154 | 23.5793 | 13.156 | 15.885 | 16.749 | 18.8469 | 3.338 | 3.577 | 3.984 | 5.2630 |
|  | 50% | 19.141 | 24.506 | 28.856 | 29.3113 | 18.869 | 22.556 | 25.094 | 26.7644 | 12.830 | 14.579 | 14.992 | 17.5411 |
|  | 95% | 21.942 | 27.875 | 32.711 | 33.4328 | 22.464 | 28.234 | 32.791 | 33.6621 | 21.615 | 26.905 | 30.059 | 31.2522 |
|  | Mean | 18.974 | 24.485 | 28.593 | 29.0366 | 18.429 | 22.529 | 25.269 | 26.5332 | 13.140 | 15.479 | 16.741 | 18.5656 |
| DL: Delay CDF [s] | 5% | 0.199 | 0.154 | 0.131 | 0.1299 | 0.196 | 0.154 | 0.133 | 0.1287 | 0.207 | 0.157 | 0.134 | 0.1298 |
|  | 50% | 0.231 | 0.179 | 0.153 | 0.1511 | 0.242 | 0.199 | 0.182 | 0.1775 | 0.423 | 0.371 | 0.355 | 0.3203 |
|  | 95% | 0.285 | 0.218 | 0.185 | 0.1816 | 0.394 | 0.333 | 0.347 | 0.2845 | 1.573 | 1.578 | 1.499 | 1.1062 |
|  | Mean | 0.235 | 0.181 | 0.155 | 0.1530 | 0.260 | 0.218 | 0.204 | 0.1877 | 0.599 | 0.569 | 0.563 | 0.4358 |
| RU |  | 19.901 | 20.004 | 20.336 | 21.3362 | 41.500 | 41.596 | 42.071 | 42.0447 | 62.487 | 62.860 | 63.484 | 62.6252 |
| 𝜆 |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.1-4: Results with 1MB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 13.824 | 18.191 | 20.425 | 21.2024 | 9.341 | 10.827 | 11.188 | 13.1793 | 3.357 | 3.680 | 3.890 | 7.1495 |
|  | 50% | 20.387 | 26.671 | 30.857 | 31.3104 | 20.531 | 24.567 | 27.425 | 28.6351 | 16.160 | 17.640 | 18.193 | 21.4345 |
|  | 95% | 25.062 | 32.626 | 37.538 | 37.8323 | 25.562 | 32.873 | 38.263 | 38.9219 | 26.209 | 32.953 | 37.276 | 38.6371 |
|  | Mean | 20.497 | 26.388 | 30.467 | 30.7521 | 19.779 | 24.266 | 27.060 | 28.3242 | 15.546 | 18.152 | 19.447 | 21.8021 |
| DL: Delay CDF [s] | 5% | 0.340 | 0.257 | 0.223 | 0.2214 | 0.347 | 0.274 | 0.222 | 13.1793 | 0.336 | 0.265 | 0.227 | 7.1495 |
|  | 50% | 0.420 | 0.321 | 0.278 | 0.2728 | 0.434 | 0.366 | 0.313 | 28.6351 | 0.622 | 0.549 | 0.542 | 21.4345 |
|  | 95% | 0.577 | 0.440 | 0.372 | 0.3775 | 0.972 | 0.898 | 0.733 | 38.9219 | 2.602 | 2.356 | 2.464 | 38.6371 |
|  | Mean | 0.431 | 0.333 | 0.290 | 0.2879 | 0.484 | 0.409 | 0.352 | 28.3242 | 0.850 | 0.785 | 0.791 | 21.8021 |
| RU |  | 20.687 | 20.761 | 21.018 | 22.6705 | 42.209 | 42.222 | 42.709 | 43.2472 | 61.198 | 61.639 | 62.513 | 61.5606 |
| 𝜆 |  | 0.5 |  |  |  | 1 |  |  |  | 1.5 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

B.2 Evaluation results with DL traffic and 0ms core network delay

Table B.2-1: Results with 100kbit file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 4.6917 | 7.3425 | 10.1940 | 10.0015 | 4.0583 | 5.8846 | 7.4901 | 6.9306 | 0.1475 | 0.1652 | 0.2076 | 0.2472 |
|  | 50% | 4.8027 | 7.7573 | 10.9814 | 10.6579 | 4.6009 | 7.0707 | 9.5314 | 9.4312 | 4.0702 | 5.9189 | 7.4772 | 7.6998 |
|  | 95% | 4.8772 | 8.1235 | 11.7935 | 11.2756 | 4.8952 | 7.9934 | 11.5251 | 11.1660 | 4.8560 | 7.8225 | 11.2063 | 11.0127 |
|  | Mean | 4.7953 | 7.7386 | 10.9836 | 10.6726 | 4.5667 | 7.0567 | 9.6131 | 9.4170 | 3.2073 | 4.7547 | 6.1723 | 6.5688 |
| DL: Delay CDF [s] | 5% | 0.0222 | 0.0134 | 0.0092 | 0.0096 | 0.0222 | 0.0135 | 0.0093 | 0.0096 | 0.0223 | 0.0137 | 0.0095 | 0.0097 |
|  | 50% | 0.0229 | 0.0141 | 0.0100 | 0.0103 | 0.0243 | 0.0158 | 0.0118 | 0.0120 | 0.0290 | 0.0204 | 0.0161 | 0.0161 |
|  | 95% | 0.0237 | 0.0151 | 0.0108 | 0.0110 | 0.0287 | 0.0196 | 0.0158 | 0.0172 | 1.2285 | 1.2766 | 1.3693 | 0.8319 |
|  | Mean | 0.0229 | 0.0142 | 0.0100 | 0.0103 | 0.0247 | 0.0161 | 0.0120 | 0.0125 | 0.2191 | 0.2204 | 0.2339 | 0.1497 |
| RU |  | 20.4970 | 20.7350 | 20.6586 | 26.3340 | 45.4760 | 45.8822 | 46.1431 | 53.2560 | 66.1716 | 66.6744 | 67.2940 | 71.5730 |
| 𝜆 |  | 40 |  |  |  | 40 |  |  |  | 80 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.2-2: Results with 100kB file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 17.6469 | 25.0747 | 29.3540 | 30.3349 | 12.4772 | 15.0259 | 16.1946 | 18.2821 | 1.0630 | 1.2719 | 1.5864 | 1.8496 |
|  | 50% | 18.7934 | 28.6268 | 33.6199 | 35.5846 | 16.7899 | 22.6137 | 26.0684 | 27.5932 | 12.0753 | 14.1659 | 15.1013 | 17.6739 |
|  | 95% | 19.6400 | 31.7421 | 37.5620 | 41.2882 | 19.6025 | 30.9845 | 35.9645 | 39.2496 | 19.3781 | 28.9365 | 33.2371 | 36.5238 |
|  | Mean | 18.7281 | 28.4343 | 33.5681 | 35.7476 | 16.6618 | 23.1218 | 26.3110 | 28.3273 | 10.8497 | 14.1127 | 15.6010 | 18.5023 |
| DL: Delay CDF [s] | 5% | 0.0447 | 0.0279 | 0.0228 | 0.0209 | 0.0438 | 0.0280 | 0.0240 | 0.0222 | 0.0435 | 0.0301 | 0.0259 | 0.0237 |
|  | 50% | 0.0476 | 0.0316 | 0.0264 | 0.0250 | 0.0549 | 0.0407 | 0.0358 | 0.0345 | 0.0922 | 0.0823 | 0.0885 | 0.0651 |
|  | 95% | 0.0520 | 0.0364 | 0.0310 | 0.0300 | 0.0839 | 0.0685 | 0.0646 | 0.0571 | 1.3742 | 1.4880 | 1.3034 | 0.9382 |
|  | Mean | 0.0480 | 0.0319 | 0.0266 | 0.0254 | 0.0581 | 0.0442 | 0.0405 | 0.0365 | 0.2953 | 0.3004 | 0.3052 | 0.1773 |
| RU |  | 19.6555 | 19.5994 | 19.8584 | 19.4333 | 42.9524 | 43.2944 | 43.9873 | 42.3778 | 65.1751 | 65.5080 | 66.1869 | 63.9922 |
| 𝜆 |  | 5 |  |  |  | 10 |  |  |  | 15 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.2-3: Results with 500kB file transfer in DL, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 23.7288 | 30.7905 | 32.7829 | 34.6370 | 18.0848 | 19.7121 | 19.5472 | 23.1678 | 3.8573 | 4.4975 | 4.6217 | 6.7240 |
|  | 50% | 28.3768 | 42.2248 | 47.3298 | 50.3404 | 25.2785 | 32.5592 | 34.5528 | 37.4676 | 16.4967 | 18.9736 | 19.0183 | 22.8119 |
|  | 95% | 31.7293 | 46.4316 | 56.7143 | 58.8879 | 31.2697 | 45.9729 | 52.2405 | 55.2801 | 30.5819 | 39.6148 | 43.2715 | 46.7666 |
|  | Mean | 28.1258 | 40.1698 | 47.0089 | 49.5241 | 25.3755 | 32.2747 | 35.1944 | 38.1057 | 17.1468 | 20.7985 | 21.6559 | 24.7660 |
| DL: Delay CDF [s] | 5% | 0.1333 | 0.0874 | 0.0723 | 0.0696 | 0.1348 | 0.0914 | 0.0799 | 0.0749 | 0.1401 | 0.1031 | 0.0973 | 0.0894 |
|  | 50% | 0.1542 | 0.1061 | 0.0918 | 0.0871 | 0.1787 | 0.1466 | 0.1392 | 0.1266 | 0.3340 | 0.3208 | 0.3524 | 0.2699 |
|  | 95% | 0.1849 | 0.1367 | 0.1327 | 0.1245 | 0.3166 | 0.2950 | 0.3255 | 0.2561 | 1.4486 | 1.5412 | 1.4870 | 1.0083 |
|  | Mean | 0.1562 | 0.1110 | 0.0959 | 0.0906 | 0.1966 | 0.1657 | 0.1594 | 0.1397 | 0.5255 | 0.5131 | 0.5240 | 0.3750 |
| RU |  | 19.8461 | 19.5465 | 19.7689 | 18.8785 | 40.9476 | 41.1818 | 41.6793 | 39.4122 | 62.3156 | 62.5919 | 63.3082 | 60.8088 |
| 𝜆 |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |

Table B.2-4: Results with 1MB file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS | 14 OS | 7OS | 3/4 OS | Dynamic OS |
| DL: UPT CDF [Mbps] | 5% | 20.6713 | 29.5131 | 33.2234 | 35.7304 | 12.0626 | 14.0944 | 14.7406 | 17.6446 | 4.5606 | 5.8520 | 5.2458 | 8.0355 |
|  | 50% | 30.9070 | 42.3016 | 48.5211 | 50.5125 | 27.8022 | 35.1595 | 37.5010 | 39.9854 | 19.4586 | 20.4785 | 21.1384 | 27.8111 |
|  | 95% | 36.4907 | 52.3080 | 61.1163 | 63.9371 | 36.9131 | 52.8467 | 57.3839 | 61.8598 | 36.0908 | 47.6759 | 49.0287 | 51.9954 |
|  | Mean | 30.0849 | 42.4208 | 48.2050 | 51.1970 | 27.0395 | 34.7495 | 36.9441 | 39.8749 | 20.0266 | 23.8729 | 24.2925 | 28.0381 |
| DL: Delay CDF [s] | 5% | 0.2306 | 0.1596 | 0.1321 | 0.1257 | 0.2373 | 0.1596 | 0.1417 | 0.1334 | 0.2338 | 0.1689 | 0.1571 | 0.1492 |
|  | 50% | 0.2768 | 0.1997 | 0.1744 | 0.1654 | 0.3289 | 0.2657 | 0.2669 | 0.2489 | 0.5111 | 0.4897 | 0.5150 | 0.4118 |
|  | 95% | 0.3690 | 0.2995 | 0.2612 | 0.2379 | 0.8191 | 0.6874 | 0.6937 | 0.6237 | 1.9169 | 1.7183 | 1.8455 | 1.3328 |
|  | Mean | 0.2900 | 0.2097 | 0.1845 | 0.1726 | 0.3740 | 0.3131 | 0.3091 | 0.2730 | 0.7076 | 0.6714 | 0.7056 | 0.5417 |
| RU |  | 20.3944 | 20.3395 | 20.5135 | 19.5532 | 41.7564 | 41.4526 | 42.3368 | 40.1603 | 60.9434 | 61.3426 | 62.3995 | 59.6621 |
| 𝜆 |  | 0.5 |  |  |  | 1.0 |  |  |  | 1.5 |  |  |  |
| Notes: |  | MTK, R1-16XXXX, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |