TSG-RAN WG1 #84bis	R1-163909

Busan, South Korea, April 11 – 15, 2016

Source:	Ericsson, Vodafone, Samsung, Nokia, …

Title:	WF on the Rural macro scenario

Agenda Item:	8.2.1

Document for:	Discussion and Decision

# Introduction

In the TR on Study on Scenarios and Requirements for Next Generation Access Technologies [6], a rural macrocell scenario is defined for deployment of 5G. There is a need for evaluating performance in relation to the requirements.

A Rural macrocell channel model is specified as part of the IMT-Advanced channel model [1]. However, this model is 2D in the sense that elevation angles are not modeled. An extension to 3D modeling was done in the 3D SCM model [2] though only for the Urban macrocell and microcell scenarios. In this contribution we investigate the possibility to make a 3D RMa scenario and propose suitable model parameters.

# Gap analysis of the IMT-Adv RMa model

The IMT-Adv RMa model provides path loss and LOS probability models as well as channel model parameters including angular and cluster level parameters. However in comparison with the 3D SCM model it lacks elevation angle parameters. A gap analysis can be done by transferring the parameters from the parameter tables of [1] to the parameter tables of 36.873 [2]. This is done below:

Table 7.2-1: Pathloss models UMa path loss models can be completely specified using Table A1-2 in [1]

Table 7.2-2: LOS probabilities A UMa LOS probability model can be reused from Table A1-3 in [1]

Table 7.3-6: Channel model parameters This table can only be partially filled with parameters from [1] as seen below

Table 7.3-9: ZSD and ZoD offset parameters for 3D-RMa There are no parameters in [1] that could be used to construct such a table. For reference, the corresponding table 7.3-7 for 3D-UMa is shown below

In other words, there is a need to develop parameters for all missing entries in these tables (highlighted in yellow).

Table 7.3-6: Channel model parameters

| Scenarios |  | 3D-UMi |  |  | 3D-UMa |  |  | 3D-RMa |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O-to-I | LOS | NLOS | O-to-I | LOS | NLOS |
| Delay spread (DS) log10([s]) | DS | -7.19 | -6.89 | -6.62 | -7.03 | -6.44 | -6.62 | -7.49 | -7.43 |
|  | DS | 0.40 | 0.54 | 0.32 | 0.66 | 0.39 | 0.32 | 0.55 | 0.48 |
| AoD spread (σASD) log10([]) | ASD | 1.20 | 1.41 | 1.25 | 1.15 | 1.41 | 1.25 | 0.90 | 0.95 |
|  | ASD | 0.43 | 0.17 | 0.42 | 0.28 | 0.28 | 0.42 | 0.38 | 0.45 |
| AoA spread (σASA) log10([]) | ASA | 1.75 | 1.84 | 1.76 | 1.81 | 1.87 | 1.76 | 1.52 | 1.52 |
|  | ASA | 0.19 | 0.15 | 0.16 | 0.20 | 0.11 | 0.16 | 0.24 | 0.13 |
| ZoA spread (σZSA) log10([])2) | ZSA | 0.60 | 0.88 | 1.01 | 0.95 | 1.26 | 1.01 |  |  |
|  | ZSA | 0.16 | 0.16 | 0.43 | 0.16 | 0.16 | 0.43 |  |  |
| Shadow fading (SF) [dB] | SF | 3 | 4 | 7 | 4 | 6 | 7 | 4 | 8 |
| K-factor (K) [dB] | K | 9 | N/A | N/A | 9 | N/A | N/A | 7 | N/A |
|  | K | 5 | N/A | N/A | 3.5 | N/A | N/A | 4 | N/A |
| Cross-Correlations | ASD vs DS | 0.5 | 0 | 0.4 | 0.4 | 0.4 | 0.4 | 0 | -0.4 |
|  | ASA vs DS | 0.8 | 0.4 | 0.4 | 0.8 | 0.6 | 0.4 | 0 | 0 |
|  | ASA vs SF | -0.4 | -0.4 | 0 | -0.5 | 0 | 0 | 0 | 0 |
|  | ASD vs SF | -0.5 | 0 | 0.2 | -0.5 | -0.6 | 0.2 | 0 | 0.6 |
|  | DS vs SF | -0.4 | -0.7 | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
|  | ASD vs ASA | 0.4 | 0 | 0 | 0 | 0.4 | 0 | 0 | 0 |
|  | ASD vs  | -0.2 | N/A | N/A | 0 | N/A | N/A | 0 | N/A |
|  | ASA vs  | -0.3 | N/A | N/A | -0.2 | N/A | N/A | 0 | N/A |
|  | DS vs  | -0.7 | N/A | N/A | -0.4 | N/A | N/A | 0 | N/A |
|  | SF vs  | 0.5 | N/A | N/A | 0 | N/A | N/A | 0 | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
|  | ZSA vs SF | 0 | 0 | 0 | -0.8 | -0.4 | 0 |  |  |
|  | ZSD vs K | 0 | N/A | N/A | 0 | N/A | N/A |  |  |
|  | ZSA vs K | 0 | N/A | N/A | 0 | N/A | N/A |  |  |
|  | ZSD vs DS | 0 | -0.5 | -0.6 | -0.2 | -0.5 | -0.6 |  |  |
|  | ZSA vs DS | 0.2 | 0 | -0.2 | 0 | 0 | -0.2 |  |  |
|  | ZSD vs ASD | 0.5 | 0.5 | -0.2 | 0.5 | 0.5 | -0.2 |  |  |
|  | ZSA vs ASD | 0.3 | 0.5 | 0 | 0 | -0.1 | 0 |  |  |
|  | ZSD vs ASA | 0 | 0 | 0 | -0.3 | 0 | 0 |  |  |
|  | ZSA vs ASA | 0 | 0.2 | 0.5 | 0.4 | 0 | 0.5 |  |  |
|  | ZSD vs ZSA | 0 | 0 | 0.5 | 0 | 0 | 0.5 |  |  |
| Delay distribution |  | Exp | Exp | Exp | Exp | Exp | Exp | Exp | Exp |
| AoD and AoA distribution |  | Wrapped Gaussian |  |  | Wrapped Gaussian |  |  | Wrapped Gaussian |  |
| ZoD and ZoA distribution |  | Laplacian |  |  | Laplacian |  |  |  |  |
| Delay scaling parameter r |  | 3.2 | 3 | 2.2 | 2.5 | 2.3 | 2.2 | 3.8 | 1.7 |
| XPR [dB] 6) |  | 9 | 8.0 | 9 | 8 | 7 | 9 | 12 | 7 |
|  |  | 3 | 3 | 5 | 4 | 3 | 5 |  |  |
| Number of clusters |  | 12 | 19 | 12 | 12 | 20 | 12 | 11 | 10 |
| Number of rays per cluster |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Cluster ASD |  | 3 | 10 | 5 | 5 | 2 | 5 | 2 | 2 |
| Cluster ASA |  | 17 | 22 | 8 | 11 | 15 | 8 | 3 | 3 |
| Cluser ZSA2) |  | 7 | 7 | 3 | 7 | 7 | 3 |  |  |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 4 | 3 | 3 | 4 | 3 | 3 |
| Correlation distance in the horizontal plane [m]3) | DS | 7 | 10 | 10 | 30 | 40 | 10 | 50 | 36 |
|  | ASD | 8 | 10 | 11 | 18 | 50 | 11 | 25 | 30 |
|  | ASA | 8 | 9 | 17 | 15 | 50 | 17 | 35 | 40 |
|  | SF | 10 | 13 | 7 | 37 | 50 | 7 | 37 | 120 |
|  |  | 15 | N/A | N/A | 12 | N/A | N/A | 40 | N/A |
|  | ZSA | 12 | 10 | 25 | 15 | 50 | 25 |  |  |
|  | ZSD | 12 | 10 | 25 | 15 | 50 | 25 |  |  |

Table 7.3-7: ZSD and ZoD offset parameters for 3D-UMa

| Scenarios |  | 3D-UMa |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O-to-I |  |
|  |  |  |  | LOS O-to-I | NLOS O-to I |
| ZoD spread1) (σZSD) log10([]) | µZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] | max[-0.5, -2.1(d2D/1000)-0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |
|  | ZSD | 0.40 | 0.49 | 0.40 | 0.49 |
| ZoD offset | µoffset,ZOD | 0 | -10^{-0.62log10(max(10, d2D))+1.93-0.07(hUT-1.5)} | 0 | -10^{-0.62log10(max(10, d2D))+1.93-0.07(hUT-1.5)} |
| NOTE: The proposed average ESD is smaller than that of Winner+ |  |  |  |  |  |

# 3D extension of the IMT-Adv RMa model

The ZoA spread will be dependent on the surroundings of the UE which in the rural scenario is expected to consist of semi-open areas, trees, and a few lower buildings. Thus an educated guess would be to use ZoA spread parameters which are significantly lower than in the 3D-UMa scenario, perhaps more like those in the 3D-UMi scenario where the median ZoA spread is 4 and 7.6 degrees in LOS and NLOS respectively. It is therefore proposed to reuse the 3D-UMi ZoA spread for the 3D-RMa scenario.

The cross-correlations of ZSD and ZSA vs other channel parameters are proposed to be reused from the 3D UMa scenario without any particular motivation. While this is of course a pure guess it should be noted that these parameters will have a quite minor impact on simulation results and are therefore less important to set correctly.

The ZoD and ZoA distributions are Laplacian in both 3D-UMa and 3D-UMi, so it is proposed to use the same distribution also in 3D-RMa. It is known that different angular distributions give very similar simulation results if the angular spread is smaller than the beamwidth of the antenna, so it appear that this choice will also be of less importance.

The standard deviation of the XPR is not specified in the IMT-Adv model. In [3], XPR measurements are done over distances up to 4.5 km in different environments including rural areas. It is found that the standard deviation of the XPR is 3 dB which would motivate the use of this value for the 3D-RMa model.

For the cluster ZSA, the value can be expected to be a certain fraction of the ZoA spread. A value of 3 degrees seems reasonable from this consideration.

The correlation distances of the ZSA and ZSD are 15 and 50 m in LOS and NLOS in the 3D-UMa scenario. It is proposed to reuse these values for the 3D-RMa scenario. Since the ZoA and ZoD spreads will be low the correlation distances will have a very minor impact on any simulation results.

The ZoD spread in the 3D-UMa scenario is plotted in Figure 1. As can be seen, the ZoD spread reduces to a fraction of a degree for distances larger than about 500 m. Few direct measurements of the ZoD spread in rural cells are available in the literature, however in [4] the ZoD spread is estimated from vertical diversity measurements in macrocells with BS-UE separations of up to 7400 m. Table VII in [4] summarizes these estimates and is copied below for convenience. The estimated elevation spreads in these measurements are in the range 0.2-0.8 degrees which is very similar to the 3D-UMa values. It is therefore proposed to have a constant ZoD spread of 0.3 degrees in the 3D-RMa scenario.

The ZoD offset angle is a function of the geometry as illustrated in Figure 2. It can therefore be expected that the ZoD offset angle of the 3D-UMa scenario will not be representative of the rural scenario. Instead, an estimate of the ZoD offset angle can be determined by assuming a certain clutter height in relation to the eNB height (35 m) and the UE height (1.5 m). 5 m seems like a reasonable clutter height that would be representative of single buildings, lower trees etc. The ZoD offset angle will then be the difference between the zenith angle towards the UE assuming no obstructions and the zenith angle to the top of the clutter, i.e.

μoffset,ZoD=arctan35-5d2D-arctan35-1.5d2D

This model is plotted in Figure 3.

Figure 1 ZoD spread in the 3D-UMa scenario

Figure 2 Illustration of the ZoD offset angle in an urban scenario (left) and a rural scenario (right)

Figure 3 ZoD offset angle for 3D-UMa and the proposed model for 3D-RMa

All these proposed parameters are summarized in the tables in the Appendix. The IMT-Adv channel model is specified for the frequency range 2-6 GHz and the models and parameters can probably not be extrapolated far outside this frequency range. However, it is known from [5] that most channel parameters exhibit weak or no frequency dependence so it can be assumed that the model could be extended to 7 GHz without major errors occurring.

# Conclusion

The following is proposed:

Proposal: For the Rural macrocell (RMa) scenario as described in TR 38.913 [6], agree on the RMa channel model parameters in Appendix A as a working assumption

- This model is valid up to 7 GHz

- Take a majority view on which TR to capture this channel model

  - Alt 1 TR38.900 --- in this case, the RMa scenario description is also captured in the TR.

  - Alt 2 TR38.913

# References

- ITU-R M.2135 “Guidelines for evaluation of radio interface technologies for IMT-Advanced” (12/2009).

- 3GPP TR 36.873 “Study on 3D channel model for LTE”, v12.0.0, June 2015.

- H. Asplund, J-E. Berg, F. Harrysson, J. Medbo, and M. Riback, “Propagation characteristics of polarized radio waves in cellular communications”, in Proc of IEEE VTC 2007 fall, Boston, 2007.

- H. Asplund, A. A. Glazunov, A. F. Molisch, K. I. Pedersen, and M. Steinbauer, “The COST 259 directional channel model - Part II: Macrocells”, IEEE Transactions on Wireless Communications, pp. 3434-3450, Vol. 5, Issue 12, 2006.

- Aalto University, AT&T, BUPT, CMCC, Ericsson, Huawei, Intel, KT Corporation, Nokia, NTT DOCOMO, New York University, Qualcomm, Samsung, University of Bristol, University of Southern California, “5G Channel Model for bands up to 100 GHz (v2.0)”, Mar. 2016 (http://www.5gworkshops.com/5GCM.html).

- 3GPP TR 36.913, “Study on Scenarios and Requirements for Next Generation Access Technologies;” v0.3.0, March 2016.

# Appendix A

Table 7.2-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and distance is in meters | Shadow fading std [dB]7) | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| 3D-RMa LOS |  |  |  |
| 3D-RMa NLOS |  |  |  |

Table 7.2-2: LOS probabilities

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| 3D-RMa |  |

Table 7.3-6: Channel model parameters

| Scenarios |  | 3D-RMa |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| Delay spread (DS) log10([s]) | DS | -7.49 | -7.43 |
|  | DS | 0.55 | 0.48 |
| AoD spread (σASD) log10([]) | ASD | 0.90 | 0.95 |
|  | ASD | 0.38 | 0.45 |
| AoA spread (σASA) log10([]) | ASA | 1.52 | 1.52 |
|  | ASA | 0.24 | 0.13 |
| ZoA spread (σZSA) log10([])2) | ZSA | 0.60 | 0.88 |
|  | ZSA | 0.16 | 0.16 |
| Shadow fading (SF) [dB] | SF | 4 | 8 |
| K-factor (K) [dB] | K | 7 | N/A |
|  | K | 4 | N/A |
| Cross-Correlations | ASD vs DS | 0 | -0.4 |
|  | ASA vs DS | 0 | 0 |
|  | ASA vs SF | 0 | 0 |
|  | ASD vs SF | 0 | 0.6 |
|  | DS vs SF | -0.5 | -0.5 |
|  | ASD vs ASA | 0 | 0 |
|  | ASD vs  | 0 | N/A |
|  | ASA vs  | 0 | N/A |
|  | DS vs  | 0 | N/A |
|  | SF vs  | 0 | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.4 |
|  | ZSD vs K | 0 | N/A |
|  | ZSA vs K | 0 | N/A |
|  | ZSD vs DS | -0.2 | -0.5 |
|  | ZSA vs DS | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | -0.1 |
|  | ZSD vs ASA | -0.3 | 0 |
|  | ZSA vs ASA | 0.4 | 0 |
|  | ZSD vs ZSA | 0 | 0 |
| Delay distribution |  | Exp | Exp |
| AoD and AoA distribution |  | Wrapped Gaussian |  |
| ZoD and ZoA distribution |  | Laplacian |  |
| Delay scaling parameter r |  | 3.8 | 1.7 |
| XPR [dB] 6) |  | 12 | 7 |
|  |  | 4 | 3 |
| Number of clusters |  | 11 | 10 |
| Number of rays per cluster |  | 20 | 20 |
| Cluster ASD |  | 2 | 2 |
| Cluster ASA |  | 3 | 3 |
| Cluser ZSA2) |  | 3 | 3 |
| Per cluster shadowing std  [dB] |  | 3 | 3 |
| Correlation distance in the horizontal plane [m]3) | DS | 50 | 36 |
|  | ASD | 25 | 30 |
|  | ASA | 35 | 40 |
|  | SF | 37 | 120 |
|  |  | 40 | N/A |
|  | ZSA | 15 | 50 |
|  | ZSD | 15 | 50 |

Table 7.3-9: ZSD and ZoD offset parameters for 3D-RMa

| Scenarios |  | 3D-UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS |  |
| ZoD spread1) (σZSD) log10([]) | µZSD | 0.3 | 0.3 |  |
|  | ZSD | 0.40 | 0.49 |  |
| ZoD offset | µoffset,ZOD | 0 | arctan((35-5)/ d2D )- arctan((35-1.5)/ d2D ) |  |