3GPP TSG-RAN WG1 Meeting #86	R1-167571

Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:	8.1.5

Source:	InterDigital Communications

Title:	On UL Channel Reciprocity for NR

Document for:	Discussion

# Introduction

In RAN #85, many relevant aspects to maasive MIMO operation in NR were discussed [1]. An interesting topic related to massive MIMO operation is potential use of channel reciprocity for downlink and/or uplink beamforming.

The RAN #85 agreements recognized reciprocity-based feedback among the potential CSI reporting schemes which can be combined with other reporting mechanisms [2]. It was also stated that RS design should consider support for measurements required for reciprocity-based precoding [3]. In addition, further studies for uplink MIMO precoding based on full or partial reciprocity was included [4].

In this contribution, we discuss a few scenarios where partial reciprocity could be used to enhanced uplink MIMO operation.

# Reciprocity

To implement an efficient beamforming, the transmitter requires an accurate knowledge of the channel state information (CSI). Such information could be provided explicitly by the receiving-end based on the measurements performed on the reference signals. However, it imposes some delay and extra overhead on the control channel. In NR, the overhead problem is further exacerbated due to very large number of antennas that motivates development of resource-efficient CSI acquisition schemes.

An alternative scheme to CSI reporting is based on the principle of channel reciprocity. The principal idea behind this approach is derived from the Lorentz reciprocity theorem [5]. In brief, given a radiating source at point A and a distant point of measurement B, the relationship between the radiating source and the resulting electric field is maintained if the source is moved to point B and the measurement is performed at point A. An example of such scenario is TDD-based systems where the downlink channel estimation can be used for uplink beamforming as long as the time between the measurement and the uplink transmission is shorter than the channel’s coherence time. Moreover, the validity of channel reciprocity depends also on the interference observed at the transmitter and at the receiver. As such, estimating the reciprocal channel should take this interference asymmetry into account.

Proposal 1: RAN1 considers study on the effect of interference asymmetry on the use of channel reciprocity for uplink MIMO.

# Potential Use Cases for Uplink MIMO for TDD

Channel reciprocity may be used in conjunction with reporting-based CSI acquisition to enhance the performance and reduce the CSI feedback overhead. In this section, we discuss two scenarios where combined use of reciprocity- and report-based CSI could result in significant improvement in system performance.

## Dual Precoder Design for UL MIMO

In LTE Rel. 10, dual precoder design was introduced to enable 8TX antenna transmission in TM9 without requiring an excessively large codebook. A dual precoder structure helps accuracy of precoding by allowing to split the CSI feedback into long and short term components. For uplink NR, given the much smaller size of the antennas at mmWave, mobile units may become equipped with a large number of antennas. Therefore, a dual precoder structure may become essential to support implementation of uplink beamforming.

Assuming a dual precoder structure, the overall precoder W=W1W2 can be designed such that W1 and W2 are determined based on reciprocity- and report-based CSI, repectively. In other words, W2 would be an eNB-directed precoder (report-based) while W1 would be a UE-directed precoder (reciprocity-based) without requiring any feedback. In a simple example, the precoder W1 can be based on long term statistical direction of the channel derived using the reciprocal channel, while W2 provides short term updates to enhance channel tracking. Figure 1 shows basic operation of UL MIMO system with partial use of reciprocity for enhanced beamforming.

Proposal 2: RAN1 considers further study on dual precoder design based on combined use of reported-  and reciprocity-based CSI for uplink MIMO.

Figure 1 Combined use of reciprocity- and reported-based CSI for uplink MIMO

## UL MU-MIMO based on Interference Feedback

An attractive feature of UL MU-MIMO operation in LTE is its transparency to the MU-MIMO UE pair. The pairing is performed by the eNB based on finding the best UE pair that exhibits the least correlation between their channels over a given set of resources. Furthermore, there is no coordination between the UE pair to minimize their mutual interference, and the separation of the UE’s transmissions is the eNB’s responsibility. The intra-cell interference problem may be further exacerbated if UE’s will solely rely on channel reciprocity for uplink beamforming.

To improve the performance of the uplink MU-MIMO for NR, MU-MIMO pair may be assisted to reduce their cross interference. An efficient way to implement the mechanism is through using reciprocity-based beamforming to avoid requiring extra reporting and feedback overhead. Figure 2 shows the basic steps of the operation for uplink MU-MIMO based on intereference feedback. In this process, the eNB determines the direction of each uplink transmsion based on the received uplink DMRS of each UE, and it provides the information to the opposing UE in form of a PMI. Then by relying on the channel reciprocity, each UE estimates the uplink channel. The uplink beamforming vector is designed to maximize transmission efficiency in the direction of the uplink channel while minimizing the interference in the direction of the interference PMI.

Proposal 3: RAN1 considers enhanced uplink MU-MIMO operation by relying on reciprocity-based CSI and intereference feedback.

Figure 2 Uplink MU-MIMO based on Interference Feedback

# Summary

The main motivations for use of channel reciprocity is to enhance the performance and avoid unnecessary increase in CSI feedback overhead. In this contribution, we presenetd the examplery use cases of channel reciprocity to enhance the performance of uplink MIMO in NR. Based on the discussion, the following proposals are presented;

Proposal 1: RAN1 considers study on the effect of interference asymmetry on the use of channel reciprocity for uplink MIMO.

Proposal 2: RAN1 considers further study on dual precoder design based on combined use of reported-  and reciprocity-based CSI for uplink MIMO.

Proposal 3: RAN1 considers enhanced uplink MU-MIMO operation by relying on reciprocity-based CSI and intereference feedback.

# References

[1] “RAN1 Chairman’s Notes”, 3GPP TSG RAN WG1 Meeting #85, Nanjing, China 23rd - 27th May 2016

[2] R1-165654, “WF on CSI Acquisition Framework in NR,” Huawei, HiSilicon, NTT DOCOMO, Intel, China Telecom, ZTE, China Unicom, Deutsche Telecom, Softbank, ETRI, Ericsson, Nokia, CATR, ASB, RAN1 #85, Nanjing, China 23rd - 27th May 2016

[3] R1-165843, “WF on CSI Acquisition in NR,” Huawei, HiSilicon, Samsung, RAN1 #85, Nanjing, China 23rd - 27th May 2016

[4] R1-165808, “WF on NR UL MIMO,” Intel, NTT DOCOMO, Ericsson, RAN1 #85, Nanjing, China 23rd - 27th May 2016

[5] H.A. Lorentz, “The theorem of Poynting concerning the energy in the electromagnetic field and two general propositions concerning the propagation of light,” Amsterdammer Akademie der Wetenschappen, vol. 176, 1896.