3GPP TSG RAN WG1 Meeting #88		R1-1703033

Athens, Greece, 13th - 17th February 2017

Source: 	Ericsson

Title:	Pool design for V2V

Agenda Item:	7.2.1.4

Document for:	Discussion and Decision

# Introduction

During RAN1#86, the following was agreed [1]:

Agreement:

- V2V pool is defined by a repeating bitmap mapped to all subframes except for at least SLSS subframes which are skipped

  - The bitmap length is 16, 20, or 100.

  - FFS if/how to handle the case where the bitmap does not repeat an integer number of times within the DFN period

- The bitmap defines which subframes are allowed for V2V SA/data transmission and/or reception for a pool.

This agreement was extended during RAN1#86bis with the following [2]:

Agreements:

- A pool is configured with a number of reserved subframes such that the bitmap is repeated in an integer number within the DFN range.

  - V2V logical subframe index is not allocated to a reserved subframe.

  - The location of the reserved subframes is indicated by an implicit way (details FFS until next meeting).

In this paper we discuss the indication of the reserved subframes.

# Indication of the reserved subframes

To indicate the reserved subframes, including their locations, we think that it is sufficient to have a simple periodic formula (similar to SLSS). We propose that the reserved subframes can be indicated by both preconfiguration and eNB signaling using the following two parameters:

- The number of reserved subframes (Nreserved).

- The offset (Noffset) with respect to subframe#0.

Given this information, a UE can implicitly derive the location of the subframes by applying the following formula

R = Noffset + i * floor( 10240 / Nreserved )

for i = 1, 2, …. noting that 0 ≤ R ≤ 10240.

We illustrate the application for Noffset = 200 and Nreserved = 51 in Figure 1. We also illustrate the resulting 100-subframe periods after removing the SLSS and the reserved subframes. The pool configuration bit map is repeatedly applied to these periods. Although it is not illustrated here, 16-bit and 20-bit maps are also supported in the spec.

Figure 1. Location of the reserved subframes for Noffset = 200 and Preserved = 200.

Proposal 1:

- The reserved subrames are indicated in the preconfiguration or signaled by the eNB using the following two parameters:

  - The number of reserved subframes (Nreserved).

  - The offset with respect to subframe#0 (Noffset).

- The reserved subframes correspond to the subframes with indices R = Noffset + i * floor( 10240 / Nreserved ) for i = 1, 2, …. noting that 0 ≤ R ≤ 10240.

## Configuration for 160 ms SLSS periodicity

### Configuration without SLSS resources

Consider the configuration in which SLSS is not transmitted. To ensure that the number of available subframes for V2V transmission is a multiple of 100, 40 subframes are reserved (10240 – 40 = 10200). The indices of the reserved subframes are i*256 for i={1, …, 40} plus an additional offset term. This configuration works well for both 20-bit and 100-bit maps.

Proposal 2:

- For the configuration without SLSS:

  - Nreserved = 40.

  - Noffset is (pre)configured.

### Configuration using 2 SLSS resources

Consider the configuration in which 2 resources are used for SLSS with a periodicity of 160 ms. This results in 128 subframes used for SLSS in every 10240-subframe period. To ensure that the number of available subframes for V2V transmission is a multiple of 100, 12 subframes are reserved (10240 – 128 – 12 = 10100). The indices of the reserved subframes are {853, 1706, 2559, 3412, 4265, 5118, 5971, 6824, 7677, 8530, 9383, 10236} plus an additional offset term. This offset term may be determined based on the actual choice of SLSS resources (e.g., to minimize the number of times that 3 resources are excluded to create a 100-subframe period). This configuration works well for both 20-bit and 100-bit maps.

Proposal 3:

- For the agreed SLSS configuration using 2 resources with 160 ms periodicity:

  - Nreserved = 12.

  - Noffset is (pre)configured.

### Configuration using 3 SLSS resources

Consider the configuration in which 3 resources are used for SLSS with a periodicity of 160 ms. This results in 192 subframes used for SLSS in every 10240-subframe period. To ensure that the number of available subframes for V2V transmission is a multiple of 100, 48 subframes are reserved (10240 – 192 – 48 = 10000). The indices of the reserved subframes are i*213 for i={1, …, 48} plus an additional offset term. This offset term may be determined based on the actual choice of SLSS resources (e.g., to minimize the number of times that 3 resources are excluded to create a 100-subframe period). This configuration works well for both 20-bit and 100-bit maps.

Proposal 4:

- For the agreed SLSS configuration using 3 resources with 160 ms periodicity:

  - Nreserved = 48.

  - Noffset is (pre)configured.

# Conclusions

In this paper we have proposed the following:

Proposal 1:

- The reserved subrames are indicated in the preconfiguration or signaled by the eNB using the following two parameters:

  - The number of reserved subframes (Nreserved).

  - The offset with respect to subframe#0 (Noffset).

- The reserved subframes correspond to the subframes with indices R = Noffset + i * floor( 10240 / Nreserved ) for i = 1, 2, …. noting that 0 ≤ R ≤ 10240.

Proposal 2:

- For the configuration without SLSS:

  - Nreserved = 40.

  - Noffset is (pre)configured.

Proposal 3:

- For the agreed SLSS configuration using 2 resources with 160 ms periodicity:

  - Nreserved = 12.

  - Noffset is (pre)configured.

Proposal 4:

- For the agreed SLSS configuration using 3 resources with 160 ms periodicity:

  - Nreserved = 48.

  - Noffset is (pre)configured.

# References

- RAN1#86, Chairman’s notes

- RAN1#86bis, Chairman’s notes