3GPP TSG-RAN WG1 #88b	                                  		                   R1-1704656

Spokane, Washington, 3-7th April 2017

Source:	Mitsubishi Electric

Title:	On phase and frequency tracking for DFTsOFDM

Agenda Item:	8.1.2.4.3

Document for: 	Discussion/Decision

# Introduction

At the last RAN1 NR AH meeting [1], it was decided that:

- NR considers frequency offset and PN compensation for DFTsOFDM

  - FFS the exact method (e.g. pre-DFT /post-DFT insertion of PT-RS, blind detection, DM-RS)

  - Consider receiver complexity, PAPR, modulation order to be supported, etc

Moreover, at the last RAN1#88 meeting [2], it was agreed that usage of PT-RS, e.g. CFO/Doppler correction, is not precluded.

In this contribution, we give our view on phase and frequency tracking for DFTsOFDM.

# Discussion

Phase variations in the time domain can be caused by different phenomena: the presence of phase noise, frequency drifts due to Doppler shift or to insufficient frequency synchronization, etc. The phase variations translating all these effects need compensation.

When mild, these effects can be absorbed by the DM-RS in the channel estimation process. But when strong phase variations are present, more frequent support than the one provided by DM-RS is necessary in order to track the remaining variations at OFDM symbol level. This is particularly crucial in scenarios with sparse DM-RS patterns (e.g. low speed, or long slots with front-loaded DM-RS), where important phase drifts can occur between the latest DM-RS position and the current symbol [3], irrespective of the employed waveform.

It is known for a long time in the literature that OFDM systems are more sensitive to phase noise than single-carrier systems [4], and it is partly for this reason that systems subject to particularly challenging phase noise environments (like for example satellite systems) avoided using OFDM waveforms. Time domain effects are easier to monitor and compensate in the time domain. Time domain training sequences are known to be particularly effective for phase noise and carrier offset compensation [5].

For DFTsOFDM, the insertion of time-domain training sequences can easily be supported by pre-DFT training sequence insertion, for example PT-RS insertion together with data at the DFT input. This gives a better time domain resolution and means of tracking time domain phenomena than in the case of OFDM. Indeed, in OFDM, PT-RS inserted in the frequency domain are spread by the IDFT in the time domain, and only an observation with a minimum resolution of one OFDM symbol is possible.

With DFTsOFDM, specific precautions need to be taken in order not to break the PAPR property. Several means of achieving CFO and PN compensation were discussed at the last meeting:

- Pre-DFT PT-RS insertion

- Post-DFT PT-RS insertion

- Blind detection

For simplicity we refer to this RS as PT-RS, but it can serve as support for CFO/Doppler compensation also, as explained here-above

## Pre-DFT PT-RS insertion

Pre-DFT PT-RS insertion can be easily achieved based on a scheme as depicted in Figure 1

Figure 1 Pre-DFT PT-RS insertion for DFTsOFDM

The PT-RS sequence is a known sequence whose overhead can be controlled per user by the amount of symbols K in the known sequence. As long as the symbols of the PT-RS sequence do not have higher modulus than the data constellation symbols, the PAPR of the resulting signal is strictly the same between DFTsOFDM symbols carrying PT-RS and DFTsOFDM symbols carrying data only. Since phase errors can be absorbed by the DM-RS in the channel estimation process and phase drifts are more important away from the DM-RS position, one way of reducing the overhead of the PT-RS sequence is, for example, to take into account the DM-RS positions.

Pre-DFT PT-RS insertion yields good performance results without imposing any high complexity processing, as already shown in [6], [7] and confirmed in Figure 2 and Figure 3.

The presence and density of PT-RS should be UE-specifically configured, depending at least on the modcod and spectral allocation. There is a tradeoff between the FER gain brought by improved phase tracking on the PT-RS sequence and the spectral efficiency penalty brought by PT-RS insertion, as it can be seen from Figure 2. PT-RS insertion can bring important SNR gain, being able to efficiently compensate as well the phase noise and the residual CFO, as it can be seen from Figure 3.

Results were obtained on a CDL-A 300ns channel. Realistic channel estimation based on a full front-loaded DM-RS was performed before phase noise compensation. Phase tracking is performed at symbol level, assumed phase noise being the pole-zero model in [8] at Fc=30GHz. A residual carrier frequency offset (CFO) uniformly distributed in the interval [-0.1 0.1]ppm is considered in Figure 3. We considered the following numerology: BW=80MHz, Fs=122.88MHz, SCS = 120kHz, N_FFT=1024. Here, we used a Zadoff-Chu sequence as PT-RS, and 50RB (full) allocation was assumed.

Observation: Pre-DFT PT-RS insertion yields good performance results with low complexity processing while conserving the PAPR.

Figure 2 Phase noise compensation performance with different PT-RS length, DFTsOFDM

Figure 3 Phase noise compensation performance at different coding rates, DFTsOFDM

## Post-DFT PT-RS insertion

Post-DFT PT-RS insertion can be achieved based on schemes as depicted in Figure 4. PT-RS can be inserted onto carriers non-occupied by data, or can puncture occupied subcarriers. From a complexity point of view, in the first case, DFTs of different sizes need to be implemented. In the second case, data puncturing leads to demodulation performance degradation.

Both insertion solutions lead to degraded PAPR. Nevertheless, puncturing in Figure 4b) is less PAPR destructive than (M-K)-point DFT in Figure 4a). Moreover, (M-K)-point DFT has some increased complexity (extra DFT sizes, TBS variations). Puncturing is preferred over (M-K)-point DFT post-DFT insertion.

At the receiver side, CFO/PN compensation is conducted in the frequency domain, as in OFDM. Only an average observation is thus available for the DFTsOFDM symbols carrying PT-RS, less precise than in the case of pre-DFT PT-RS insertion.

Observation: In the case of post-DFT PT-RS insertion, puncturing is preferred over (M-K)-point DFT.

| - Post-DFT PT-RS insertion with (M-K)-point DFT | - Post-DFT PT-RS puncturing |
| --- | --- |

Figure 4 Post-DFT PT-RS insertion

## Blind methods

Blind PN compensation methods were also reported in the literature, e.g. [9] . Since no PT-RS insertion is needed, there is no PAPR impact. These methods nevertheless usually require iterative methods and are more complex than RS-based methods. Performance is variable depending on the severity of the phase noise [7].

# Conclusion

The conclusions of the above analysis is summarized in the following table

|  | PAPR preserving | Low complexity |
| --- | --- | --- |
| Pre-DFT insertion | Yes | Yes |
| Post-DFT insertion | No | Yes |
| Blind detection | Yes | No |

Proposal: Support pre-DFT PT-RS insertion for DFTsOFDM.

# References

### Chairman’s notes, RAN1 NR AH, Spokane, Washington, January 2017.

### Chairman’s notes, RAN1#88, Athens, Greece, February 2017.

### R1-1702617, Phase and frequency tracking reference signal considerations, Qualcomm, Athens, Greece, February 2017.

### T. Pollet, M. V. Bladel, and M. Moeneclaey, “BER sensitivity of OFDM systems to carrier frequency offset and wiener phase noise,” IEEE Trans. Commun., vol. 43, pp. 191–193, Feb.–Apr. 1995.

### Luc Deneire, Bert Gyselinckx, and Marc Engels, ”Training Sequence versus Cyclic Prefix—A New Look on Single Carrier Communication,” IEEE Commun. Letters, Vol. 5, no. 7, July 2001.

### R1-1700646 On the need of PT-RS for DFTsOFDM, Mitsubishi Electric, Spokane, Washington, January 2017.

### R1-1702332, On PTRS for DFT-s-OFDM, Interdigital, Athens, Greece, February 2017.

### R1-163984, “Discussion on Phase Noise Modelling,” Samsung, RAN1#85, Nanjing, China, May 2016.

### Gomaa, A., & Al-Dhahir, N. (2012). Blind phase noise compensation for SC-FDMA with application to LTE-uplink. In IEEE International Conference on Communications. (pp. 4305-4309).