3GPP TSG RAN WG1 Meeting #89			R1-1707225

Hangzhou, P. R. China, 15th – 19th May 2017

Source:		SoftBank

Title:	NCJT on TM9

Agenda Item:	6.2.4.4

Document for:	Discussion

Introduction

As agreed in RAN1#88bis, RAN1 is requested to study further on the support of NCJT on TM9.

Agreements:

  - NC-JT is supported at least for TM10

    - FFS on applicability of subset of enhancements on TM9

This contribution shares an initial analysis on the benefit of NCJT on TM9.

Discussions

TM9 is now being deployed in the practical network while TM10 is not so common. Assuming TM10 is not implemented by a UE, the UE needs to start from implementing minimum TM10 features (i.e. interference measurement etc.) to support NCJT specific features. This means that NCJT on TM10 may have less advantage over NCJT on TM9 in terms of UE implementation efforts, though TM10 features have some commonality with NCJT from standard point of view. Thus, we see a possibility that NCJT on TM9 may ease the UE implementations compared to that on TM10.

Observation:

NCJT on TM9 may have a potencial benefit in terms of UE complexity while NCJT on TM10 have a performance benefit.

If NCJT on TM9 is specified, we should aim at the simplest design to ease the UE implementation. Otherwise, the benefit of NCJT on TM9 cannot be justified.

Resource allocation

It is proposed by a company to employ only fully overlapped resource allocation for NCJT (see R1-1705006) to simplify the UE equalizer as well as the design of feedback mechanisms. This concept should be agreed at least to TM9 to achieve a simple system design.

Proposal 1:

If NCJT on TM9 is specified,

  - only fully overlapped resource allocation from different TPs is supported.

DPS as a fallback scheme

The motivation to support NCJT is to achieve a visible gain for 4-Rx UEs by 4-layer transmission utilizing two 2-Tx-TPs. Thus, we don’t think that DPS (i.e. switching to a TP not belonging to the serving cell) is an essential feature. Thus, we propose the following:

Proposal 2:

If NCJT on TM9 is specified,

  - DPS (i.e. dynamic switching to non-serving TP(s) ) is not supported as a fallback scheme.

Feedback design

It seems to be straightforward and simple to employ two CSI processes given proposals 1 and 2 above:

CSI process 0: CSI assuming TP1 is serving alone.

CSI process 1: CSI assuming TP1 and TP2 are jointly serving, where CSI-RS from TP1 and TP2 are not necessarily quasi-colocated.

Such a two-process feedback would be feasible by reusing class B CSI with K>1. Furthermore, this mechanism doesn’t require the support of interference measurement by ZP-CSI-RS, which is very beneficial for TM9.

Proposal 3:

If NCJT on TM9 is specified,

  - Class B CSI with K>1 (specifically K=2) should be reused.

    - One is used for single point transmission, and the other is used for NCJT.

    - RAN1 needs to confirm that a sufficient gain can be achieved without IMR.

Note that this CSI mechanism may require new codebook for optimization. This aspect can be discussed after the framework design of NCJT on TM10 is finalized.

Conclusion

We propose that NCJT on TM9 (if specified) should be extremely simple to ease the UE implementation compared with TM10. The detailed technical proposals are summarized as follows:

If NCJT on TM9 is specified,

  - only fully overlapped resource allocation from different TPs is supported.

  - DPS (i.e. dynamic switching to non-serving TP(s) ) is not supported as a fallback scheme.

  - Class B CSI with K>1 (specifically K=2) should be reused.

    - One is used for single point transmission, and the other is used for NCJT.

    - RAN1 needs to confirm that a sufficient gain can be achieved without IMR.