3GPP TSG RAN WG1 #89 	R1-1707520

Hangzhou, China, 15th - 19th May 2017

Source:	CATT

Title:	IR-HARQ scheme for eMBB LDPC codes

Agenda Item:	7.1.4.1.1.3

Document for:	Discussion and Decision

Introduction

In RAN1 #88 meeting, the following agreement has been reached for eMBB channel coding [1].

Agreement:

- Built-in puncturing of systematic bits is supported for LDPC coding, that is:

  - At least for the initial transmission, the coded bits are taken after skipping the first Nsys,punct systematic bits

- Nsys,punct is selected from: 0, Z, and 2*Z

  - Circulant weight is the number of superimposed circularly shifted ZZ identity matrices

- The rate matching for LDPC code is circular buffer based (same concept as in LTE)

  - The circular buffer is filled with an ordered sequence of systematic bits and parity bits

- FFS: Order of the bits in the circular buffer

  - For IR-HARQ, each Redundancy Version (RV), RVi, is assigned a starting bit location Si on the circular buffer

  - For IR retransmission of RVi, the coded bits are read out sequentially from the circular buffer, starting with the bit location Si

  - Limited buffer rate matching (LBRM) is supported

This contribution discusses the IR-HARQ scheme for eMBB LDPC codes.

Discussion

2.1 Interleaver for LDPC codes

According to the different  bit interleaver schemes, we compared BLER performance of the following three schemes,

- transmitted without interleaver,

- the information bits and the parity bits entering the interleaver separately,

- the whole codeword entering the interleaver.

Figure 1: Performance comparison of LDPC codes with different interleaving schemes

From the results of simulation, LDPC codes with interleaver shows better performance in fading channel (QPSK modulation, Fd=200, Ts=0.00005). At the BLER of 10-3, the performance gain of LDPC codes with interleaver for both schemes is about 0.5dB over that without interleaver. However, LDPC codes with interleaving over the whole codeword (scheme 3) shows slightly better performance compared with that interleaving separately (scheme 2)

Observation1: Interleaving the whole codeword of LDPC codes shows better performance than interleaving information bits and parity bits of LDPC codes separately.

2.2 IR-HARQ scheme for LDPC codes

For IR-HARQ, each Redundancy Version (RV), (), is assigned a starting bit location  on the circular buffer.  We choose the starting bit position  from the following expression [3]:

Where is the codeword length and is the length of puncture bits.

We propose three IR-HARQ schemes with the starting positions of each retransmission fixed in the following,

(1)Scheme 1: retransmission without punctured bits

Figure 2: IR-HARQ scheme of LDPC codes (transmission without punctured bits)

The IR-HARQ transmission scheme for LDPC code with maximum 4 retransmissions is illustrated in Figure 2. The details are as follows.

- In the first transmission, the block of length N is transmitted from the location of . The length of the information bits is K and the code rate is R;

- In the second transmission, parity bits of length N are generated and transmitted from the location of .  The effective code rate after combining is R/2;

- In the third transmission, retransmission starting point shifts to right by lengthfrom the location of. The effective code rate after combining is R/3;

- In the fourth transmission, retransmission starting point shifts to right by lengthfrom the location of . Then the remaining parity bits and some information bits are transmitted. Note that we bypass the punctured bits in this retransmission. The block length is N. The effective code rate after combining is R/4

Note that: At the receiver, the current LLRs can be combined with LLRs stored in the buffer.   The combined soft bits are delivered to the decoder as a soft input for iterative decoding.

(2)Scheme 2(punctured bits transmitted in the fourth retransmission)

Figure 3: IR-HARQ scheme of LDPC codes (transmitting puncture bits in the fourth transmission)

This IR-HARQ scheme of LDPC code with maximum 4 retransmissions is illustrated in Figure 3. The details are as follows.

- In the first transmission, the block of length N is transmitted from the location of . The length of the information bits is K and the code rate is R;

- In the second transmission, parity bits of length N are generated and transmitted from the location of .  The effective code rate after combining is R/2;

- In the third transmission, retransmission starting point shifts to right by lengthfrom the location of . The effective code rate after combining is R/3;

- In the fourth transmission, retransmission starting point shifts to right by lengthfrom the location of .Then,  the remaining parity bits ，the punctured bits and some information bits are transmitted. The block length is N. The effective code rate after combining is R/4

(3)Scheme 3(punctured bits transmitted in the second and fourth retransmissions)

Figure 4: IR-HARQ scheme of LDPC codes (transmitting puncture bits in the second and fourth transmission)

LDPC code with maximum 4 retransmissions is illustrated in Figure 4. The details are as follows.

- In the first transmission, the block of length N is transmitted from the location of. The length of the information bits is K and the code rate is R;

- In the second transmission, transmitted all the puncture bits and the parity bits from the location of , the block size is N. The effective code rate after combining is R/2;

- In the third transmission, retransmission starting point shifts to right by lengthfrom the location of . The effective code rate after combining is R/3;

- In the fourth transmission, retransmission starting point shifts to right by lengthfrom the location of .Then, the remaining parity bits, the puncture bits and some information bits are transmitted. The block length is N. The effective code rate after combining is R/4

(4)Scheme 4(punctured bits transmitted in the second transmission)

Figure 5: IR-HARQ scheme of LDPC codes (transmitting puncture bits in the second transmission only)

LDPC code with maximum 4 retransmissions is illustrated in Figure 5. The details are as follows.

- In the first transmission, the block of length N is transmitted from the location of. The length of the information bits is K and the code rate is R;

- In the second transmission, transmitted all the puncture bits and the parity bits from the location of , the block size is N. The effective code rate after combining is R/2;

- In the third transmission, retransmission starting point shifts to right by lengthfrom the location of . The effective code rate after combining is R/3;

- In the fourth transmission, retransmission starting point shifts to right by lengthfrom the location of .Then, the remaining parity bits and some information bits are transmitted. The block length is N. The effective code rate after combining is R/4

(a) Performance comparison between scheme 1, scheme 2 and scheme 3 with four retransmissions.

(b) Performance of the second transmission              (c) Performance of the third transmission

(d) Performance of the fourth transmission

Figure 6: Performance comparison between scheme 1, 2 and 3(with K=256, 1Tx code rate R=1/2, QPSK modulation)

As shown in Figure 6, the scheme 2 and scheme 3(retransmission with puncture bits) outperform the scheme 1(retransmission without puncture bits). Moreover, according to the performance comparison, transmitting the puncture bits in the second transmission exhibits better performance than in the fourth transmission. Therefore, for the LDPC codes with built-in puncturing columns, retransmitting the punctured information bits in the HARQ scheme should be preferred, and the earlier the better.

Observation2: For eMBB LDPC codes with build-in punctured columns, retransmitting the punctured information bits in the HARQ scheme shall be better.

(a) Performance comparison between scheme 3 and scheme 4 with four retransmissions.

(b) Performance of the fourth transmission

Figure 7: Performance comparison between scheme 3 and scheme 4(with K=256, 1Tx code rate R=1/2, QPSK modulation)

As shown in Figure 7, the scheme 3 (transmitting puncture bits in the second transmission and fourth transmission) and scheme 4 (transmitting puncture bits in the second transmission only) have almost the same performance.

Observation3: No gains can be achieved by retransmitting the punctured information bits over and over again.

Conclusion

The above discussion is summarized with following observations and proposals:

- Observation 1: Interleaving the whole codeword of LDPC codes shows better performance than interleaving information bits and parity bits of LDPC codes separately.

- Observation2: For eMBB LDPC codes with build-in punctured columns, retransmitting the punctured information bits in the HARQ scheme shall be better.

- Observation3: No gains can be achieved by retransmitting the punctured information bits over and over again.

- Proposal 1: Retransmitting the punctured information bits in the HARQ scheme should be supported for NR LDPC codes.

References

[1] Chairman’s Notes, 3GPP TSG RAN WG1 #88 meeting.

[2] R1-1700240, “IR-HARQ scheme for LDPC codes”, CATT.

[3] R1-1700248, “Bit rearrangement of LDPC for high order modulation”, ZTE.

[4] R1-1704589, “IR-HARQ scheme for eMBB LDPC codes”, CATT