3GPP TSG RAN WG1 Meeting#89	R1-1709033

# Hangzhou, P.R. China, 15th – 19th May 2017

Agenda item:	7.1.2.2.3

Source: 	Samsung

Title: 		   System level evaluation results for beam management

Document for:	Discussion and Decision

# Introduction

Agreements made on beam management in RAN1 #88bis [1] can be summarized as follows:

Agreements:

- For UE RRC connected mode, periodic signal is supported at least for P1 procedure (Tx/Rx beam alignment) using following options in addition to UE-specifically configured CSI-RS. Down selection from following options will be conducted in the next meeting.

  - Opt. 1: SS blocks

  - Opt. 2: Cell-specifically configured CSI-RS

    - Configuration of CSI-RS is obtained from the broadcast message (e.g., MIB, SIB)

  - Opt. 3: No additional option

Agreements:

- Aim for low-overhead indication for spatial QCL assumption to assist UE-side beamforming/receiving

  - FFS details (e.g., tag-based where the tag refers to previous CSI-RS resources, BPL-based, referring to previous measurement reports, indication one resource (set) out of multiple resource (set)s configured by RRC, CSI-RS resource/port index based, etc.)

Agreements:

- Confirm the WA from RAN1 AH1701 with the following update:

  - NR supports at least one NW-controlled mechanism for beam management for UL transmission(s)

    - At least when beam correspondence does not hold

    - Considering at least SRS to support U-1/U-2/U-3 procedures

    - FFS the details

Agreements:

- Study whether or not the UE to provide information to gNB to assist UL beam management without UE beam correspondence

  - E.g., the amount of SRS resources that is needed to train UE Tx beams, based on DL beam management results if available

- Study whether and how UE to use same transmission power for SRS transmission during one round of beam sweeping

  - E.g., derived from beam-specific power control signalling and maximum transmit power

  - FFS: spec. impact

Agreements:

- For beam reporting, companies are encouraged to perform detailed analysis w.r.t. comparing Alt 1 and Alt 2, particularly considering the overhead (feedback overhead, signaling overhead, etc.), performance, flexibility in operation, etc.

  - Aim to down-select one of the two alternative s with the possibility of merging into a single alternative (if so, the corresponding analysis) at next meeting

  - Each company to state the assumed UE implementation in the analysis

In this contribution, we present system level simulation results and analysis on beam measurement.

# Evaluation Results and Analysis

One of the main issue related to beam measurement is the accuracy needed for beam measurement, which will decide both the time and frequency domain density of reference signals. In order to find the necessary accuracy for beam measurement, we simulate how system performance will change as error occurs in the beam measurement procedure. If there is no beam measurement error, the analog beam of gNB will directly point to the AoD and EoD of the selected UE. If there are beam measurement errors, the analog beam will point to the AoD and EoD plus error. The error angles are uniformly distributed within range [-E, E], where E is called the BM error in the simulation. Other simulation parameters are in the Appendix A.

- (b)

Figure 1: DL SINR (a) and spectral efficiency for UMa 30GHz

- (b)

Figure 2: DL SINR(a) and spectral efficiency for UMi 30GHz

Figure 1 and Figure 2 shows the simulation results on the SINR and spectral efficiency for both UMi and UMa 30GHz. As we can see from both figures, when the BM error increases, the system performance will decrease. Especially, we found that when the BM error is smaller than the 3dB-beam width angle, the system performance degradation is small. Note that the antenna pattern used for gNB in this simulation is (4,8,2,2,2). The 3dB-beam width in azimuth domain is 6.4 degrees. Therefore, we can say that for both UMi and UMa 30GHz NR, the reference signals pattern should be design so that the beam measurement accuracy is smaller than the 3dB-beam width of the antenna at gNBs.

Observation 1: For both UMi and UMa 30GHz NR, when the beam measurement error is within the 3-dB beam width of the gNB, the system performance, e.g., spectral efficiency, does not degrade too much.

By the UT mobility modelling in TR38.900[2], the cluster departure angles( and) and arrival angles ( and) are updated as

where  is 2D distance between Tx/Rx;  is 3D distance between Tx/Rx; , ,  and  are cluster specific reflection surface angles. These angles equal 0 for LOS path and are generated using spatially consistent random numbers with uniform distribution U(-180o, 180o) and 50m correlation distance for  and ; and U(-90o, 90o) and 100m correlation distance for  and .

Assume the same antenna configuration in Appendix A is considered at a gNB. In UMi case, the minimum gNB to UE distance is 10 meters. When the UE speed is 3km/h, the period when the UE moves out of the 3dB-beam width, i.e., the period of beam measurement, can be estimated according to

seconds.

The result of 2.68 seconds is based on the minimum BS to UE distance. Based on the UE location, the distance from UE to gNB will be larger than the minimum distance of 10 meters. Accordingly, the BM period can be increased. In the UMa case, the UE speed can be larger as well as the UE to BS distance, which may not change the level of beam measurement period, which is in several seconds or at least in the level of 0.1 seconds. Beam measurement more frequently will not increase the system performance but increasing the system overhead. To see the above phenomenon more directly, system level simulation on the periodicity of beam measurement is conducted. The results are shown in the following figures.

- (b)

Figure 3 DL SINR(a) and spectral efficiency for UMa 30GHz with different BM period

In Figure 3, the spectral efficiency is calculated every slot, where the slot length is 0.25ms. There are totally 100 drops and in each drop 4000 slots are simulated. Two different beam measurement periods (2.5ms and 15ms) with two different UE speeds (3km/h and 120km/h) are simulated. From this figure, we can find that there is virtually no performance loss with beam measurement period as large as 15 miliseconds (60 slots). Note that the results here are geometry-based simulation results and there is no small scale fading considered. This is the reason why the two curves with different beam measurement period are exactly the same. Doppler and multipath effect may make the beam measurement performance worse.

Observation 2: For 30GHz NR, there is virtually no performance loss with beam measurement period as large as 15 ms (60 slots for 60kHz SCS).

# Conclusions

This contribution presents our system level simulation results on beam management. The following observation is made:

Observation 1: For both UMi and UMa 30GHz NR, when the beam measurement error is within the 3-dB beam width of the gNB, the system performance, e.g., spectral efficiency, does not degrade too much.

Observation 2: For 30GHz NR, there is virtually no performance loss with beam measurement period as large as 15 ms (60 slots for 60kHz SCS).

# References

[1] RAN1#88 meetings chairman’s note, February 2017.

[2] TR 38.900, Study on channel model for frequency spectrum above 6GHz, 3GPP

[3] R1-1703533, Summary of [NRAH-01] email discussion on evaluation on beam management, ZTE

Appendix A

Table 1 Simulation assumption for system level simulation

| Attributes | Values of assumptions |
| --- | --- |
| Scenarios (Carrier Frequency) | - Urban macro: 4 GHz, 30GHz; - Dense Urban: - For 4 GHz: Evaluate macro layer - For 30 GHz: Evaluate micro layer Note: other antenna configurations should be considered as well. |
| Mode | DL SU-MIMO |
| Simulation bandwidth | 4GHz: 20MHz (DL) 30GHz: 80MHz.(DL) |
| Channel Model | Following related assumption in TR 38.802 |
| TXRU mapping to antenna elements | 2D DFT based beam per polarization as a baseline; - 4GHz: 1D DFT per vertical dimension per polarization as baseline; |
| TXRU mapping weights | All TXRUs in the gNB are used for the selected UE. |
| Criteria for selection for serving TRP | Maximum RSRP. |
| Criteria for beam selection for serving TRP | According to the AoD and EoD |
| Constraints for the range of selective beams per TRP sector | No constraints |
| BS antenna configurations | For 4GHz: (M,N,P, Mg,Ng) = (8,8,2,1,1) as baseline. (dV,dH) = (0.8, 0.5)λ. For 30GHz: (M,N,P, Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | For 4GHz: According to TR36.873 For 30 GHz: According to TR38.802 |
| UE antenna configurations | For 4GHz: Mg = 1, Ng = 1, P =2, (dV,dH) = (0.5, 0.5)λ, MxNxP<=8 (companies report M,N) For 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. *Θmg,ng=90 deg; Ω0,1=Ω0,0+180 (deg); |
| UE antenna element radiation pattern | For 4GHz: Omni-directional with 5dBi gain For 30GHz: See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration | Ideal, non-idea following 38.802 (optional) |
| UE receiver type | MRC receiver |
| BF scheme | Analog beamforming |
| UE mobility feature | Follow Phase 3 calibration i.e. Add-on features including UE mobility, rotation, blockage, etc. can be considered. Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained |
| MCS | Use LTE MCS |
| Metric | Spectral efficiency (evaluated under full buffer) |