3GPP TSG RAN WG1 Meeting 90bis															R1-1718675

Prague, CZ, 9th – 13th, October 2017

Agenda item:		7.1.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining details of CB segmentation

Document for:		Decision

# 1 	Introduction

In RAN1 #90 meeting, several agreements were made on code segmentation including how to use the maximum block size supported by the base graphs in the code segmentation procedures. In this contribution, we provide details for the remaining items in code segmentation procedure.

# 2.    Discussion

## 2.1	TBS determination to provide similar CBS

Ran1 #90 meeting agreed to the following [1],

Agreement:

- Equal code block size after segmentation

- Working Assumption: TBS determination procedure ensures that TBS plus TB-CRC can be factored into the number of CBs multiplied by the CBS (before addition of LDPC encoding filler bits).

  - (If a special case emerges where the TBS determination procedure cannot achieve the above criterion, equal CBS would be achieved by zero-padding.)

The TBS table can be significantly large considering the different slot configurations, DMRS assumptions, use of control resources and many other flexibilities provided in NR. Therefore, having a TBS table as in LTE can be cumbersome, and companies are considering simplifying the TBS calculations with a formula. When using a predefined formula, it should consider the number of (available) REs, the intended spectral efficiency (MCS), and the number of MIMO layer. Also, we agreed to have TBS determination procedure such a way that we ensure the equal size of CB in the segmentation process. The following formula helps to obtain such features in the segmentation process.

The TBS can be calculated using the following formula.

TBS = NRE×Qm×R×v 8* NRE×Qm×R×v +24Kcb-24* 8* NRE×Qm×R×v +24Kcb-24

where,

NRE = Resource elements

Qm = Modulation order

R = code rate

v = number of layers

Kcb= maximum code block size

As the number of REs is used for the calculation, the gNB and the UE need to have the common understanding of the definition. This would likely require some pre-known assumptions on the overhead. It is possible that TBS calculations with formula could lead to incorrect calculations at the gNB and UE if the implementations are fixed-point with lower precision. Such situation should be avoided by including a reference table with the specification.

Proposal 1: TBS determination can be done as TBS = NRE×Qm×R×v 8* NRE×Qm×R×v +24Kcb-24* 8* NRE×Qm×R×v +24Kcb-24, where NRE = resource elements, Qm = modulation order, R = code rate, v = number of layers, Kcb= max CBS to ensure that TBS plus TB-CRC can be factored into the number of CBs multiplied by the CBS.

## 2.2 	Segmentation with BG #2

In Ran1 NR AH#3, the following discussion was happened on CB segmentation with the base graph #2.

Agreement:

- Rinit value to be used in the Working Assumption from RAN1#90 is

  - 0.25

Working Assumption from RAN1#90, to be checked after finalisation of the TBS table and confirmed if TBSs exist for which the following is meaningfully beneficial and does not cause meaningful degradation:

- For initial transmissions with code rate Rinit > 1/4, BG2 is not used when TBS>3824

  - If the FFS on UE capabilities w.r.t. support of both BGs is resolved such that it is possible that a UE does not support BG1, then the above bullet only applies if the UE supports BG1.

- BG2 is used for initial transmissions with code rate Rinit <= ¼ for all TBS supported at that code rate

  - For BG2 with TBSs larger than 3824, the TB is segmented into CBs no larger than 3840

Alternative proposal: BG2 is not used when TBS>3824.

For the Rinit lower than or equal to 1/4, TBS greater than 3824 can be segmented into CBs with the maximum size supported by BG #2. In Figure 1, we provide BLER versus SNR for several TBS for code ¼. We consider two cases,

- TBS segmentation is done only with BG#1. (BG1)

- TBS segmentation is done with BG#2 for lower rates. (BG2)

Figure 1: Rate 1/4 performance with BG#1 and BG#2

As observed in Figure 1, there is some performance gain for rate ¼ with BG2 segmentation compared to using BG1 for the TB sizes which are larger than 4K.

Observation 1: For Rate ¼, there is about 0.2 dB gain by using BG#2 and segmenting the TBS above 4K.

Then the remaining question is how often we use these TBSs in practice. We assumed a possible MCS table in NR and checked the TBS based on the formula given in Section 2.1. The TBS calculations used Kcb = 8448 for rates above ¼ and Kcb = 3840 for below that. Figure 2 illustrates the possible outcome of such a TBS table.

MCS increasing

Number of REs increasing

|  |  |  | No segmentation |  |  |  |  |  |  |  |  |  |  |  | Segmentation with BG#2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Segmentation with BG #1 |  |  |  |  |  |  |
|  | No segmentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 2: Partitions of TBS table

In Table 1, we provide the estimated TBS for larger PRB allocations like 50, 100, 200 and 275. 275 PRB is the maximum number that can be allocated in release 15. We see that TBS is not exceeding 18000 for rates below ¼, and it is hard to imagine large resource allocations for lower code rates.

Table 1: TBS calculations with different REs.

| MCS | Modulation Order | Code rate | SE | #PRB and #REs (assuming 14 symbol slot with 11 used for data) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 PRB | 100 PRB | 200 PRB | 275 PRB |
|  |  |  |  | 6600 RE | 13200 RE | 26400 RE | 36300 RE |
| 0 | 2 | 1/16 | 0.125 | 832 | 1656 | 3304 | 4544 |
| 1 | 2 | 1/8 | 0.25 | 1656 | 3304 | 6608 | 9096 |
| 2 | 2 | 3/16 | 0.375 | 2480 | 4960 | 9912 | 13632 |
| 3 | 2 | 1/4 | 0.5 | 3304 | 6608 | 13216 | 18160 |
| 4 | 2 | 5/16 | 0.625 | 4128 | 8256 | 16512 | 22704 |
| 5 | 2 | 3/8 | 0.75 | 4952 | 9904 | 19800 | 27232 |
| 6 | 2 | 7/16 | 0.875 | 5776 | 11552 | 23112 | 31776 |
| 7 | 2 | 1/2 | 1 | 6600 | 13200 | 26400 | 36320 |
| 8 | 2 | 9/16 | 1.125 | 7432 | 14864 | 29728 | 40840 |
| 9 | 2 | 5/8 | 1.25 | 8256 | 16512 | 33024 | 45408 |
| 10 | 4 | 3/8 | 1.5 | 9904 | 19800 | 39600 | 54488 |
| 11 | 4 | 7/16 | 1.75 | 11552 | 23112 | 46224 | 63552 |
| 12 | 4 | 1/2 | 2 | 13200 | 26400 | 52808 | 72648 |
| 13 | 4 | 9/16 | 2.25 | 14864 | 29728 | 59456 | 81680 |
| 14 | 4 | 5/8 | 2.5 | 16512 | 33024 | 66048 | 90816 |
| 15 | 4 | 11/16 | 2.75 | 18168 | 36320 | 72648 | 99840 |
| 16 | 4 | 3/4 | 3 | 19800 | 39600 | 79200 | 108992 |
| 17 | 6 | 9/16 | 3.375 | 22296 | 44592 | 89144 | 122520 |
| 18 | 6 | 5/8 | 3.75 | 24768 | 49536 | 99072 | 136136 |
| 19 | 6 | 11/16 | 4.125 | 27232 | 54488 | 108992 | 149760 |
| 20 | 6 | 3/4 | 4.5 | 29728 | 59456 | 118800 | 163360 |
| 21 | 6 | 13/16 | 4.875 | 32192 | 64384 | 128768 | 177056 |
| 22 | 6 | 7/8 | 5.25 | 34680 | 69336 | 138720 | 190624 |
| 23 | 8 | 11/16 | 5.5 | 36320 | 72648 | 145296 | 199680 |
| 24 | 8 | 3/4 | 6 | 39600 | 79200 | 158536 | 217984 |
| 25 | 8 | 13/16 | 6.5 | 42912 | 85800 | 171696 | 236176 |
| 26 | 8 | 7/8 | 7 | 46224 | 92400 | 184800 | 254200 |
| 27 | 8 | 15/16 | 7.5 | 49536 | 99072 | 198144 | 272448 |
| 28 | 2 | Reserved |  |  |  |  |  |
| 29 | 4 |  |  |  |  |  |  |
| 30 | 6 |  |  |  |  |  |  |
| 31 | 8 |  |  |  |  |  |  |

Observation 2: When the code rates are lower than ¼, it is hard to see meaningful benefits by using CB segmentation with base graph #2. As larger TBS only appear when the resource allocation is huge, this could be a feature that may never be used in practice.

Proposal 2: BG2 is not used when TBS>3824.

# 3 	Conclusions

In this contribution, we discuss the remaining details of segmentation and we have following observation and proposals,

Observation 1: For Rate ¼, there is about 0.2 dB gain by using BG#2 and segmenting the TBS above 4K.

Observation 2: When the code rates are lower than ¼, it is hard to see meaningful benefits by using CB segmentation with base graph #2. As larger TBS only appear when the resource allocation is huge, this could be a feature that may never be used in practice.

Proposal 1: TBS determination can be done as TBS = NRE×Qm×R×v 8* NRE×Qm×R×v +24Kcb-24* 8* NRE×Qm×R×v +24Kcb-24, where NRE = resource elements, Qm = modulation order, R = code rate, v = number of layers, Kcb= max CBS to ensure that TBS plus TB-CRC can be factored into the number of CBs multiplied by the CBS.

Proposal 2: BG2 is not used when TBS>3824.

# References

- 3GPP RAN1 #90, Chairman Notes.