3GPP TSG RAN WG1 Meeting #93	R1- 1805985

Busan, Korea, May 21 – 25, 2018

Agenda Item:	6.2.5.3

Source:	Huawei, HiSilicon

Title:	Solutions for Rel-14 and Rel-15 UE Pool sharing

Document for:	Discussion and decision

# Introduction

At the RAN1#92 meeting, the following was agreed [1]:

- Rel-15 Mode 3 UEs shall set the resource reservation field in SCI-1 to the SPS period.

In addition, RAN2 agreed on the following scenarios to be supported/unsupported at RAN2 #101b meeting:

- Support resource pool sharing between Rel-15 mode-3 and Rel-15 mode-4 UE.

- Support resource pool sharing between Rel-15 mode-3 and Rel-14 mode-4 UE.

- Not support resource pool sharing between Rel-14 mode-3 and Rel-15 mode-4 UE.

At the RAN1#92b meeting, although no agreements were reached, different options of resource pool sharing are proposed and discussed offline [2]:

- Option 1: To enable Mode-3 UE to do sensing/measurement and report to network on the sensing result, in order for network scheduler to be aware of the interference environment.

  - Note: Focus on impact to signalling overhead and latency issues.

  - Option 1-1: Sensing reusing Mode-4 sensing mechanism, and reporting S-RSSI on each subframe in previous [x]ms.

    - FFS details, including trigger(s) to send report.

  - Option 1-2: Sensing reusing Mode-4 sensing mechanism, and reporting a bitmap based on S-RSSI threshold in previous [x]ms.

    - FFS details, including determination of the subset of resources, indication of the subset of resource, whether to use periodic report or event-triggered report, whether to use BSR.

  - Option 1-3: If eNB allocated resource is collided with Mode-4 UE, Mode-3 UE sends SR.

- Option 2: No additional specification work to support resource pool sharing between Mode-3 UE and Mode-4 UE in RAN1

- Option 3: Use 1 reserved bit in SCI-1 to indicated Mode-3 UE, then Rel-15 Mode-4 UE can avoid to select occupied resource of Mode-3 UE.

In this contribution, the remaining issues of pool sharing will be discussed.

# Discussion

As listed in the introduction, several options were discussed. Option 1 requires new signaling to request/report sensing measurements. This option definitely improves performance with resource pool sharing. It is still under discussion in RAN2. In addition, according to our RAN2 contributions [3][4], to support the reporting mechanism, the sensing process of mode-4 UE can be reused for mode-3 UE and the work mainly focus on higher layer signaling. Thus we assume that the decision to standardize option 1 will be taken by RAN2, and we will not discuss it in this paper any more.

Option 2, given the previous decision of RAN1 to set the reservation field of the SCI to the SPS periodicity, enables pool sharing while providing a performance benefit for both mode-3 and mode-4 UEs.

Option 3 can have some benefits when Rel-15 mode-4 UEs coexist with Rel-15 mode-3 UEs in the same resource pool. When a mode-4 UE identifies after decoding the SCI that a UE is a mode-3 UE, it skips the step of comparing the RSRP with the threshold and excludes the resources of mode-3 UEs. This bit can also be viewed, and used as a specific case of “a higher priority for mode-3 UE” and can increase the reliability for mode-3 UEs.

Proposal: When Rel-15 mode-4 UEs coexist with Rel-15 mode-3 UEs, reserved bits in SCI format 1 can indicate if a Rel-15 UE uses mode-3 or mode-4.

# Conclusion

In this contribution, the remaining issues of pool sharing are discussed. We propose the following:

Proposal: When Rel-15 mode-4 UEs coexist with Rel-15 mode-3 UEs, reserved bits in SCI format 1 can indicate if a Rel-15 UE uses mode-3 or mode-4.

# References

“RAN1 Chairman’s Notes”, RAN1 #92, Athens, Greece, Feb 26-Mar 2, 2018.

R1-1805392, “Summary of offline discussion for resource pool sharing between Mode-3 UE and Mode-4 UE”, RAN1 #92bis, Sanya, China, Apr. 16-20, 2018.

R2-1808314, “Mode-3 sensing and reporting for resource pool sharing”, RAN2 #102, Busan, Republic of Korea, May 21-25, 2018.

[4]	R2-1808315, “Draft CR on mode-3 sensing and reporting”, RAN2 #102, Busan, Republic of Korea, May 21-25, 2018.