3GPP TSG-RAN WG1 #93

Busan, South Korea, May 21th – 25th, 2018    			       R1-1806124

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on Type - II CSI Reporting Scheme

Agenda item:       7.1.2.2.6

Document for:	Decision

# Introduction

At the previous meetings [1-2], the following was agreed:

Agreements:

- FFS to support a common design of W2 for Cat. 1, Cat. 2 and Cat. 3

  - FFS for Cat. 3, W2 only feedback is allowed

    - FFS amplitude feedback for W2 (e.g., wideband, subband, etc.)

  - Note: this does not mean NR supports all three categories

- FFS whether or not to merge Category 1 and Category 3 using a unified codebook formulation

Agreements:

- Slides 4 to 24 in R1-1709232 are agreed

  - For slide 20, FFS whether or not support frequency-dependent parameterization and if so, the details

  - FFS whether or not to further enhance analog beamforming related operations especially for >1 layers

This contribution is an extension of our previous contribution R1-1804118.

In Type-II CSI reporting, it is assumed that dual-stage precoding is performed per sub-band, i.e., a single precoder is calculated for a group of adjacent PRBs, referred to as ‘subband.’ The dual-stage precoder comprises two components: the first-stage precoder F1 that is identical for all subbands contains the selected entries/beams from a DFT-based codebook, and the second stage precoder F2 contains the subband-dependent beam-combining coefficients. The feedback overhead of the Type-II scheme increases approximately linearly with the number of subbands and becomes considerably large for large numbers of subbands.

The Type II CSI feedback in Rel. 15 New Radio supports only up to 2-layer transmission and 4 orthogonal beam combination for each layer.

However, the performance of Type II codebook is far from being achieved by optimal precoding schemes. Therefore, several enhancements have been proposed to extend the Type II CSI codebook to achieve higher performance gains. For example, extension to 8 beams combination in order to spatially separate multiple users with high resolution and support of >2 layer [3-4] transmissions are few proposals to mention. Inclusion of these enhancements results in a high feedback overhead. The contributions [4-6] have studied feedback reduction schemes by applying a frequency-domain transformation of the second stage precoder.

The problem of increased feedback overhead has been addressed in [7] by considering the design of the precoder in the delay-domain. It has been shown that, in addition to the feedback overhead reduction, precoding directly in the delay-domain which is realized by the so-called space-delay precoder results in enhanced system performance compared to the sub-band-based precoding.

In this contribution, we present a general framework for feedback compression for Type-II CSI reporting which captures the contributions presented in [4], [5] and [7]. Also, simulation results are shown for one of the presented frameworks.

# Compression of second stage precoder F2

In this section, we present a general framework for feedback compression using beam independent and beam dependent transformations.

Assuming rank-1 transmission and a dual-polarized antenna array with configuration (N1, N2, P=2), the conventional double-stage precoder per subband is given by

|  | Fs=F1f2s | (1) |
| --- | --- | --- |

where F1∈CN×2U is a wideband matrix identical for all subbands that contains 2U spatial beams and f2s∈C2U×1 is a subband vector that contains 2U complex frequency-domain combining-coefficients associated with 2U spatial beams.

Collecting the precoders for all subbands S in a matrix F, we obtain

|  | F=F1f20 f21…f2S-1, =F1F2, | (2) |
| --- | --- | --- |

where matrix F2 can also be written as F2=f2,1   f2,2  …  f2,2UT, whose u-th row contains the complex combining-coefficients associated with the u-th beam over all subbands,

f2,u= f2,u0    f2,u1  …  f2,uS-1T∈ CS×1.

Beam-dependent transformation

For the compression of the coefficients in f2,u, we introduce a beam-dependent transformation matrix Wu, such that

|  | f2,u=Wuku, | (3) |
| --- | --- | --- |

where matrix Wu= wu,1  wu,2   …  wu,L is of size S×L, and vector ku= ku1…  kul  … kuLT∈CL× 1  contains L transform-domain complex combining-coefficients associated with the u-th beam. Using (3), the precoder in (2) can be expressed as

|  | F= F1⋅K⋅W, | (4) |
| --- | --- | --- |

where

K=k10L⋮0L0Lk2⋮0L…0L0L⋮k2UT∈C2U×2LU is a block-diagonal matrix containing the transform-domain complex combining-coefficients associated with the 2U spatial beams, and W=W1 W2…W2UT∈C2LU×S is the overall beam-dependent transformation matrix.

Beam-independent transformation

When an identical transformation matrix is considered for all beams, the following holds

W=Wu, ∀u=1, …, 2U,

where W is the beam-independent transformation matrix. Then, the precoder (2) is given by

|  | F= F1⋅K⋅W, | (5) |
| --- | --- | --- |

where K=k1    k2   …   k2UT∈ C2U×L  and  W= w1  w2   …  wLT∈CL×S.

DFT -based transformation matrix

The two contributions [4] and [5] consider the compression of F2 and apply DFT-based matrices for the generation of the transformation matrix W, resulting in delay-domain complex-combining coefficients ku per beam. In [5], the transformation matrix is beam-dependent, and the associated delays with each beam may differ for different beams. In contrast to [5], the transformation matrix in [4] is beam-independent, and the associated delays are identical for all beams.

The approach in [7] uses space-delay precoding to calculate the precoder matrix F. The space-delay precoder can be expressed as

|  | F=K⋅W, | (6) |
| --- | --- | --- |

where K contains the complex delay-domain combining-coefficients, and W is a non-uniform DFT (NU-DFT) matrix of size L×S. Introducing a double-stage precoder structure to K, similar to (2), we can express the delay-domain matrix K as

|  | K=F1⋅K | (7) |
| --- | --- | --- |

Considering (6) and (7), we find that the space-delay precoder has a similar structure to (5), whereas the transformation matrix is given by the entries of a non-uniform DFT-matrix. This non-uniform DFT-matrix may be represented by an oversampled DFT-matrix.

The proposals [4], [5] and [7] have in common that each column of the transformation matrix is selected from a frequency-domain codebook represented by a DFT-based matrix.

Proposal 1: Study DFT-codebook-based representations of F2 with either beam-dependent or beam-independent transformation matrices.

The feedback of the beam-dependent transformation includes L delays per beam and L delay-domain combining-coefficients associated with the L beam-specific delays. The L delays are associated with entries of a DFT-based codebook. Therefore, the total feedback amounts to 2UL delay-domain combining coefficients and 2UL-1 delays. The contribution in [5] argued that each beam is associated with a single mean delay, and the remaining L-1 delays are defined around each mean delay. Therefore, instead of  2UL delays, only 2U-1 delays need to be fed back to the transmitter.

The feedback of the beam-independent transformation includes L-1 delays and 2UL delay-domain combining coefficients associated with the L-1 delays. Similar to the beam-dependent transformation, the L-1 delays are associated with the entries of a DFT-based codebook.

In general, when optimizing the combining coefficients directly in the delay-domain, the associated delays may no longer be given by integer values. Therefore, they cannot be associated with entries of a DFT-matrix. Moreover, each beam is often associated with a specific direction and delay of a channel path component, and the delays of the channel path components may not be represented by integer values. Therefore, the flexibility of optimizing the delays can be increased when using an oversampled frequency-domain codebook. Let Of be the oversampling factor. Then, the oversampled frequency-domain codebook may be represented by

|  | D=d0,d1,…,dSOf-1, | (8) |
| --- | --- | --- |

where di= 1  e-j2πiOfS … e-j2πi(S-1)OfST∈ CS×1. The transformation matrices Wu are given by the entries of codebook D.

Observation 1: The delays associated with the delay-domain complex combining coefficients may not be given by integer values, and therefore they cannot be associated with entries of a DFT-matrix.

Proposal 2: Consider oversampled DFT-matrices for the frequency-domain codebook for further study in NR.

When optimizing the delays and combining coefficients directly in the delay domain, the first dimension of the frequency-domain codebook can be flexibly chosen. For example, the value of S can be set to the total number of PRBs. Unlike subband precoder, the precoder coefficients can vary per PRB and may not be constant over a subband. This leads to a higher flexibility in the optimization and possibly results in performance improvement.

Proposal 3: Different values for the first dimension of the frequency-domain codebook can be studied in NR.

# Simulation results

In this section, the performance and the amount of feedback provided by the space-delay (SD) precoder using the beam-independent transformation is presented. For comparison, the subband (SB) precoder using 6 subbands and the MRT precoder are considered. The simulations are performed using 50 PRBs and the frequency-domain DFT codebook with Of=1 (no oversampling) for simplicity.

The simulation parameters and simulation setup details are summarized in Table. 2. The performance measure used to evaluate the different precoders is given by

ηb%=AMIb-AMI1AMI1×100.

Here, ηb denotes the spectral efficiency improvement of b subbands/b delays with respect to a single subband/delay. Note that  F2 is identical and frequency-flat for the SB and SD precoders when using a single subband/delay. Therefore, AMI(1) refers to the average mutual information achieved using a wideband second stage precoder F2.

Fig. 1 shows the values of η for the SB precoder using S=6 subbands and the SD precoder using different number of delays L. It can be seen that when using L=2 delays, the SD precoder suffers only from a small performance loss compared to the SB precoder. Moreover, for L=3 delays, a sharp improvement in spectral efficiency and a similar performance than the SB precoder is obtained. Further gradual improvement is observed when increasing further the delay values.

Figure 1: Spectral efficiency improvement of subband, space-delay and MRT precoder with respect to a wideband precoder at an SNR of 10dB.

Table. 1 shows the spectral efficiency improvement and the amount of feedback in bits required for SB, SD and MRT precoders. The amplitude and phase of the precoder coefficients were quantized with three bits each. The delay values were quantized with 6 bits.

As seen from Table 1, for approximately equal values of η, a feedback overhead reduction of 41% is achieved for the SD precoder (L=3 delays) compared to the SB precoder. Similarly, for approximately identical feedback overhead, the SD precoder (L=5 delays) achieves a performance gain of 5% compared to the SB precoder (S=6 subbands).

Table 1: Spectral efficiency improvement and feedback overhead of the space-delay precoder, subband- and MR- precoder schemes.

| Precoder scheme | η[%] | Feedback overhead in bits | Feedback overhead reduction compared to subband precoder |
| --- | --- | --- | --- |
| Subband precoder (S=6) | 13.64% | 320 | - |
| Space-delay precoder (L=2) | 10.27% | 134 | 58.13% |
| Space-delay precoder (L=3) | 14.52% | 188 | 41.25% |
| Space-delay precoder (L=4) | 16.79% | 242 | 24.38% |
| Space-delay precoder (L=5) | 18.26% | 296 | 7.5% |
| MRT | 25.87% | 2432 | - |

Observation 2: For equal performance, a significant feedback overhead reduction is achieved by space-delay precoding over sub-band based precoding.

Observation 3: For equal feedback overhead, a performance gain is achieved by space-delay precoding over sub-band based precoding.

Proposal 4: Further study of performance gain and feedback reduction using delay-domain compression of F2 is required in NR.

# Conclusions

This contribution discusses a framework for codebook-based compression for the Type-II CSI feedback scheme and present preliminary simulations results with the following observations and proposals.

Proposal 1: Study DFT-codebook-based representations of F2 with either beam-dependent or beam-independent transformation matrices.

Observation 1: The delays associated with the delay-domain complex combining coefficients may not be given by integer values, and therefore they cannot be associated with entries of a DFT-matrix.

Proposal 2: Consider oversampled DFT-matrices for the frequency-domain codebook for further study in NR.

Proposal 3: Different values for the first dimension of the frequency-domain codebook can be studied in NR.

Observation 2: For equal performance, a significant feedback overhead reduction is achieved by space-delay precoding over sub-band based precoding.

Observation 3: For equal feedback overhead, a performance gain is achieved by space-delay precoding over sub-band based precoding.

Proposal 4: Further study of performance gain and feedback reduction using delay-domain compression of F2 is required in NR.

# References

“RAN1 Chairman’s Notes”, Spokane, USA, April 3-7, 2017.

“RAN1 Chairman’s Notes”, Hangzhou, China, May 15-19, 2017.

NTT DOCOMO, “On Type II codebook enhancement,” R1-1802473, Athens, Greece, Feb. 26 – March 2, 2018.

Huawei, HiSilicon, “Further enhancements on CSI reporting and codebook design,” R1-1802074, Athens, Greece, Feb. 26 – March 2, 2018.

Ericsson, “Frequency parameterization for Type II CSI feedback,” R1-1802748, Athens, Greece, Feb. 26 – March 2, 2018.

LG Electronics, “Discussion on overhead reduction for Type II codebook,” R1-1713144, Prague, Czech Republic, August 21 – 25, 2017.

Fraunhofer IIS, Fraunhofer HHI, “Space-delay versus sub-band precoding for mmWave channels,” R1-1800597, Vancouver, Canada, Jan. 22-26, 2018.

Table 2: Simulation parameters and setup

| Simulation parameters | Values |
| --- | --- |
| Channel setup | Single user MISO |
| Channel scenario | 3GPP Urban Macro NLOS |
| Carrier Frequency | 2.6GHz |
| System Bandwidth | 10MHz |
| Number of subcarriers (Q) | 1024 |
| Number of PRBs | 50 |
| Number of subbands (S) | 6 |
| Number of delays (L) | 1-5 |
| BS array antenna configuration (M,N,P) | M = 4; N= 4; P = 2 X-pol (+/-90) (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna | Isotropic antenna with vertical polarization |
| Cell radius | 250m |
| Base station height | 25m |
| UE speed | 3Km/h |