3GPP TSG RAN WG1 Meeting #93	R1-1807134

Busan, Korea, May 21 – May 25, 2018

Agenda Item:	7.1.2.3.8

Source:	Huawei, HiSilicon

Title:	Remaining issues for SRS switching among CCs

Document for:	Discussion and decision

# Introduction

In 5G NR, SRS switching among CCs (also commonly referred as SRS carrier-based switching) is supported, and details and RRC parameters for SRS switching among CCs were provided and standardized in TS 38.212, TS 38.213, TS 38.214, and TS 38.331.

In this contribution we clarify a few remaining issues with the current specifications and suggest text proposals for SRS switching among CCs. This contribution is updated from R1-1804448.

# Remaining issues

## Remaining issues on some collision handling aspects in 38.214

In Rel-14, it is supported that A/N, SR, RI/PTI/CRI, PRACH > Rel-14 A-SRS > other A-periodic CSI > Rel-14 P-SRS > other CSI > Legacy SRS. This can be largely reused, with further consideration of SP-SRS, SP CSI, new CSI reporting quantities (e.g., L1-RSRP, SSBRI), new PUCCH transmissions (e.g., PUCCH carrying beam failure recovery request), and other SRS related designs already introduced in NR Rel-15.

In the latest CR for TS 38.214, v15.1.0 (R1-1805796), it specifies in 6.2.1 that:

For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK.  In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only.

In other words, it specifies PUCCH0/2 with CSI/RSRP > SP/P SRS, PUCCH-A/N > A/SP/P SRS, and PUCCH-BFR > A SRS > SP/P PUCCH with SP/P CSI/RSRP, on the same serving cell configured with PUSCH/PUCCH/SRS. Though these rules are for scenarios different from SRS switching, their principles can be considered in SRS switching collision handling. For example, we can see that PUCCH carrying beam failure request should have higher priority than SRS switching. In addition, SP/P SRS are generally associated with the same priority, and SP/P CSI are generally associated with the same priority. These can be adopted for SRS switching.

Regarding new CSI reporting quantities, first we note that some of the new quantities, e.g., i1, LI, always accompany CRI/RI, and hence their priorities can be implied and do not need to be explicitly described in the specification. For SSBRI, it should have the same priority as CRI/RI and hence higher than SRS. Note that PUCCH-BFR detail is still being developed, and hence it is included only if it is specified in the standards and we put it in brackets in our proposal and text proposal.

Combining the above, the following is then proposed:

Proposal 1: For collision handling, support

- SSBRI, [PUCCH-BFR] > SRS

- A SRS > SP/P PUCCH with SP/P CSI/RSRP > SP/P SRS

- P CSI and SP CSI have the same priority for collision handling with SRS switching.

A TP for TS 38.214 is provided to capture the proposal.

## Remaining issue on a parameter in 38.213

In TS 38.213, 11.4, it describes the UE group common PDCCH for SRS switching. However, some of the descriptions are not fully aligned with TS 38.212, TS 38.214, or other agreements.

- The terms “block” and “field” have different meanings according to TS 38.212, and one block may have multiple fields. The description “an index for a location in DCI format 2_3 of a first bit for a field by higher layer parameter startingBitOfFormat2-3” should apply to a block rather than a field.

- For “an index of a serving cell where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by higher layer parameter srs-SwitchFromServCellIndex”, the index is needed only for SRS carrier switching but not for separate SRS/PUSCH power control. Also in 38.214, UL/SUL are described for the switching-from CC configuration, which should also be reflected here.

- The “higher layer parameter srs-CellToSFI” is not correct here. SFI is not relevant. And for typeB, the CC associated with the block can also be determined by higher layer parameter cc-SetIndex which has only one set for the UE.

- The terms “uplink carrier” and “serving cell” need to be correctly reflected.

A TP for TS 38.213 is provided.

Proposal 2: Remove the parameter srs-CellToSFI from 38.213.

# Conclusions

Based the discussions above, we have the following proposals:

Proposal 1: For collision handling, support

- SSBRI, [PUCCH-BFR] > SRS

- A SRS > SP/P PUCCH with SP/P CSI/RSRP > SP/P SRS

- P CSI and SP CSI have the same priority for collision handling with SRS switching.

Proposal 2: Remove the parameter srs-CellToSFI from 38.213.

Text proposals for the specification are at the end of this contribution.

# References

- Chairman’s notes, RAN1#90bis, October 2017.

- R1-1613787, ‘List of agreements for “SRS Carrier-Based Switching”’, Huawei, RAN1#87, Reno, USA, Nov. 2016.

# Text proposals:

38.214

------------------------------------------ Start of Text Proposal ----------------------------------------------

---------------------------------------- Unchanged parts omitted --------------------------------------------

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH [and/or PUCCH formats 0 and 2 consisting of beam failure request] happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI or PUCCH formats 0 and 2 carrying only CSI report(s) or only L1-RSRP report(s) happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying semi-persistent/periodic CSI comprising only CQI/PMI, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI and semi-persistent or periodic PUCCH transmission carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the SRS_capability [13, TS 38.306].

----------------------- End of Text Proposal --------------------------

38.213

------------------------------------------ Start of Text Proposal ----------------------------------------------

---------------------------------------- Unchanged parts omitted --------------------------------------------

## 11.4	SRS switching

DCI format 2_3 is applicable for serving cells where a UE is not configured for PUSCH/PUCCH transmission or for a serving cell where higher layer parameter srs-pcadjustment-state-config indicates a separate power control adjustment state between SRS transmissions and PUSCH transmissions.

A UE configured by higher layers with parameter SRS-CarrierSwitching is provided with

-	a TPC-SRS-RNTI for a DCI format 2_3 provided by higher layer parameter srs-TPC-RNTI

-	an index of a serving cell where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by higher layer parameter srs-SwitchFromServCellIndex in the case that the serving cell is configured with one uplink carrier. This index is present only if the DCI format 2_3 is for a SRS transmission on a carrier not configured for PUSCH/PUCCH transmission.

-	an index of a serving cell and an index of an uplink carrier of the serving cell where the UE interrupts transmission in order to transmit SRS on one or more other serving cells by higher layer parameters srs-SwitchFromServCellIndex and srs-SwitchFromCarrier in the case that the serving cell is configured with two uplink carriers. These indexes are present only if the DCI format 2_3 is for a SRS transmission on a carrier not configured for PUSCH/PUCCH transmission.

-	a DCI format 2_3 block configuration type by higher layer parameter typeA or higher layer parameter typeB

  - for higher layer parameter typeA, an index for a set of uplink carriers of serving cells is provided by higher layer parameter cc-SetIndex, indexes of the uplink carriers of the serving cells in the set of uplink carriers of serving cells are provided by higher layer parameter cc-IndexInOneCC-Set, and one DCI format 2_3 block includes a TPC command for each uplink carrier of serving cells from the set of uplink carriers of the serving cells, and can also include one SRS request for SRS transmissions on all the uplink carriers of serving cells in the set of uplink carriers of serving cells

  - for higher layer parameter typeB, a DCI format 2_3 block includes a TPC command for an uplink carrier of a serving cell and can also include a SRS request for SRS transmission on the uplink of the serving cell - a mapping between the uplink of the serving cell for SRS transmission and a DCI format 2_3 block is provided by higher layer parameter startingBitOfFormat2-3

-	an indication for a set of uplink carriers of serving cells (typeA) or an uplink carrier of a serving cell (typeB) for whether or not a block in DCI format 2_3 includes a SRS request field by higher layer parameter fieldTypeFormat2-3 where a value of 0/1 indicates absence/presence of the SRS request – a mapping for a 2 bit SRS request to SRS resource sets is as provided in [6, TS 38.214]

-	an index for a location in DCI format 2_3 of a first bit for a block by higher layer parameter startingBitOfFormat2-3

----------------------- End of Text Proposal --------------------------