3GPP TSG RAN WG1 Meeting #93							R1-1807150

Busan, Korea, 21th May – 25th May, 2018

Source:	NTT DOCOMO, INC.

Title:	Remaining details on PTRS

Agenda Item:	7.1.2.3.4

Document for: 	Discussion and Decision

- Introduction

In this contribution, remaining details on PT-RS are discussed.

- UL PT-RS power boosting for non-codebook UL transmission

At the last RAN1 meeting, how to determine the power boosting factor for non-codebook based UL transmission was discussed. For PT-RS power boosting determination, we believe straightforward approach is reusing the table for PT-RS power boosting for codebook based UL transmission. More discussion is needed how to assume the pre-coder type, i.e. full coherent, partial coherent, or non-coherent, to determine PT-RS power boosting for non-codebook UL transmission.

Proposal 1:

- For non-codebook based UL transmission, re-use the same table for PT-RS power boosting for codebook based UL transmission.

- Time domain PT-RS density for UCI-only on PUSCH

In RAN1 #92, it was agreed that modulation order and code rate are signalled by DCI for UCI-only on PUSCH.

Agreements (RAN1 #92): For UCI-only multiplexed on PUSCH without UL-SCH - Modulation order and code rate are signalled in DCI. - Resource determination following the same principle as UCI multiplexing on PUSCH with UL-SCH. - FFS: A-CSI only without UL-SCH on PUSCH is triggered explicitly based on adding one bit in DCI or triggered implicitly based on a special combination of certain existing fields in DCI. - FFS: how modulation and code rate are signalled.

In addition to above agreements, the following working assumption was made in the last meeting [1].

Working assumption (RAN1 #92b): Support variable time density LPT-RS for UCI-only PUSCH where information (e.g. MCS) in DCI is used to determine LPT-RS.

In clause 6.2.3 in TS38.214, PT-RS density is determined by MCS. How to determine MCS for UCI-only on PUSCH is now discussed in control session. If the method for determination of MCS is concluded, PT-RS density for UCI-only on PUSCH is determined by same method as described at clause 6.2.3 in TS38.214.

So, no spec. change is required to determine time domain PT-RS density for UCI-only on PUSCH.

Proposal 2:

- No spec. change is required to determine time domain PT-RS density for UCI-only on PUSCH.

  - Note: How to determine MCS for UCI-only on PUSCH will be concluded in control session.

- DL power boosting

According to Table 4.1-2 of TS38.214, there is a case that the power boosting of PT-RS is larger than 6dB. Whether the maximum power boosting of PT-RS should be limited to 6dB was discussed in the last meeting. Similar discussion was made for the maximum power boosting of CSI-RS at LTE-Advanced Rel. 10, and RAN1 sent LS to RAN4 whether it is feasible [2]. RAN4 replied LS [3] and they said that the maximum power boosting of CSI-RS should be limited 6dB considering EVM. Hence, we believe that the maximum power boosting of PT-RS should be also limited to 6dB.

Proposal 3:

- The maximum power boosting of DL PT-RS should be limited to 6dB.

- Signaling of BW threshold for PT-RS frequency density

According to clause 6.3.2 in TS38.331, there are RRC parameters of which value range is {1…276} to signal BW threshold for PT-RS frequency density, i.e. for CP-OFDM, frequencyDensity has 2 values of which value range is {1…276}; and for DFT-S-OFDM, sampleDensity has 5 values of which value range is {1…276}.

6.3.2 Radio resource control information elements:[…] PTRS-DownlinkConfig ::= SEQUENCE { frequencyDensity SEQUENCE (SIZE (2)) OF INTEGER (1..276) OPTIONAL, -- Need S […] PTRS-UplinkConfig ::= SEQUENCE { […] cp-OFDM SEQUENCE { frequencyDensity SEQUENCE (SIZE (2)) OF INTEGER (1..276) OPTIONAL, -- Need S […] dft-S-OFDM SEQUENCE { sampleDensity SEQUENCE (SIZE (5)) OF INTEGER (1..276), […]

In RAN2 #101, RAN2 declared TS38.331 is frozen [4]. So, it is not feasible to reduce the size of RRC parameters at this stage. Additionally, from the performance point of view, RRC overhead reduction for BW threshold signaling for PT-RS frequency density would not have large performance improvement. Hence, we believe further discussion is not necessary how to reduce signaling of BW thresholds.

Proposal 4:

- No need to discuss RRC overhead reduction for BW threshold signalling for PT-RS frequency density.

- Conclusion

In this contribution, we discussed remaining details on PT-RS, and following proposals were made:

Proposal 1:

- For non-codebook based UL transmission, re-use the same table for PT-RS power boosting for codebook based UL transmission.

Proposal 2:

- No spec. change is required to determine time domain PT-RS density for UCI-only on PUSCH.

  - Note: How to determine MCS for UCI-only on PUSCH will be concluded in control session.

Proposal 3:

- The maximum power boosting of DL PT-RS should be limited to 6dB.

Proposal 4:

- No need to discuss RRC overhead reduction for BW threshold signalling for PT-RS frequency density.

References

- 3GPP RAN1#92bis, Chairman’s note.

- R4-101853, “LS on CSI RS boosting impact on BS transmission distortion”, TSG-RAN WG1

- R1-103331, “Reply LS on CSI RS boosting impact on BS transmission distortion,”

- R2-1804201, “Report of 3GPP TSG RAN2#101 meeting,” Athens, Greece.