3GPP TSG RAN WG1 Meeting #93	R1-1807245

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	6.2.6.6

Source: 	Sony

Title:	Remaining issues in explicit uplink HARQ-ACK feedback

Document for:	Discussion / decision

# Introduction

In RAN1#92bis we agreed the following:

The working assumption from RAN1#91 is replaced by the following agreement

- One unused DCI state in each CE mode is used for indicating:

  - Early termination of any ongoing PUSCH transmission (without early termination of MPDCCH monitoring except for the case of acknowledging RRC Connection Release, for which MPDCCH monitoring is terminated)

For the UE-specific UL HARQ-ACK feedback, ACK is indicated by DCI format 6-0A and 6-0B for CE mode A and CE mode B, respectively, by setting all fields except the ones below to default fixed value

  - ‘Flag for format 6-0A/6-1A differentiation’ in CE mode A and ‘Flag for format 6-0B/6-1B differentiation’ in CE mode B

  - ‘DCI subframe repetition number’ field

  - ‘Resource block assignment’ field in DCI format 6-0A and ‘MCS’ field in DCI format 6-0B

    - Values; TBD

Note Exact default fixed value is up to specification editor

This contribution discusses remaining issues, i.e. Group Common DCI and the MPDCCH search space for explicit UL HARQ-ACK feedback.

# Discussions

#### 2.1 Group Common DCI

One of the remaining issues for explicit HARQ-ACK is whether to introduce a Group Common DCI to provide HARQ-ACK feedbacks to multiple UEs.  The drawbacks in using this scheme are:

- The eNB would need to group UEs in the same coverage level so that their feedback can be transmitted in the same DCI since it is not efficient to multiplex UEs in good coverage with those in poor coverage in a single DCI.

- It is unclear how this can be used for early termination of PUSCH repetition.  It is challenging for the scheduler to ensure that all PUSCH end at the same time including those with early PUSCH termination.

- Multiplexing of multiple UEs would lead to a larger than required DCI size leading to higher repetitions for each individual UE.

In [1] it is proposed that each HARQ-ACK feedback field in a GC-DCI acknowledges a specific PUSCH resource and hence the eNB does not need to perform prior UE grouping into a GC-DCI.  However, there can be a lot of PUSCH resources that can be used in a system bandwidth which may either mean that the GC-DCI would need a lot of fields, one for each PRB in the system bandwidth, or there need to be some restrictions imposed on which PUSCH resources can be allocated.  Furthermore this does not address issues 2 and 3 above.

Observation 1: Multiplexing UL HARQ-ACK feedbacks for multiple UEs in a single common DCI is not efficient and may increase scheduler complexity.

Proposal 1: The explicit UL HARQ-ACK feedback is carried only by a UE specific DCI.

#### 2.2 MPDCCH Search Space for Early Repetition Termination (ERT)

In [2], we noted that the explicit UL HARQ-ACK feedback for ERT can be transmitted using the current USS including the remaining candidates in the search space that schedules the PUSCH.  The ERT can be transmitted using MPDCCH candidates of the existing USS that overlap the PUSCH transmission.  For example in Figure 1, we have an MPDCCH SS with periodicity of PMPDCCH and offset OMPDCCH.  At time t1, the UE receives an uplink grant from an MPDCCH candidate with repetition R1 shown highlighted in blue, which schedules a PUSCH transmission of RPUSCH repetitions that starts at time t2.  It can be observed that there are MPDCCH candidates that overlap this PUSCH transmission between time t3 and t8 highlighted in beige.  These candidates, which consist of (1) the remaining MPDCCH in the search space that schedules the uplink grant (i.e. unblocked candidates) and (2) those in the subsequent MPDCCH search space, can act as a search space for the transmission of ERT.

Figure 1: Using overlapping candidates for transmission of ERT

Observation 2: The MDPDCCH candidates that overlap the PUSCH transmission can be monitored for Early PUSCH Repetition Termination indicator.

Referring to Figure 1 again, it can be observed that there isn’t much point monitoring for an ERT if it is carried by the MPDCCH candidate starting at time t7 with repetition R1, since by the time the ERT is received at the UE, the PUSCH transmission would have ended.  Similarly some candidates that occur at the beginning of the PUSCH transmission are also not suitable since the PUSCH is not expected to be received successfully at this early stage.  Hence, we can exclude these candidates, for example in Figure 2 we do not include all candidates that overlap the PUSCH transmission but exclude those that are within TSTART from the start of the PUSCH transmission and those that fall within TEND from the end of the PUSCH transmission, i.e. the candidates between time t4 and t7 in Figure 2 are candidates for ERT.  These candidates can form a search space for ERT monitoring.

Figure 2: MPDCCH candidates for transmission of ERT

Observation 3: MPDCCH candidates overlapping the PUSCH transmission that are located too early in the transmission or too late in the transmission are not suitable to carry ERT.

Proposal 2: The MPDCCH candidates overlapping the PUSCH transmission, excluding those within TSTART from the beginning of the PUSCH transmission and TEND from the end of the PUSCH transmission, are used for monitoring of an ERT indicator.

In [3], it is noted that the periodicity of the MPDCCH UE Specific Search Space (USS) may not provide opportunities to signal an early PUSCH repetition termination (ERT).  An example is shown in Figure 3 where the periodicity of the MPDCCH USS is PMPDCCH.  Here the scheduled PUSCH transmission between t1 and t2 does not overlap any MPDCCH USS and since the ERT indicator is carried by MPDCCH, there is no opportunity for the eNB to transmit an ERT indicator for this PUSCH transmission.

Figure 3: MPDCCH periodicity prevents ERT indicator transmission

If there is no MPDCCH candidates that overlaps with the scheduled PUSCH transmission, then it can be deemed that the eNB does not require the UE to monitor for ERT.  Specifically, if there is no MPDCCH candidates overlapping the time window between TSTART from beginning of PUSCH transmission and TEND from end of the PUSCH transmission, then the UE does not need to monitor for ERT during this PUSCH transmission.

Proposal 3: If there is no MPDCCH candidate overlapping the time window between TSTART from beginning of PUSCH transmission and TEND from end of the PUSCH transmission, then the UE does not need to monitor for ERT during this PUSCH transmission.

# Conclusion

In this contribution we discuss the remaining aspects in introducing explicit HARQ_ACK.  We observed the following:

Observation 1: Multiplexing UL HARQ-ACK feedbacks for multiple UEs in a single common DCI is not efficient and may increase scheduler complexity.

Observation 2: The MDPDCCH candidates that overlap the PUSCH transmission can be monitored for Early PUSCH Repetition Termination indicator.

Observation 3: MPDCCH candidates overlapping the PUSCH transmission that are located too early in the transmission or too late in the transmission are not suitable to carry ERT.

We therefore propose the following:

Proposal 1: The explicit UL HARQ-ACK feedback is carried only by a UE specific DCI.

Proposal 2: The MPDCCH candidates overlapping the PUSCH transmission, excluding those within TSTART from the beginning of the PUSCH transmission and TEND from the end of the PUSCH transmission, are used for monitoring of an ERT indicator.

Proposal 3: If there is no MPDCCH candidate overlapping the time window between TSTART from beginning of PUSCH transmission and TEND from end of the PUSCH transmission, then the UE does not need to monitor for ERT during this PUSCH transmission.

# References

[1] R1-1803886, “Further considerations on HARQ-ACK feedback for PUSCH in eFeMTC,” Huawei, HiSilicon, RAN1#92bis

[2] R1-1804605, “Remaining issues in explicit uplink HARQ-ACK feedback for efeMTC	,” Sony, RAN1#92bis

[3] R1-1804175, “Remaining issues on UL HARQ-ACK feedback for MTC,” ZTE, SaneChips, RAN1#92bis