3GPP TSG RAN1#93	R1-1807680

# Busan, South Korea

# 21-25 May, 2018

# Agenda item:     	7.8.2
Source: 	Motorola-Mobility, Lenovo

# Title:	Initial evaluation results of the Dense Urban-eMBB scenario for IMT-2020 evaluation

# Document for: 	Discussion

# Introduction

The preliminary work plan of ITU self-evaluation was agreed in the RAN#77 meeting [1].  Offline discussion in the RAN1#92 meeting, resulted in a set of evaluation assumptions for self-evaluation [2].  This contribution provides initial Dense Urban-eMBB simulation results for the ITU IMT-2020 self-evaluation.

# Results

# This section provides preliminary simulation results for the FDD DL Dense Urban-eMBB Configuration A scenario.  The number of Tx/Rx antenna ports is 32 and 4 respectively.  Type II CSI feedback was simulated with a periodicity and delay of 5 ms.  Several simulation assumptions limit the spectral efficiencies obtained. These include a maximum modulation order of 64 QAM instead of 256 QAM and a maximum number of co-scheduled UEs of 4.  Simulations without these restrictive assumptions will be performed in the future and the results reported in a subsequent contribution.  Additional simulation assumptions are given in the Appendix.

The average cell spectral efficiency and the 5th  percentile user spectral efficiency are reported in Table 1. Also reported are the corresponding spectral efficiencies required for ITU submission as given in [3].

Table 1. Dense Urban – eMBB  Configuration A DL Spectral Efficiency

| Average Cell SE (bits/s/Hz/TRP) |  | 5th Percentile User SE (bits/s/Hz) |  |
| --- | --- | --- | --- |
| Simulated | Required | Simulated | Required |
| 8.9 | 7.8 | 0.23 | 0.225 |

# Looking at Table 1, the following observation can be made:

# Observation: The ITU requirements on average spectral efficiency and cell edge spectra efficiency can be met with 32 transmit antenna ports and 4 receive antenna ports.

# Conclusions

This contribution provided preliminary downlink simulation results of Dense Urban-eMBB Configuration A, including the DL average spectral efficiency and the DL 5th percentile user spectral efficiency. From the results the following observation can be made:

# Observation: The ITU requirements on average spectral efficiency and cell edge spectra efficiency can be met with 32 transmit antenna ports and 4 receive antenna ports.

# References

- R1-1803386, “Summary of offline discussion for IMT-2020 self-evaluation”, Huawei, February 2018.

- [2] R1-1803386, “Summary of offline discussion for IMT-2020 self-evaluation”, Huawei, February 2018.

- “Minimum requirements related to technical performance for IMT-2020 radio interface(s),” Report ITU-R M.2410‑0, November 2017.

Appendix

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Environments |  | Dense Urban – eMBB, Configuration A (4GHz), Channel model A |  |
| System bandwidth |  | 10MHz |  |
| Subcarrier spacing |  | 15kHz |  |
| Duplex mode |  | FDD |  |
| Traffic model |  | Full buffer |  |
| gNB transmit power |  | 41 dBm |  |
| gNB antenna element configuration |  | (M,N,P,Mg,Ng) = (8,16,2,1,1), (dV,dH) = ( 0.5, 0.5 ) λ, 90° mechanical tilt |  |
| gNB antenna port configuration |  | (Mp,Np,P,Mg,Ng) = (1,16,2,1,1), Electrical tilt = 108° |  |
| UE Rx antenna config. |  | (M,N,P) = (1,2,2) dH = 0.5 λ 0°, 90° polarization |  |
| UE antenna pattern |  | Omni |  |
| Receiver |  | MMSE-IRC |  |
| Channel modeling |  | Ideal |  |
| Interference modeling |  | Ideal |  |
| DL CSI feedback |  | CQI, PMI, RI reporting every 5ms with a delay of 5ms. Type II codebook, 4 beams, 3 phase bits, sub-band amplitudes |  |
| Modulation |  | Up to 64 QAM |  |
| DL scheduler |  | SU/MU-PF, frequency-selective, maximum number of grouped users is 4 |  |
| DL Overhead | SS/PBCH block | 480 REs per 10 ms |  |
|  | DMRS (Type II) | 24 REs per PRB per subframe |  |
|  | CSI-RS | 32-port CSI-RS per 10 ms |  |
|  | TRS | 8 REs per PRB per 20 ms |  |
|  | CSI-IM | 4 REs per PRB per 10 ms |  |
|  | PDCCH | 8 CCEs |  |