3GPP TSG RAN WG1 Meeting #92bis	R1-1805901

Busan, Korea, May 21st – 25th, 2018

Agenda Item:	7.1.3.1.5

Source:	Huawei, HiSilicon

Title:	Summary of remaining issues on PDCCH monitoring with TP

Document for:	Discussion and decision

# Introduction

At RAN1 # 92bis, the following agreements were reached regarding the monitoring of the DL control channel.

Agreements:

- UE is not expected to be configured with PDCCH monitoring in CSS(s) for more than what UE can monitor in terms of numbers of BDs/CCEs

Working assumption:

- At least for Case 1-1 and Case 1-2, map all candidates of USS  search-space-set with lower SS set ID before candidates of USS with higher ID

  - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped)

- Case 2 FFS

In this contribution, we discuss the PDCCH candidate mapping rules when either or both limit(s) of (number of blind decodes, CCEs for channel estimation) is/are met for Case 2.

# Discussion

## 2.1 PDCCH monitoring occasion configuration

In NR, there are at most 10 search-space-sets could be configured to one UE，and for each search-space-set, the UE determines a PDCCH monitoring occasion(s) exists in a slot from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern. Also it has been agreed for case 2 that the minimum time separation between two unicast DCIs for a same UE could be 2 OS for 15 kHz, 4OS for 30 kHz, and 7OS for 60 kHz with NCP. That is to say, if the UE is configured with one USS whose PDCCH monitoring occasion is per 2 OS in a slot, then there is 7 occasions in one slot could be used to schedule URLLC, which is enough to guarantee the latency, so there is no need to configured with another USS whose PDCCH monitoring occasion is also per 2 OS in a slot but the starting symbol is different from the first one. In other words, from URLLC perspective, the time separation between two monitoring occasions could be at least 2 OS for 15 kHz, 4OS for 30 kHz, 7OS for 60 kHz, 14 OS for 120 kHz. For 15 kHz, as an example, when there are one or two USS(s), the configurations of monitoring occasion could be seen in Figure 1 (a) and (b) respectively, and the two USSs could be linked to the same or different CORESETs.

Figure 1 Configuration of monitoring occasions for 15 kHz

Now there is an association between the search space set and a control resource set, and the duration of control resource set could be 1 or 2 or 3 symbols, then UE should monitor PDCCH in a monitoring windows defined by the configuration of the USS set and the duration of associated CORESET. So even if  the time separation between two monitoring occasion is at least 2 OS for 15 kHz, there are some cases the UE should monitor PDCCH in overlapped monitoring windows, which could be illustrated in Figure 2. Because the PDCCH is mapped in time domain firstly, so there should be no time-domain overlap between different monitoring windows with different occasions.

Figure 2 Illustration of overlapped monitoring windows

Proposal 1: UE is not expected to be configured with partially overlapped monitoring windows with different monitoring occasions.

- A monitoring window is defined by a monitoring occasion of the USS set and the duration of associated CORESET.

## 2.2 PDCCH candidates mapping rule for case 2

It has been agreed that a UE can be configured with a set of ALs and a number of PDCCH candidates for each AL, but there may be some cases when the number of required PDCCH BDs in a slot would exceed the maximum number of allowed PDCCH BDs. Then, not all of the configured candidates can be mapped. In RAN1#92, it has been agreed that the CSS has priority over USS, and in last meeting, it is agreed that UE is not expected to be configured with PDCCH monitoring in CSS(s) for more than what UE can monitor in terms of numbers of BDs/CCEs, and for case 1-1 and case 1-2, map all candidates of USS  search-space-set with lower SS set ID before candidates of USS with higher ID, if all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped).

For case 1-1 and case 1-2, PDCCH monitoring periodicity is 14 or more symbols, so the above mapping rule is enough. But for case 2, there are more than one PDCCH occasions in one slot and more than one USSs may distribute in these occasions, which could be shown in Figure 1. There are three USSs in one slot，and total 6 occasions should be monitored by the UE in Figure 3. If the symbols occupied by the occasions of different SS sets are same, they could be seen as one occasion. Assuming using the same rule as case 1-1 and case 1-2 that if all candidates in a USS can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped, sometimes all candidates in a USS set 3 maybe be dropped, which will limit the scheduling chance of URLLC UEs and influence the latency of these UEs. So instead of mapping all candidates of USS with lower SS set ID before candidates of USS with higher ID, the remaining BDs should be distributed in all occasions uniformly, and then within one monitoring occasion mapping all candidates of USS with lower SS set ID before candidates of USS with higher ID. This mapping rule could guarantee a chance for the UE to be scheduled in each configured monitoring occasion.

Figure 3: USS search-space-sets distribution in one slot for Case 2

One example is given in table 1, the SCS is 15 kHz, the max number of PDCCH BDs per slot is then 44, the total number of PDCCH BDs for CSS is 10. And 36 BDs would be needed together for the three USSs(USS1:(2+4)*4=24; USS2:(2+4)*2=12; USS3:(2+4)*2=12). In all 3 configured USSs the UE is supposed to monitor two DCI formats, and the monitoring occasions of these 3 USSs is shown in Figure 1.

Table 1 The assumptions of one example

| Content | Assumption |
| --- | --- |
| SCS | 15kHz |
| the max number of PDCCH BDs per slot | 44 |
| SS type and number | CSS:1 USS:3(USS 1,USS 2,USS 3) |
| DCI format to be monitored in one USS | DCI format 1 and DCI format 2 |
| (AL, candidates) configuration for each DCI format | DCI format 1: (16,2) total BD =2 DCI format 2: (4,2),(2,2), total BD =4 |

The distribution of the max number of available PDCCH BDs can be seen in Figure 4. The UE should monitor CSSs firstly and spends 10 BDs for that, then the remaining 34 BDs are used for monitoring the USSs. Ideally, 36 BDs would be needed to monitor all configured candidates. But since only 34 BD are available, the UE should divide these 34 BDs into 6 occasions, for example, each occasion in the first 5 occasions has 34/6=6 BDs, and the 6th occasion has 34-30=4 BDs.

Then in each occasion, UE could monitor all candidates of USS with lower SS set ID before candidates of USS with higher ID, if all candidates in a SS can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped). Following this rule, for occasion #3, #5, the UE monitor USS 1 only, which spends 6 BDs. For occasion #2, #4, the UE monitor USS 3 and USS 2 respectively. For occasion #1, the UE should monitor USS 1 firstly and all candidates in USS 2 are dropped.  There are only 4 BDs remaining for occasion #6, in total, 6 BDs would be required for USS1, therefore all candidates in USS1 and USS2 would be dropped.

Figure 4: Illustration of the divide of the max number of PDCCH BDs

The same considerations should be taken into account to define rules for adapting the PDCCH mapping when the number of CCEs for channel estimation is limited. According to the above analysis, we give the following proposal and the corresponding text proposal is provided in Appendix 1.

Proposal 2: For Case 2, PDCCH candidates are mapped to UE-specific search-space-sets until either or both limit(s) of (number of blind decodes, CCEs for channel estimation) is/are met at least with the following rule,

- SS type order, CSS  before USS

- For USS, the reaming number of blind decodes or CCEs for channel estimation should be divided into each occasion averagely

- Within one occasion, map all candidates of USS  search-space-set with lower SS set ID before candidates of USS with higher ID

  - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped)

# Conclusion

In this contribution, we provide our view on PDCCH monitoring aspects. We describe how to restrict the PDCCH mapping in case of a limited number of available BDs and/or CCEs for channel estimation. We make the following proposals.

Proposal 1: UE is not expected to be configured with partially overlapped monitoring windows with different monitoring occasions.

- A monitoring window is defined by a monitoring occasion of the USS set and the duration of associated CORESET.

Proposal 2: For Case 2, PDCCH candidates are mapped to UE-specific search-space-sets until either or both limit(s) of (number of blind decodes, CCEs for channel estimation) is/are met at least with the following rule,

- SS type order, CSS  before USS

- For USS, the reaming number of blind decodes or CCEs for channel estimation should be divided into each occasion averagely

- Within one occasion, map all candidates of USS  search-space-set with lower SS set ID before candidates of USS with higher ID

  - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped)

# References

- R1-1701664, “DL control and data channel reliability considerations for URLLC”, Huawei, HiSilicon

# Appendix 1

---------------------------------------------Start of text proposal--------------------------------------------------------

## 10.1	UE procedure for determining physical downlink control channel assignment

…

A UE determines a PDCCH monitoring occasion from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set  in control resource set , the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number  [4, TS 38.211] in a frame with number  if nfNslotframe,μ+ns,fμ-op,smod kp,s=0.

If the symbols occupied by the occasions of different SS sets are same, they could be seen as one occasion, and the total number of occasion is

…

If the higher layer parameter monitoringSlotPeriodicityAndOffset indicates to a UE more than one PDCCH monitoring occasion within a slot for a respective search space set, the UE allocates monitored PDCCH candidates to UE-specific search space sets in slot  according to the following pseudocode. A UE does not monitor PDCCH in a UE-specific search space set without monitored PDCCH candidates.

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the monitored PDCCH candidates for the common search space sets and the monitored PDCCH candidates for all search space sets , .

For occasion 1 to  (N-1), Set MPDCCHUSS=MPDCCHmax,slot-MPDCCHcss/ N

For occasion N , Set MPDCCHUSS=MPDCCHmax,slot-MPDCCHcss-MPDCCHmax,slot-MPDCCHcss*(N-1)

Set

while AND

allocate  monitored PDCCH candidates to UE-specific search space set

;

;

;

end while

--------------------------------------------- End of text proposal--------------------------------------------------------