3GPP TSG RAN WG1 Meeting #93				                    		       R1-1806287

Busan, Korea, May 21st –25th, 2018

Source:		CATT

Title:			Remaining issues on QCL

Agenda Item:		7.1.2.3.7

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss some of the remaining issues and propose corresponding corrections regarding QCL.

TCI State structure

In RAN1 #91 meeting[1], the following agreement on QCL type, linkage as well as TCI State structure were reached.

Agreement: QCL types A: Doppler shift, Doppler spread, average delay, delay spread QCL types B: Doppler shift, Doppler spread QCL types C: average delay, Doppler shift QCL types D: Spatial Rx QCL linkage for below 6GHz before RRC; signalling. SSB DMRS for PDSCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz). SSB DMRS for PDCCH w.r.t Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters (spatial RX parameters are used only for above 6GHz). QCL linkage for below 6GHz after RRC; signalling. SSB TRS: Doppler shift, average delay; QCL type: C. TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. TRS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. TRS CSI-RS for CSI acquisition: Doppler shift, Doppler spread; QCL type: B. CSI-RS DMRS: Doppler shift, Doppler spread, average delay, delay spread; QCL type: A. QCL linkage for above 6GHz after RRC; signalling. SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters; QCL type: C + D. TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters; QCL type: C+D. SSB CSI-RS for CSI w.r.t, spatial RX parameters; QCL type: D. SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters; QCL type: D. CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters; QCL type: D. CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI; QCL type: A+D. CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters; QCL type: D. - If the QCL target RS is periodic CSI-RS or TRS, - QCL-Info IE is placed in CSI-RS-Resource-Config and refers to the TCI-State index - TCI-State structure for one RS set is either: - DL Reference RS1¦ QCL_type1, DL Reference RS2¦ QCL_type2 - Note that there is no overlap of QCL_type1 and QCL_type2 - DL Reference RS1¦ QCL_type1 - Note that QCL_type1 and QCL_type2 are selected from {QCL types A, QCL types B, QCL types C, QCL types D} - Note that if DL Reference RS1 and DL Reference RS2 are the same, both QCL types apply

According to the above agreement, if two reference RSs are given, there should have no overlap of QCL_type1 and QCL_type2. However, the above agreement is not fully captured in current 214.

TS 38.214

5.1.5 Antenna ports quasi co-location

…

The UE can be configured with up to M TCI-States by higher layer signalling to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one RS set TCI-RS-SetConfig. Each TCI-RS-SetConfig contains parameters for configuring quasi co-location relationship between the reference signals in the RS set and the DM-RS port group of the PDSCH. The RS set contains a reference to either one or two DL RSs and an associated quasi co-location type (QCL-Type) for each one configured by the higher layer parameter QCL-Type. For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types indicated to the UE are based on the higher layer parameter QCL-Type and may take one or a combination of the following types:

…

According to the above agreement, if two reference RSs are given, there should have no overlap of QCL_type1 and QCL_type2. This implies that the only possible combination of QCL_types is type A+D and C+D, as all the remaining combinations of two QCL_types are overlapped. It’s noted that, based on agreed QCL linkages, a reference signal cannot be indicated/configured with type C and D QCL sources from two different reference signals separately. Therefore, for the case where two reference RSs are configured as QCL sources, the only valid combination of QCL types are A+D.

Based on the agreement in RAN1 #91 and discussion above, possible combinations can only be:

- CSI-RS for BM as the target RS

| TRS  CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| --- | --- |
| CSI-RS for BM  CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters | QCL type: D |

| SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters | QCL type: C+D |
| --- | --- |

- CSI-RS for CSI as the target RS

| TRS  CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| --- | --- |
| SSB  CSI-RS for CSI w.r.t, spatial RX parameters | QCL type: D |
| CSI-RS for BM  CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters | QCL type: D |

- TRS as the target RS

| SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters | QCL type: C + D |
| --- | --- |

- DMRS for PDSCH as the target RS

| TRS  DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| --- | --- |
| CSI-RS for BM  DMRS for PDSCH w.r.t., spatial RX parameters | QCL type: D |

| SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| --- | --- |

| CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI | QCL type: A+D |
| --- | --- |

- DMRS for PDCCH as the target RS

| TRS  DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| --- | --- |
| CSI-RS for BM  DMRS for PDCCH w.r.t., spatial RX parameters | QCL type: D |

| SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| --- | --- |

Therefore, we have the following proposals:

Proposal 1: To capture the agreement in RAN1 #91 more precisely, modify the description of relationship between the two reference RSs in section 5.1.5 of 214.

Number of supported QCL groups

The following agreements on QCL were reached in RAN1#90[2].

Agreements:

- Support the QCL indication of DM-RS for PDSCH via DCI signaling:

  - The N-bit indicator field in the agreed WF R1-1714885 is extended to support:

    - Each state refers to one or two RS sets, which indicates a QCL relationship for one or two DMRS port group (s), respectively

      - Each RS set refers to one or more RS(s) which are QCLed with DM-RS ports within corresponding DM-RS group

      - Note: The RSs within a RS set may be of different types

      - If there are more than one RS per RS set, each of them may be associated with different QCL parameters, e.g. one RS may be associated with spatial QCL while another RS may be associated with other QCL parameters, etc

      - Configuration of RS set for each state can be done via higher layer signaling

        - E.g., RRC/RRC + MAC CE

  - FFS the timing when the QCL is applied relative to the time of the QCL indication

However, the agreement that each TCI state refers to one or two RS sets is not captured in the current version of TS 38.214[3]. As stated in section 5.1.5 of 214 that each configured TCI state includes only one RS set TCI-RS-SetConfig. To address this issue, and support the case with two DMRS port groups, we propose to modify this part with the text shown in conclusion.

Proposal 2: To capture the agreement on supporting up to 2 DMRS groups, modify the description in section 5.15 of 214.

QCL linkage between SSB and CSI-RS for CSI

In section 5.1.5 of 214, the following statement is presented.

TS 38.214

5.1.5 Antenna ports quasi co-location

…

-	If a CSI-RS resource is in a CSI-RS resource set configured without higher layer parameter TRS-Info and without CSI-RS-ResourceRep, the UE should only expect ‘QCL-TypeA’ or ‘QCL-TypeB’ configuration with a CSI-RS resource in a CSI-RS resource set configured with higher layer parameter TRS-Info or ‘QCL-TypeD’ with a CSI-RS resource in a CSI-RS resource set configured with higher layer parameter CSI-RS-ResourceRep.

…

In the above paragraph, as shown below, the agreed QCL linkage between SSB and CSI-RS for CSI w.r.t. spatial RX parameters is not captured.

| QCL linkage for above 6GHz after RRC | signalling |
| --- | --- |
| SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters | QCL type: C + D |
| TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation | QCL type: A |
| SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters | QCL type: C+D |
| SSB CSI-RS for CSI w.r.t, spatial RX parameters | QCL type: D |
| SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters | QCL type: A+D |
| CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters | QCL type: D |
| CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters | QCL type: D |
| CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI | QCL type: A+D |
| CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters | QCL type: D |

Proposal 3: To capture the agreement on QCL linkage between SSB and CSI-RS for CSI, modify the description in section 5.15 of 214.

The proposed corrections corresponding to the above issues is incorporated in the text proposal and shown in section 5.

Conclusions

Based on the discussion above, we have the following proposals. It’s noted that proposal 2 has impact on RRC parameters.

Proposal 1: To capture the agreement in RAN1 #91 more precisely, modify the description of relationship between the two reference RSs in section 5.1.5 of 214.

Proposal 2: To capture the agreement on supporting up to 2 DMRS groups, modify the description in section 5.15 of 214.

Proposal 3: To capture the agreement on QCL linkage between SSB and CSI-RS for CSI, modify the description in section 5.15 of 214.

Based on discussion above, we propose to modify the text in section 5.1.5 of 214 as shown below.

-------------------------------------------------------------Start of TP to 38.214---------------------------------------------------- 5.1.5 Antenna ports quasi co-location The UE can be configured with a list of up to M TCI-State configurations within higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI state includes one or two RS sets TCI-RS-SetConfig. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS port group of the PDSCH. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, there is no overlap between the two QCL types, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location type corresponding to each DL RS given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values: - 'QCL-TypeA': {Doppler shift, Doppler spread, average delay, delay spread} - 'QCL-TypeB': {Doppler shift, Doppler spread} - 'QCL-TypeC': { Doppler shift, average delay} - 'QCL-TypeD': {Spatial Rx parameter} The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ.After a UE receives [initial] higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If tci-PresentInDCI is set as 'disabled' for the CORESET scheduling the PDSCH or the PDSCH is scheduled by a DCI format 1_0, for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the CORESET used for the PDCCH transmission. If the tci-PresentInDCI is set as 'enabled', when the PDSCH is scheduled by DCI format 1_1, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is based on reported UE capability [12, TS 38.331]. For both the cases when tci-PresentInDCI = 'enabled' and tci-PresentInDCI = 'disabled', if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi co-location indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are configured for the UE. If all configured TCI states do not contain 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. A UE should expect only the following qcl-Type configurations in the higher layer parameter TCI-State: - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect - 'QCL-TypeC' or {QCL-TypeC' and QCL-TypeD'} configurations with SS/PBCH block, or - ’QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition , the UE should only expect - 'QCL-TypeA' or 'QCL-TypeB' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info or - 'QCL-TypeD' with a SS/PBCH block or CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect - 'QCL-TypeA' configuration with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {'QCL-TypeC' and 'QCL-TypeD'} configurations with SS/PBCH block, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect - 'QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {'QCL-TypeA' and 'QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with CSI-RS in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition. - For the DM-RS of PDSCH, the UE should only expect - ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, or - ’QCL-TypeA' configuration with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, or - {QCL-TypeA' and QCL-TypeD'} configuration with SS/PBCH block if UE is not configured with a CSI-RS resource in a NZP-CSI-RS-ResourceSet with higher layer parameter trs-Info or QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - {QCL-TypeA' and QCL-TypeD'} configuration with CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition. --------------------------------------------------------------END of TP to 38.214--------------------------------------------------

References

- RAN1 #91 chairman’s notes.

- RAN1 #90 chairman’s notes.

- R1-1805796 CR to 38.214.