3GPP TSG RAN WG1 Meeting #94	R1-1808050

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on the design of NoMA receiver

Document for:	Discussion and decision

- Introduction

A general NoMA receiver has been agreed in RAN1#92b [1]. As shown in Figure 1, it is adopted as the general block diagram of multi-user receiver for UL data transmissions.

- The algorithms for the detector block (for data) can be e.g. MMSE, MF, ESE, MAP, MPA, and EPA.

- The interference cancellation can be hard, soft, or hybrid, and can be implemented in serial, parallel, or hybrid.

  - Note: the IC block may consist of an input of the received signal for some types of IC implementations

- The interference cancellation block may or may not be used.

  - Note: if not used, an input of interference estimation to the decoder may be required for some cases.

- The input to interference cancellation may come directly from the Detector for some cases

Figure 1: A high-level block diagram of multi-user receiver

In RAN1#93, it is further agreed that when presenting the performance results, the complexity of the receiver should also be discussed and provided with details FFS.

In this contribution, we discuss about several multi-user (MU) receivers based on the agreed high-level diagram in Figure 1. The currently proposed receivers can be classified into two families: chip-wise and block-wise, as summarized in Table 1.

Table 1 Classification of NoMA Receiver Algorithms

| #### Chip-wise MU receiver | #### Block-wise MU receiver |
| --- | --- |
| MPA | Block-MMSE |
| EPA |  |
| ESE |  |
| Chip-MMSE (i.e. MMSE-IRC) |  |

- MU Detector Algorithms

  - Chip-wise MUD

A chip-wise MUD operates on each RE independently. In order to exploit the signal processing of NoMA, a certain process is applied to iteratively refine the estimate of multi-user signals. One way is iterative message passing or belief propagation which is a mature receiver architecture that has been used by LDPC decoder to achieve a high throughput in a commercial system. In this family, we have MPA (message-passing-algorithm) [7] and EPA (expectation propagation algorithm) that is simpler but with a similar performance.  Another approach is using the soft information from FEC to refine the approximation of inter-user interference, which is done in ESE. On top of these, the baseline chip-MMSE (i.e. MMSE-IRC) falls in this family of chip-wise MUDs.

MPA

MPA can reach a near-ML (Maximum likelihood) detection performance. It passes the conditional probability back and forth between every FN (function node, representing RE) and VN (variable node, representing data layer) edge in the factor graph of a NoMA scheme. In each iteration, the values (probabilities) on FNs and VNs are updated respectively. After a number of inner loops, the LLRs for the coded bits are calculated based on the current probabilities and then input to the channel decoder [2].

Its arithmetic complexity order is O(MPdf) per iteration, where Mp denotes the number of the points on one RE constellation corresponding to a log2M-bit mapping.  M≥Mp, e.g., M=8, Mp =4; M=16, Mp =9; M=64, Mp =16.  df   denotes the number of the (data) layers colliding over each RE.  This complexity order can be further reduced by restricting the maximum number of layers: ds<=df , from O(MPdf)  to O(MPds). This MPA with SIC is called SIC-MPA in [2] and [8].

Its major advantages are:

- Nearly-ML performance

- numeric stability

- intrinsic  divider-and-conquer scheme

- Common message-passing architecture

Observation 1: MPA receiver reaches a nearly-ML performance with a numeric stability. Its intrinsic divider-and-conquer scheme allows a high parallelism message-passing architecture.

EPA

To alleviate the high computational complexity of MPA on higher modulation (Mp), we propose an EPA receiver that employs the classic approximate Bayesian inference technique [10][11][12]. EPA is also a mature algorithm widely used in machine learning area. It projects the true posterior distribution of the transmitted symbols into a family of Gaussian distributions by iteratively matching the means and variances with the true posterior distribution. The fundamental principles and detailed algorithms of EPA receiver are provided in the Appendix.

In one sense, EPA is a Gaussian approximation to MPA but with consideration of the non-Gaussian nature of the transmitted symbols as well. The direct benefit of this approximation is a linear complexity with respect to M (Mp if low projection mapping is used) and df (ds if SIC-EPA is used) with nearly the same performance as MPA in most scenarios of interest [4][9].

To prove it, we set up CB-OFDMA (contention-based OFDMA) as an example for transmitter processing and adopt iterative hybrid PIC shown in Figure 4(b) (to be discussed later). We investigate MPA, EPA, and ESE in a chip-by-chip manner. Details of simulation parameters are listed in the Appendix. As shown in Figure 2, EPA can achieve similar performances as MPA in many scenarios, while ESE has some performance loss and slower convergence rate compared with MPA, especially when the load gets higher. It is noted that EPA can be implemented on the message passing architecture too.

(a) 4UE, CB-OFDMA, SE=0.2			                (b) 6UE, CB-OFDMA, SE=0.2

Figure 2: BLER performances with different receivers

Observation 2: EPA can achieve similar performance as MPA in many cases of interest with lower complexity.

ESE

ESE (elementary signal estimator) [5] simply approximates the ISI plus Gaussian noise. Such a Gaussian approximation can be implemented in different ways for multiple receiving antennas: If a base-station performs matched filtering (MF) in the spatial domain, the approximation is a scalar Gaussian variable. If a base-station treats all receiving antennas jointly, the approximation is a joint Gaussian vector, i.e., multivariate Gaussian.

Moreover, an ESE detector has to rely on the outer-loop iterations (feedbacks from channel decoder) to achieve an acceptable detection performance. In case of high spectrum efficiency and high overloading, the number of outer-loop iterations may be too large for base station to satisfy short latency constraint.

ESE can also be viewed as a simplification of EPA without iteratively refining the Gaussian approximation of the prior distribution (VNs).

Observation 3: ESE heavily relies on the coding gain from the FEC and thus may have slower convergence, especially at high spectral efficiency and high overloading.

Chip MMSE

MMSE approximates the prior distribution of the signal as Gaussian whose mean and variance are computed from either soft LLRs fed back by the channel, or a Gaussian approximation with zero mean and variance scaled by the signal power (if the soft feedback is unavailable) [6]. Thus, chip-MMSE can be equivalent to the baseline receiver MMSE-IRC.

A Chip-MMSE, which is the baseline MUD, is performed on each RE independently. A chip-MMSE inverses a Nr-by-Nr complex-valued covariance matrix on each RE. Chip-MMSE can be regarded as a special case of EPA without iteration but only computation on FNs.

Tandem Solution

If we consider a common message passing architecture to accommodate both EPA and MPA simultaneously, this tandem solution allows more flexibility and adaptability. For example, if the modulation order is small, this tandem platform could run MPA receiver to have the best detection. If the modulation order is great, it could adopt EPA for lower complexity.

Besides, the iterations of EPA/MPA can be adaptive in terms of factors such as the number of UEs, the payload size, and the number of receive antenna, which makes the message-passing architecture flexible enough to achieve the tradeoff between performance and complexity. ESE and the baseline chip-MMSE can be realized by this tandem solution by having no iteration in EPA.

Observation 4: A base-station could use a flexible Tandem solution to implement chip-wise MUDs (including one or more of MPA, EPA, ESE, chip-MMSE) on one common architecture and use the MUDs in an adaptive way.

One good feature of chip-wise MUDs is the capability of working for any NoMA configuration or a mixture of configurations no matter what NoMA signal processing is used at the transmitter side. Chip-wise MUDs, e.g. EPA, can also be used to improve the MU-MIMO performance for legacy Rel-15 UEs. The chip-MMSE is the same as the baseline MMSE-IRC receiver for MU-MIMO. Figure 3 shows EPA achieves better performance than the baseline chip-MMSE (or MMSE-IRC) receiver. The detailed parameters to generate the figure are listed in the Appendix Table A-2. Thus, once the network implements advanced chip-wise MUD for NoMA UEs, the same advanced chip-wise MUD can be used to improve the performance of legacy UEs and UEs that do not support NoMA.

Figure 3: Performance comparison between EPA and chip-MMSE

Observation 5: Chip-wise MUD can improve the MU-MIMO performance of legacy UEs and UEs not supporting NoMA.

  - Block-wise MUDs

A block-wise MUD operates on a set of REs (block) to exploit the symbol structure provided by NoMA transmit side signal processing. The most common structure is linear spreading which can be exploited by using a block MMSE detector.

A block-MMSE that is performed jointly on the spreading block of size L REs inverses Nr×L-by-Nr×L complex-valued covariance matrix. Compared to the baseline chip-MMSE, a block-MMSE has much higher complexity, i.e., ONr3L3 vs. ONr3L, especially when the spreading factor L is large. When the matrix size (Nr·L by Nr·L) is beyond certain level, numeric stability becomes a priority in reality and parallelization would suffer exponentially.

In general, it is hard to design a common architecture for both chip-MMSE and block-MMSE and to switch between them depending on the system requirement. In particular, to have short latency, a special hardware-implementation is needed for block-MMSE.

Observation 6: A block-MMSE has higher complexity than a chip-MMSE.

Observation 7: It is difficult to have a flexible implementation on a common architecture that allows dynamic usage of both chip-wise MUD and block-MMSE.

Furthermore, block-MMSE relies on a certain structure (i.e. linear spreading) provided by the NoMA transmitter signal processing over a block of REs. Once this structure (i.e. linear spreading) does not exist, block-MMSE has to be converted into chip-MMSE, i.e. the block length reduces to 1. As there is no block spreading for legacy UEs’ transmission, applying the block-MMSE to legacy UEs can only achieve the baseline chip-MMSE (i.e. MMSE-IRC) performance.

Observation 8: Block-MMSE cannot be used to improve the MU-MIMO performance of legacy UEs and UEs not supporting NoMA.

- Interference Cancelation

| - SU detector + SIC | - MU detector + PIC |
| --- | --- |

Figure 4: General iterative receiver structure for NoMA.

In one dimension, the interference cancellation (IC) can be hard, soft, or hybrid.

- Hard IC: a channel decoder feeds back the correctly decoded (i.e., passed CRC check) binary bits to the detector for the interference reconstruction. It feeds back only correctly decoded data streams.

- Soft IC: a channel decoder feeds back the LLR (log likelihood ratios) to the detector for the interference reconstruction, no matter whether the data stream can be correctly decoded or not. However, it is a waste to use LLR values rather than hard IC for those correctly decoded code-words.

- Hybrid IC: a channel decoder feeds back binary bits of the correctly decoded codewords and LLRs of the incorrectly decoded ones, as shown in Figure 4. For those correctly decoded users, the interferences are hard canceled (the blue dashed line); for those undecoded users, the soft information is fed back by the decoder (the orange dashed line). In this way, we can lower the implementation complexity of a soft-IC and improve the performance of hard IC.

Observation 9: Hybrid IC can take advantages of both pure soft and pure hard IC schemes and achieves the best performance with low-complexity implementations.

In another dimension, IC can be implemented in serial, parallel, or hybrid.

- SIC (Serial IC, also known as successive IC) decodes only one user at a time, as shown in Figure 4(a). Although the order of SIC depends on SINR values to take advantage of near-far effect among users, such an ordered SIC may bring about a well-known error propagation. To overcome it, in enhanced SIC, the order of SIC is revised each time a UE is successfully decoded.

- PIC (parallel IC) decodes all the active users simultaneously, thereby avoiding the order-related error propagation of SIC and improving the performance, as shown in Figure 4(b) and Figure 5. In addition, it has low decoding latency due to higher parallelism. Note that one can also decode a subset of users at a time, which is called group PIC and may acquire additional operations for group selection.

Observation 10: Compared with SIC, PIC has much lower decoding latency and can avoid the impact of error propagation problem caused by inaccurate SINR sorting.

We evaluate and compare the performances of the MMSE detector combined with four different IC methods, as shown in Figure 5, namely hard ordered SIC, hard enhanced SIC, hard PIC, soft PIC, and hybrid PIC. The simulation assumption is listed in Table A-3. For hard-PIC, soft-PIC and hybrid-PIC, the number of outer-loop iterations is OL=3.

As Figure 5 illustrates, the hard ordered SIC has the worst performance in the case of interest. Although the hard enhanced SIC outperforms hard ordered SIC, it still has an apparent performance loss to PIC. Within the family of PIC, the hybrid-PIC has slightly better performance than the soft-PIC, which has an apparent gain over the hard-PIC.

Figure 5: BLER performance of CB-OFDMA with MMSE Detector and different IC.

Observation 11: Hybrid PIC archives the best performances among different combinations of IC implementations.

| - 4UEs, 6RBs, 60bytes, 2Rx, OL 0, 1, 2, 3 | - 6UEs, 6RBs, 60bytes, 2Rx, OL 0, 1, 2, 3 |
| --- | --- |

Figure 6: BLER performance of CB-OFDMA with different number of outer-loops.

No matter which type of IC is applied, the performance of the iterative NoMA receiver can be closely related to the number of iterations between the channel decoder (backend) and the MU detectors (frontend), also known as outer-loop iterations. To show the necessity of an iterative receiver, we apply an advanced receiver to CB-OFDMA. In Figure 6, the BLER performance improves along with the increased number of outer-loop iterations until the performance is converged. Moreover, the more the multiplexed users/layers are, the larger the number of iterations is needed for convergence. The detailed parameters to generate the figure are listed in the Appendix Table A-4.

Observation 12: An iterative receiver can improve the BLER performance of the NoMA transmissions. The higher the number of users, the more iterations may be needed.

- Discussion on Implementation Cost

The detailed implementation cost closely depends on various methods and architectures to implement an algorithm in general, which is hard to be surveyed exhaustively.

In Table-2, complexity order of different MU detectors are listed, where L is the spreading factor, df is the number of superimposed users at the same resource, Mp is the modulation alphabet size, Nr is the number of receive antennas, Tinner is the number of internal iterations on the factor graph and Touter is the number of outer loop iterations.

Table 2: Complexity comparison for different MU detectors

| MU Detector | Order of Complexity (Major Operations) |
| --- | --- |
| MPA | OTouterTinnerLMpdf |
| EPA | OTouterTinnerLNr3+TouterTinnerLNr2df+TouterTinnerLNr2 |
| MF ESE | OTouterLNrdf2 |
| Chip-wise MMSE | OTouterLNr3+TouterLNr2df+TouterLNr2 |
| Block-wise MMSE | OTouterNr3L3+TouterNr2L2df+TouterNr2L2 |

Different MU-detectors may suit with different implementation architecture to exploit more benefits. For example, a MPA detector adopts a famous message-passing architecture that can generate a high T/P, implementation efficiency, and short latency. Also, on each iteration of an EPA detector, each FN inverses a Nr-by-Nr covariance matrix and these back-to-back small-size matrix inversions can be pushed into a classic systolic array pipeline to have a high-T/P, constant latency, stable and exception-free operation. With a systolic array, an EPA detector requires relatively small computational resources and shows good compatibility with current hardware design. ESE and baseline chip-MMSE detectors can also be realized using the same architecture.

For block-wise MMSE, the matrix inversion dimension L∙Nr introduces heavy arithmetic complexity. Some methods such as Sherman-Morrison have been introduced to decrease the arithmetic complexity. However, this method has some problems on an implementation level; the precision of a Sherman-Morrison implementation depends on the worst case, i.e,  Lmax∙Nrmax which means data bus, memory ports, buffer size, etc. should be designed according to the worst case too. It will waste significant physical resource when the average operating point is far less than the worst one. Furthermore, any scaling Lmax∙Nrmax would request a new optimization of the implementation, which could be costly. A typical value of L∙Nr may request a pseudo-floating-point division (<32 bit but with mantissa and exponent) to ensure a sufficient precision and the complexity of such operation can be very high. Besides, the latency of a MMSE receiver is determined by that of matrix inversion. The Sherman-Morrison method is successive and hard to be parallelized for high T/P and short latency.

Observation 13: Chip-wise MU detectors, including chip MMSE, MPA, EPA and ESE provide better tradeoff between flexibility, hardware resource efficiency, processing latency and numerical stability compared to block wise MU detectors like block MMSE.

- Conclusions

In this contribution, we discussed the common iterative receiver structure for NoMA transmissions consisting of the MU detectors and the IC methods. From the discussions, we obtained the following observations and proposals.

Observation 1: MPA receiver reaches a nearly-ML performance with a numeric stability. Its intrinsic divider-and-conquer scheme allows a high parallelism message-passing architecture.

Observation 2: EPA can achieve similar performance as MPA in many cases of interest with lower complexity.

Observation 3: ESE heavily relies on the coding gain from the FEC and thus may have slower convergence, especially at high spectral efficiency and high overloading.

Observation 4: A base-station could use a flexible Tandem solution to implement chip-wise MUDs (including one or more of MPA, EPA, ESE, chip-MMSE) on one common architecture and use the MUDs in an adaptive way.

Observation 5: Chip-wise MUD can improve the MU-MIMO performance of legacy UEs and UEs not supporting NoMA.

Observation 6: A block-MMSE has higher complexity than a chip-MMSE.

Observation 7: It is difficult to have a flexible implementation on a common architecture that allows dynamic usage of both chip-wise MUD and block-MMSE.

Observation 8: Block-MMSE cannot be used to improve the MU-MIMO performance of legacy UEs and UEs not supporting NoMA.

Observation 9: Hybrid IC can take advantages of both pure soft and pure hard IC schemes and achieves the best performance with low-complexity implementations.

Observation 10: Compared with SIC, PIC has much lower decoding latency and can avoid the impact of error propagation problem caused by inaccurate SINR sorting.

Observation 11: Hybrid PIC archives the best performances among different combinations of IC implementations.

Observation 12: An iterative receiver can improve the BLER performance of the NoMA transmissions. The higher the number of users, the more iterations may be needed.

Observation 13: Chip-wise MU detectors, including chip MMSE, MPA, EPA and ESE provide better tradeoff between flexibility, hardware resource efficiency, processing latency and numerical stability compared to block wise MU detectors like block MMSE.

References

“RAN1 Chairman’s Notes,” RAN1#92, Sanya, China, Mar 16-20, 2018.

R1-164390, “Transceiver implementation and complexity analysis for SCMA,” Huawei, HiSilicon, RAN1#85, Nanjing, China, May 16-20, 2016.

R1-166098, “Discussion on the feasibility of advanced MU-detector,” Huawei, HiSilicon, RAN1#86, Gothenburg, Sweden, Aug 22-26, 2016.

R1-1608855, “Advanced multi-user detectors for grant-free transmissions,” Huawei, HiSilicon, RAN1#86, Lisbon, Portugal, Oct 10-14, 2016.

L. Ping, L. Liu, K. Wu and W.K. Leung, “Interleave-Division Multiple-Access,” IEEE Trans. Wireless Communications, Vol. 5, No. 4, pp. 938–947, Apr. 2006.

X. Wang and H. V. Poor, “Iterative (turbo) soft interference cancellation and decoding for coded CDMA,” IEEE Trans. Communications, vol. 47, pp. 1046–1061, July 1999.

F. R. Kschischang, B. J. Frey, and H. A. Loeliger, “Factor graphs and the sum-product algorithm,” IEEE Trans. Inf. Theory, vol. 47, no. 2, pp. 498–519, Feb. 2001.

A. Bayesteh, H. Nikopour, M. Taherzadeh, H. Baligh, and J. Ma, “Low Complexity Techniques for SCMA Detection”, in IEEE GLOBECOM 2015, San Diego, CA, Dec. 2015.

X. Meng, Y. Wu, Y. Chen and M. Cheng, “Low complexity receiver for uplink SCMA system via expectation propagation,” in Proc. IEEE WCNC, San Francisco, USA, 2017.

T. P. Minka, "Expectation propagation for approximate Bayesian inference." in Proc. of the Seventeenth conference on Uncertainty in artificial intelligence. Morgan Kaufmann Publishers Inc., 2001.

Murphy, Kevin P. "Machine learning: a probabilistic perspective." MIT press, 2012.

C.M. Bishop, Pattern Recognition and Machine Learning. Springer, Aug. 2006.

Appendix A – Parameters

Table A-1: Evaluation parameters for Figure 2.

| Parameters | Values or assumptions |
| --- | --- |
| NoMA scheme | Contention based OFDMA |
| Carrier frequency | 2 GHz |
| Numerology | 14 OS slot, 2 OS DMRS overhead |
| Transmission Bandwidth | 4RB |
| Target spectral efficiency | Per UE spectral efficiency: 0.2 bps/Hz |
| Channel coding | LTE Turbo |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiplexed UEs | 4, 6 |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Advanced receivers | MPA, EPA, ESE, all with PIC |

Table A-2: Evaluation parameters for Figure 3.

| Parameters | Values or assumptions |
| --- | --- |
| NoMA scheme | Contention based OFDMA |
| Carrier frequency | 700 MHz |
| Numerology | 14 OS slot, 2 OS DMRS overhead |
| Transmission Bandwidth | 6RB |
| TBS | 20 bytes |
| Channel coding | NR LDPC |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiplexed UEs | 4 |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Advanced receivers | EPA and chip-MMSE |

Table A-3: Evaluation parameters for Figure 5.

| Parameters | Values or assumptions |
| --- | --- |
| NoMA scheme | Contention based OFDMA |
| Carrier frequency | 700 MHz |
| Numerology | 14 OS slot, 2 OS DMRS overhead |
| Transmission Bandwidth | 6RB |
| TBS | 60 bytes |
| Channel coding | NR LDPC |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiplexed UEs | 6 |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Advanced receivers | MMSE with different IC methods |

Table A-4: Evaluation parameters for Figure 6.

| Parameters | Values or assumptions |
| --- | --- |
| NoMA scheme | Contention based OFDMA |
| Carrier frequency | 700 MHz |
| Numerology | 14 OS slot, 2 OS DMRS overhead |
| Transmission Bandwidth | 6RB |
| TBS | 60 bytes |
| Channel coding | NR LDPC |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| SNR distribution of Multiple UEs | Equal SNR |
| Number of Multiplexed UEs | 4, 6 |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Advanced receiver | EPA with PIC |

Appendix B - EPA algorithm

B.1 Design Principle

EPA is a well-known approximate Bayesian inference technique which approximates the target distribution p with a simpler exponential family distribution q, where the Kullback-Leibler divergence KL(p|q is minimized [10]. It has already been widely used in the field of machine learning [11][12]. Conceptually, as shown in Figure , EPA can be viewed as an operation to project the target distribution p into the exponential family distribution set Φ, i.e.,

ProjΦp=argminq∈ΦKL(p||q)  		     					()

If p∈Φ, then the projection reduces to an identity mapping. However, in general, p∉Φ and hence such a projection is nontrivial.

Figure B-1: Basic principle of EPA.

It has been proved that the optimal solution to (1) can precisely match the sufficient statistics of target distribution p. For example, if  q is Gaussian distribution, then the mean and covariance of q will equal to the mean and covariance of the true distribution p, respectively.

For the problem of MU detection, the target distribution p usually comprises a product of factors in the form of

p=1Difix  									()

where D is normalization constant. EPA approximates (2) by a product of factors

q=1Zifix  									()

where each factor fix  corresponds to one of the factors fix in the target distribution p, and Z is a normalizing constant.  If each factor  fix comes from the exponential family, the product of factors will be also from exponential family and thus can be described by a set of sufficient statistics.  Since direct solving (1) is usually intractable, EPA optimizes each factor iteratively. Specifically, it starts by initializing all the factors, and then cycles through the factors by refining one factor at a time. For example, suppose fjx is to be refined, we first remove it from q and obtain q\j=q/fjx. Then, the new approximate distribution qnew is calculated by minimizing KL(1Zjfjx q\j||qnew). As a result, the refined factor  fjx can be calculated as qnew/ q\j. After a number of iterations, the approximate distribution q is obtained as the product of the refined factors.

B.2 Factor Graph Model

The system model of NoMA can be represented with a factor graph as shown in Figure B-2. From the perspective of factor graph, EPA can be iteratively realized in a message passing way as MPA.

Assume that K UEs transmit on a group of L resource elements (RE), and each UE has single transmit antenna. For spreading-based NoMA schemes, L is referred to as spreading factor. For NoMA schemes without spreading, L=1. Assume gNB has Nr  receive antennas. The received signal of the r-th antenna can be written as

yr=k=1Kdiag(hk,r)xk+wr,                                                            ()

where yr∈CL×1 is the received symbol vector, hk,r∈CL×1 is the channel coefficient vector between user k and r-th antenna, xk∈CL×1 is the transmitted symbol vector of user k, wris the associated additive white Gaussian noise with the power of σ2. Suppose the modulation size is M and the codebook is Xk, then xk∈Xk and the cardinality of the codebook is Xk=M.

For linear spreading schemes, the effective channel hkn takes into account both the channel response of r-th receive antenna hk,r and the user-specific spreading sequence sk∈CL×1, i.e., hk,r= hk,r⨀sk, where ⨀ denotes component-wise multiplication, while xk=xk1;…;xkL∈CL×1 is a L times repetition of transmitted symbol xk, i.e., xkl=xk, l=1,..,L. For schemes based on bits-to-symbols mapping, the effective channel hk,r is the channel response of r-th receive antenna and xk=xk1;…;xkL∈CL×1 is a L-dimentional transmitted symbol vector selected from a predefined codebook. If sparse pattern is applied, some elements of  xk are fixed to be zero all the time. In matrix form, the overall observations from Nr receive antennas can be written as

y=Hx+w    		                                                        ()

where y=y1;…;yNr, x=x1;…;xK, w=w1;…;wNr and H is the overall effective channel matrix.

The general factor graph representation for NoMA is shown in Figure B-1, which contains K variable nodes (VNs) xk and L likelihood factor nodes (FNs) fl, and K prior FN ∆k. The VNs represent the transmitted symbols xk, k=1,…,K, and the prior FN ∆k represents the prior distribution P0xk of xk, which can be computed via the feedback LLRs from the FEC decoder. The likelihood FN fl represents the likelihood probability distribution pyl|x, where yl=[yl1,…,ylNr]∈CNr×1 is the observation vector at the l-th RE.  Note that the FN fl has connection with xk if and only if the lth element of xk, xkl≠0. For ease of notation, let Vk=l:xkl≠0  denote the neighboring FNs of VN xk, and Fl=k:xkl≠0 denote the neighboring VNs of the FN l,r for each receive antenna. The cardinalities of  Vk and Fl are assumed to be Vk=dv,k=1,…,K, Fl=df, l=1,…,L， respectively. As a result, the variables nodes connected with FN fl can be represented as a vector  xl=xkl|k∈F(l)∈Cdf×1 . The linear observation equation corresponding to FN fl is

yl=Hlxl+wl                                                                             ()

where  Hl∈CNr×df and wl∈CNr×1 are the corresponding channel matrix and additive Gaussian noise vector, respectively. Then the likelihood probability distribution pyn|x can be written as

Pyl|x=Pyl|xl=CN(yl;Hlxl,σ2I)                                         ()

Figure B-2: A general Factor Graph of NoMA.

B.3 EPA Procedure

In this subsection, the basic procedure of EPA is briefly described. There are two steps for each round of iteration: FN update and VN update, respectively. For the VN update at the t-th iteration, the message  Ik→ltxk from VN xk to FN fl is computed, and for the FN update, the message Il→ktxk  of the opposite direction is computed. According to the principle of EPA, the messages are updated as follows

Ik→ltxk=ProjΦptxkIl→kt-1xk,		()

Il→ktxk=ProjΦqltxkIk→ltxk,		()

where

ptxk=IΔ→kxkn∈VkIn→kt-1xk		()

qltxk=Ik→ltxkxm,m∈Fl,m≠kpyl|xlm∈Fl,m≠kIm→ltxm		()

If the projection set Φ is chosen to be Gaussian distribution, then the messages Ik→ltxk and Il→ktxk reduce to Gaussian distribution which can be fully characterized by its mean and covariance. That is why this algorithm called expectation propagation.  As a result, the computational complexity is significantly reduced. Moreover, from (5) and (7), since the l-th likelihood factor node is only related to xkl,k∈Fl, the messages of Ik→ltxk and Il→ktxk can be further simplified to scalar complex Gaussian distributions, i.e., Ik→ltxkl, and Il→ktxkl, respectively.

Specifically, at the t-th iteration, if the message Il→ktxk, i.e., Il→kt-1xkl, from the likelihood FN fl  to VN xk is circularly complex Gaussian with mean μl→kt-1 and variance ξl→kt-1, i.e.,

Il→kt-1xkl∝CNxkl;μl→kt-1,ξl→kt-1 	                                                       ()

With (10), we can compute ptxk as

ptxk∝IΔ→kxkn∈VkCNxkn;μn→kt-1,ξn→kt-1.		()

Then, to compute the message from VN k to FN fl, the ptxk is projected to a Gaussian distribution CNxkl;μklt,ξklt by matching the mean μklt and the variance ξklt with respect to (w.r.t.) ptxk. That is, the mean μklt and the variance ξklt are computed w.r.t the approximated posterior probability ptxk in (13).

To compute the mean μklt and variance ξklt,  normalization of ptxk is needed to obtain the discrete probability for xk from the hybrid discrete and continuous density function in (13). Suppose that the modulation size is M and the codebook is Xk of size M, then

ptxk=α=IΔ→kxk=αn∈VkCNxkn=αn;μn→kt-1,ξn→kt-1a∈XkIΔ→kxk=αn∈VkCNxkn=αn;μn→kt-1,ξn→kt-1, α∈Xk               ( AUTONUM  )

Where α∈Xk is an L-dimensional vector and αn is the nth element of L-dimensional vector α∈Xk. In practice, the multiplications of Gaussian density functions in (14) can be computed in log domain and thus simplified. As a result, the mean μklt and the variance ξklt are computed as

μklt=a∈Xkptxk=ααl,                                                        ( AUTONUM  )

ξklt=a∈Xkptxk=α|αl-μklt|2,                                       ( AUTONUM  )

From (8), we can compute the message Il→ktxk as

Il→ktxk≡Ik→ltxkl=CNxkl;μklt,ξkltCNxkl;μl→kt-1,ξl→kt-1∝CNxkl;μk→lt,ξk→lt 		()

where

1ξk→lt=1ξklt-1ξl→kt-1 		 ()

μk→ltξk→lt=μkltξklt-μl→kt-1ξl→kt-1 		()

Next, the message computation from FN to VN is described. Given the Gaussian messages Il→ktxk from VN to FN in (17), it can be easily shown that qltxk in (11) is also Gaussian which is denoted by

qltxk≡qltxkl=CNxkl;μklt,ξklt                                          ()

Moreover, the mean and variance μklt,ξklt are precisely the linear MMSE estimates of xkl over a linear observation equation (6), where the message Il→ktxk can be viewed as the prior Gaussian distribution of xkl. Specifically, at the l-th RE, the prior mean and covariance matrix of xl=xkl|k∈F(l) can be denoted as mlpri=μk→lt|k∈F(l)∈Cdf×1 and Σlpri=diag(ξk→lt|k∈F(l))∈Cdf×df, respectively, then the posterior mean mlpost=μklt|k∈F(l)∈Cdf×1 and covariance Σlpost∈Cdf×df of  xl can be calculated as

mlpost=mlpri+ Σlpri HlHHlΣlpriHlH+σ2I-1yl-Hlmlpri       ( AUTONUM  )

Σlpost=Σpri-ΣlpriHlHHlΣlpriHlH+σ2I-1HlΣlpriH.                 ( AUTONUM  )

The diagonal elements of  Σlpost are the posterior variances of xkl: ξklt|k∈F(l).

Given the posterior mean μklt and variance ξklt of xkl, from (9) and (20), the messages Il→ktxk are also Gaussian, i.e., Il→ktxk≡Il→ktxkl=CNxkl;μl→kt,ξl→kt, whose mean and variance are given by

1ξl→kt=1ξklt-1ξk→lt    		()

μl→ktξl→kt=μkltξklt-μk→ltξk→lt                	 	 ()

Till now, the t-th iteration is completed. In practice, after multiple iterations, the log-likelihood-ratio (LLR) for coded bits can be calculated based on the approximated posterior probability ptxk in 	(13). After subtracting the prior LLRs, the extrinsic LLRs serve as the input to the FEC thereafter.

For DFT-s-OFDM waveform, the EPA algorithm can also be applied. The difference is after FN update and VN update, IDFT and DFT operation is applied to transform the mean and variance between time and frequency domain.

B.4 EPA Algorithm Summary

The EPA for NoMA detection is summarized in Algorithm1, where the number of iterations is Tmax.

-------------------------------------------------------------------------------------------------------------------------------

Algorithm1 – EPA Detector

-------------------------------------------------------------------------------------------------------------------------------

      - Initialization

- Initialize the mean and variance from FN to VN as μl→k0=0,ξl→k0=Inf, l=1,…,L, k∈F(l).

      - Iterations

Start with t=1, and ptxk=α=IΔ→kxk=α.

While t≤Tmax, Do

- VN Update: For k=1,…,K:

For l∈Vk :

- Compute μklt and ξklt as

μklt=a∈Xkptxk=ααl,                                                        ( AUTONUM  )

ξklt=a∈Xkptxk=α|αl-μklt|2,                                       ( AUTONUM  )

where αl is the l-th element of L-dimensional vector α∈Xk.

- Compute the mean μk→lt and variance ξk→lt as

1ξk→lt=1ξklt-1ξl→kt-1 		 ()

μk→ltξk→lt=μkltξklt-μl→kt-1ξl→kt-1 		()

- FN Update: For l=1,…,L:

- Perform chip-by-chip MMSE as

mlpost=mlpri+ Σlpri HlHHlΣlpriHlH+σ2I-1yl-Hlmlpri.       ( AUTONUM  )

Σlpost=Σpri-ΣlpriHlHHlΣlpriHlH+σ2I-1HlΣlpriH.                  ( AUTONUM  )

where mlpri=μk→lt|k∈F(l)∈Cdf×1 and Σlpri=diag(ξk→lt|k∈F(l))∈Cdf×df.

- For k∈F(l): Given the posterior mean μklt|k∈F(l)=mlpostand variances ξklt|k∈F(l)=diag(Σlpost) of xkl, compute the mean μl→kt and variance ξl→kt as

1ξl→kt=1ξklt-1ξk→lt    		()

μl→ktξl→kt=μkltξklt-μk→ltξk→lt                	 	 ()

- Update ptxk：

ptxk=α=IΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kta∈XkIΔ→kxk=αn∈VkCNxkn=αn;μn→kt,ξn→kt, α∈Xk	   ()

      - LLR Calculations:

Compute the extrinsic LLRs.

Note that in practical implementations, some numerical protection can be used, e.g., variance below a threshold is set to be the threshold. In addition, damping operation can be used in the inner iterations to further improve the convergence performance.

CB-OFDMA here refers to the contention based OFDMA where all the UEs fully share the same time and frequency resource and may transmit simultaneously without any change from the current NR OFDMA transmitter side design.