3GPP TSG RAN WG1 Meeting #94	 	            R1-1808182

Gothenburg, Sweden, August 20th – 24th, 2018

Source:	ZTE

Title:	Updated UMi calibration results for user experience data rate in the multi-layer network deployment

Agenda Item:  7.2.7.2

Document for:	Discussion

Introduction

There is an approved Study Item on Self Evaluation towards IMT-2020 submission in the RAN#75 meeting, which mainly focuses on evaluated RAN technologies based on Rel-15 and beyond to satisfy all ITU-R IMT-2020 requirements including eMBB scenario.

At RAN#77 meeting, the general work plan of self evaluation is approved [1]. Moreover, “[ITU-R AH 01] Calibration for self-evaluation”[2] has been set up for discussion on calibration and collection of calibration results.  It is observed that the calibration results are well aligned according to the results collected so far for InH_x, UMa_x and RMa_x in all IMT-2020 defined test environments.

Based on ITU-R report M.2410 and M.2412, in order to evaluate user experience data rate in the Dense Urban-eMBB’s multi-layer network deployment, the UMi_x should be considered and additional calibration work is necessary.

In this contribution, some initial considerations and calibration results of UMi_x are shared for related evaluated Dense Urban-eMBB test environment.

User experienced data rate

The evaluation method of User experienced data rate

ITU-R report M.2142 said that “The evaluation is conducted in Dense Urban-eMBB test environment.

…

In case of multi-layer configuration, system-level simulation is used. In this case, the single user data rate may be aggregated over layers and/or bands. The user experienced data rate is derived from the 5th percentile point of the CDF of single user data rate.” in the section 7.2.3

Moreover, minimum requirements of User experienced data rate are defined in the ITU-R report M.2140.

| Test environment | User experienced data rate (Mbit/s) |  |
| --- | --- | --- |
|  | DL | UL |
| Dense Urban – eMBB | 100 | 50 |

Table 2-1 Dense Urban – eMBB

Observation1: Generally, the case of single-layer layout of Dense Urban-eMBB, analysis as a typical evaluation methodology is enough for evaluation of user experience data rate.

Observation2: In the case of multi-layer layout of Dense Urban-eMBB, system-level simulation for user experience data rate is only considered in the multi-layer network layout of the test environment.

Initial consideration about UMi calibration

The following metrics are selected for calibration of self evaluation at this stage:

- DL Geometry (wideband SINR)

- Coupling loss

The above metrics will be used with the cell association mechanism as defined in calibration assumptions in Appendix; these metrics are same with previous calibration activities of IMT-2020 self-evaluation [4].

However, some additional simulation procedures of UMi_x calibration should be clarified for accuracy.

Firstly, with regard to cluster dropping, clusters of micro TRxPs are randomly and uniformly dropped within a macro layer. Because of one-sector deployment in the initial calibration of UMi, a specific dropping of TRxP are referenced according to TR 36.897, which some detail can be found in the Appendix. However, there is an obvious issue. Randomly dropping cluster will possibly have negative influence on calibration results. From our simulation observation, considering 200m ISD in Dense Urban-eMBB, number of possible location of cluster is limited.   If number of dropping in the simulation procedure is enough, then the stable calibration results can be found and convinced. Whereas, ITU-R report M.2142 said that “The micro-layer deployment (e.g. 3 micro sites per macro TRxP and there is either one or 3 TRxPs at each micro site) is not defined but should be reported by the proponent.”.  So for the evaluation of user experienced data rate, the micro-layer deployment could be different and should be reported.

Secondly, about UE dropping, because of only calibration UMi, all UEs only are randomly and uniformly dropped within cluster, which means no UE are considered dropped into macro layer.  The motivation is to have the same number of UEs in each TRxP of the cluster.

- Observation3: enough number of dropping cluster in the simulation procedure is necessary when the complexity of system-level simulations can be tolerated.

- Proposal 1: For UMi_x calibration, all UEs are randomly and uniformly dropped within cluster

UMi calibration results

In Attachment-1 (Umi Calibration.xlsx), we provide (4GHz and 30GHz) UMi_x calibration parameters and models for the Dense Urban-eMBB test environments defined in Report ITU-R M.2412. In order to keep consistency with previous IMT-2020 self-evaluaiton cailbraiton activities, most parameters are reused in the UMi_x’s ; while other parameters are found in Table A-1 in Appendix.

Attachment-1

Observation 4:  Calibration of UMi_x is necessary for IMT-2020 self-evaluation. The initial calibration results of UMi_b are provided.

- Proposal 2:  Additional self evaluation calibration activities, such UMi_x should be considered, including at least 4GHz.

Conclusion

In this document, we provide our initial consideration on UL spectral efficiency of Dense urban-eMBB test environment towards IMT-2020 submission.

Observation1: Generally, the case of single-layer layout of Dense Urban-eMBB, analysis as a typical evaluation methodology is enough for evaluation of user experience data rate.

Observation2: In the multi-layer layout of Dense Urban-eMBB, system-level simulation for user experience data rate is only considered in the multi-layer network layout of the test environment.

- Observation3: enough number of dropping cluster in the simulation procedure is necessary when the complexity of system-level simulations can be tolerated.

- Observation4: Calibration of UMi_x is necessary for IMT-2020 self-evaluation. The initial calibration results of UMi_b are provided..

- Proposal 1: For UMi_x calibration, all UEs are requested to randomly and uniformly dropped within cluster

- Proposal 2: Additional self evaluation calibration activities, such UMi_x should be considered.

- References

RP-171451, “New Study Item on Self Evaluation towards IMT-2020 Submission”, RAN#75, Ericsson et al.

Report ITU-R M.2410 - Minimum requirements related to technical performance for IMT-2020 radio interface(s).

Report ITU-R M.2412 - Guidelines for evaluation of radio interface technologies for IMT-2020.

RT-170019 “Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation”” RAN78#, 3GPP TSG RAN ITU-R ad hoc, Huawei, Lisbon

R1-1703534 “Evaluation assumptions for Phase 1 NR MIMO system level calibration” RAN1#88, ZTE, Athens

- Appendix

Table A-1 Calibration assumptions for UMi_x

| Dense Urban - eMBB | Based on Config. C for user experience data rate |  |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz available in micro layer | 30 GHz available in micro layer |
| Channel model | UMi_a, UMi_b | UMi_b |
| BS antenna height | 10 m | 10 m |
| Total transmit power per TRxP | 30 dBm for 10MHz bandwidth | 33 dBm for 40MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss only for UMi_b | 20% high loss, 80% low loss only for UMi_b |
| Minimum distance between Micro TRxP centers | 40m | 40m |
| Radius of UE dropping within a cluster | 50m | 50m |
| Minimum distance between Macro TRxP and UE | 10m | 10m |
| Minimum distance between Micro TRxP and UE | 10m | 10m |
| Minimum distance between Micro TRxP center and Macro TRxP | 20m | 20m |
| Minimum distance between small cell cluster center and Macro TRxP | 20m | 20m |
| Radius for micro TRxP center dropping in a cluster | 55m | 55m |
| TRxP placement option | One-sector deployment ：Dropping of TRP and TRP antenna orientation according to TR 36.897 (non co-channel hetnet deployment) | One-sector deployment ：Dropping of TRP and TRP antenna orientation according to TR 36.897 (non co-channel hetnet deployment) |
| Number of antenna elements per TRxP | 64Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 256Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (2,1,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (2,2,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,2), (dH,dV) = (0.5, 0.5)λ,Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Device deployment | 80% indoor, 20% outdoor (in car) 10 UEs per TRxP for multi-layer case, randomly and uniformly dropped within a cluster. | 80% indoor, 20% outdoor (in car) 10 UEs per TRxP for multi-layer case, randomly and uniformly dropped within a cluster. |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 7 dB |
| UE noise figure | 7 dB (NOTE: this parameter is different from TR38.802) | 10 dB (NOTE: this parameter is different from TR38.802) |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table A-2 | See Table A-2 |
| UE antenna element gain | 0 dBi | 5 dBi |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz |
| UE density | 10 UEs per Micro TRxP | 10 UEs per Micro TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model variant | Channel model A and B Note 1 | Channel model B Note 1 |
| Number of the micro TRxPs per macro TRxP | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | According to Zenith angle in "Beam set at TRxP" | According to Zenith angle in "Beam set at TRxP" |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix (2D DFT beam) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix (2D DFT beam) |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | NULL | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |
| Criteria for analog beam selection for interfering TRxP | Random selecting the random beams for non-serving TRxP | Random selecting the random beams for non-serving TRxP |

- Note1: Channel model A of UMi also should be calibrated, and there is a same priority with model B.

Table A-2 Micro BS antenna element radiation pattern for Dense Urban – eMBB

| Parameters | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

- Note: This is a copy of Table 8-6 in Report ITU-R M.2412