3GPP TSG RAN WG1 Meeting #94   	R1-1808338

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda Item:	7.2.2.4.4

Source:	Sony

Title:	Considerations on Enhancements to Configured Grants for NR Unlicensed operation

Revision of:        R1 - 1806571

Document for:	Discussion / Decision

Introduction

As per the discussion in RAN1#93[1], the following proposals related to configured grant have been agreed.

Agreement:

- The following modifications to the configured grant procedures are beneficial

  - Removing dependencies of HARQ process information to the timing

    - Introducing UCI on PUSCH to carry HARQ process ID, NDI, RVID

  - Introducing Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission

  - Increased flexibility on time domain resource allocation for the configured grant transmissions

  - Supporting retransmissions without explicit UL grant

Targeting Rel-16, the purpose of adopting configured grant in NR-U is reducing the legacy scheduling latency and associated signaling overhead. Unpredictable Listen-Before-Talk (LBT) outcome, however, may impose potential latency which counteracts benefits brought by the well-designed feature of configured grant. To this end, we discuss the corresponding enhancements for configured grant in NR-U and concentrate on the following aspects in this paper: 1) potential enhancements for improving device channel access possibility; 2) HARQ enhancements on the process ID selection; 3) DL to UL and UL to DL COT sharing; 4) repetition number K and its associated RV pattern sequence.

Discussion

Resource allocation

To reduce control signaling overhead in LTE enhancements in unlicensed spectrum operation, AUL transmission has been introduced in FeLAA, where time-domain resource and frequency-domain resource are configured by RRC signaling and activation DCI, respectively [2]. Because of the dynamic DCI-based activation, this SPS-like mechanism makes it more flexible for both link adaptation and resource allocation. The procedure, however, exposes latency threat caused by required LBT for L1 activation signaling.

Accordingly, one candidate scheme of only RRC configuration for resource allocation has been agreed in NR grant-free transmission scheme [2], which is called Type1 UL transmission without grant. In Type1 mode, configuration parameters can only be updated by RRC (re-)configuration signaling. This scheme is instinctively lack of link adaptation, but the latency threat caused by L1 activation signaling is also avoided. For the service like VoLTE occurred on the unlicensed band, Type1 can fully meet the requirements and reach an achievable performance gain in terms of the throughput and system latency.

Another Type2 mode is much similar as AUL manner with the only difference that RRC configuration accounts for the configuration of TPC and resource periodicity and L1 DCI is responsible for resource allocation (time + frequency), time offset and UE-specific DMRS configuration. Since L1 activation signaling responses much more promptly than higher layer signaling, the performance of link adaptation is much better than Type1. However, it is hard to rule out the fact that any L1 signaling requires to perform CCA check to access channel. The potential latency caused by continuous failure of CCA attempts will become serious if configuration adjustments need to be updated frequently through L1 signaling.

Observation 1:  Type1 and Type2 modes have their own pros and cons in terms of signaling overhead and potential channel access latency.

Type1 and Type2 modes, a.k.a. configured grant, have been supported in NR Rel-15. For both types of configured grant, if UE misses the channel access opportunity at given time spot, the corresponding UL transmission has to be postponed until next allowed access location. In order to increase the channel access possibility, multiple offsets configuration on the time domain is a promising candidate. In Fig1, within a given CG periodicity, UE is configured with three different offsets (as an example). By UE performing CCA at given positions, some benefits can be achieved: 1) economic power consumption; 2) less impact on NR existing specs. Compared with a configured CCA window, UE only needs to sense the channel for a certain period at each configured position instead of continuous sensing within the CCA window. As for the impact on specs, instead of introducing the concept of channel sensing window, slight modification on the bit field of time offset configuration can fulfill the requirements.

Proposal 1: Support multiple time offsets configuration to increase the flexibility on the time-domain resource allocation for NR-U configured grant operations.

Fig1. Multi-offsets configuration for Configured Grants

HARQ Enhancements

FeLAA has supported UE selected HARQ parameters in AUL transmission with asynchronous manner [3]. Thanks to AUL-UCI, HARQ related parameters (i.e., HARQ ID, new data indicator, and redundancy version) can be selected by UE itself, and sent back to eNB together with PUSCH. This procedure could dramatically reduce the latency when UE tries to monitor granted DCI and access channel in each HARQ occasion.

According to [4], the association between HARQ process ID and the corresponding resources would be released for achieving a more flexible selection scheme for AUL UEs. As a result of this, UE is capable to select HARQ process ID among 16 possibilities from 0 to 15 independently, and the HARQ process ID pool is configured by higher layer signaling, e.g., RRC.

Proposal 2: UE should select an ID from the higher layer configured HARQ process ID pool in regardless of the resource that it will use for the configured grant transmission.

COT sharing

The ETSI BRAN harmonized standard allows the channel access initiating device to grant an authorization to one or more associated responding devices to transmit on the current operating channel [5]. In FeLAA, regarding initiating device behavior as a baseline, two-way COT sharing has already been supported [6], which means that initiating device (eNB or UE) could share the acquired COT for DL and UL transmission, respectively. However, the agreements emphasize some confinements for AUL procedure:

- The DL transmission duration is limited to a partial ending subframe of up to 2OS length within the UE acquired COT, and only DL control information can be included due to very limit DL transmission duration.

- The last symbol of the AUL burst has to be dropped with the corresponding AUL transmission duration informed to eNB by AUL-UCI in order to create enough gap for UL to DL switching

- Only single UL to DL and/or DL to UL switching is supported.

For NR-U operation, due to the flexible slot structure and dynamic format indication, the aforementioned confinements could be released more or less. First, the DL occasion within UE shared COT is capable to transmit PDCCH and associated PDSCH to multiple UEs in order to fully utilize unlicensed resources. Then, due to the enhanced capability of UE and gNB in NR, there seems to be no need to drop any last symbols just for creating gap, and even sometimes device can skip CCA if the sensing gap <=16us.  Finally, similar as the description in [7], multiple switching points can bring more benefits for URLLC application in terms of quick DL feedback and SUL and AUL adaptation within the shared COT.

Proposal 3: DL to UL and UL to DL COT sharing for configured grant should refer to the general case of COT sharing with considerations on enhanced design for AUL-UCI and AUL-DFI.

K repetition

Since configured grant transmission is designed for the use case of URLLC, ultra-reliability as one of the crucial features, does not only rely on the conventional HARQ procedure, but also depends on the transmission repetition, a new feature dedicatedly designed for NR. Compare to this, another advantage for repetition is the collision avoidance with frequency hopping among multiple UEs, which could also provide frequency selective gain. To this end, such benefits promote the discussion on the feasibility of K inheritance for NR-U.

From the reliability viewpoint, it is very worth including repetition in NR-U configured grant operation, since the HARQ procedure will introduce extra latency due to its mechanism and LBT. However, limited COT duration exposes a threat on the degraded efficiency of time-domain resource utilization. Considering the COT selection in terms of PUSCH starting and ending symbol attributes to the UE itself, inappropriately configured repK would lead to inefficient time-domain resource utilization. In other words, transmission repetition might possibly occupy the majority of the COT.

From the view of collision avoidance, LBT has been agreed as a global harmonized channel access mechanism, with which interference from non-transmitting nodes is alleviated to the most extent. As such, repetition aiming for avoiding frequency interference seems not that much helpful for NR-U. In addition, PUSCH frequency hopping will also increase the complexity for both UE and gNB on data preparation and decoding, respectively. At this stage, it is therefore envisioned to study the benefits by utilizing repetition in NR-U.

Proposal 4: Study the benefits to support repetition for NR-U configured grant.

Conclusions

In this contribution, based on the above discussion we have the following proposals:

Proposal 1: Support multiple time offsets configuration to increase the flexibility on the time-domain resource allocation for NR-U configured grant operations.

Proposal 2: UE should select an ID from the higher layer configured HARQ process ID pool in regardless of the resource that it will use for the configured grant transmission.

Proposal 3: DL to UL and UL to DL COT sharing for configured grant should refer to the general case of COT sharing with considerations on enhanced design for AUL-UCI and AUL-DFI.

Proposal 4: Study the benefits to support repetition for NR-U configured grant.

References

- Chairman’s Notes, RAN WG1 #93 meeting, May 2018.

- Chairman’s Notes, RAN WG1 #90 meeting, August 2017.

- Chairman’s Notes, RAN WG1 #90bis meeting, October 2017.

- R1-1806547, Potential enhancements to NR configured grants to support unlicensed operation, Intel

- ETSI EN 301 893 V2.1.0, Mar, 2017.

- Chairman’s Notes, RAN WG1 #92 meeting, February 2018.

- R1-1806567, Considerations on Configured Grant on NR-U, Sony