3GPP TSG RAN WG1 Meeting #94                            R1-1808633

Gothenburg, Sweden, August 20th – 24th, 2018

Source:           ZTE

Title:              Consideration on scheduling enhancement for MTC

Agenda item:      6.2.1.3

Document for:     Discussion and Decision

- Introduction

In RAN #80 meeting, new WID RP-181450 on Rel-16 enhancements for MTC was agreed [1]. One of the objectives is scheduling enhancement.

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2]

- Enhancement of SPS can be discussed.

In this contribution, we discuss the scheduling enhancement for MTC.

- Discussion on with or without DCI scheduling

In some use cases, scheduling multiple DL/UL transport blocks can save signaling overhead, reduce the scheduling delay and improve the transmission efficiency.

Two options can be considered for scheduling multiple DL/UL TBs:

- Option 1: scheduling multiple TBs with DCI

- Option 2: scheduling multiple TBs without DCI

For Option 1, scheduling multiple TBs with DCI can significantly reduce the overhead in some scenarios which is beneficial for improving the transmission efficiency. Option 2 means SPS scheduling with multiple TBs, which is shown as method B in Figure 1. Generally, SPS is often used for periodic service and the scheduled packets are small. For SPS, if there is a need to schedule large packet, larger TBS (method C) or shorter SPS scheduling interval (method D) can be considered without specification impact. In this case, scheduling multiple TBs (method B) is not a good choice.

Figure 1 Illustration of SPS, SPS with multiple TBs, SPS with single TB, and SPS with shorter period

Observation 1: Compared with the legacy SPS, scheduling multiple TBs without DCI shows no obvious gain.

Proposal 1: Compared with scheduling multiple DL/UL transport blocks without DCI, study on scheduling multiple DL/UL transport blocks with DCI should be prioritized.

- Issues on scheduling multiple TBs with DCI for unicast

Issues related to specifying scheduling multiple TBs is mainly on DCI design and feedback mechanism.

  - DCI design issues

For DCI design of scheduling multiple TBs, the following aspects should be considered.

- Number of TBs

In Rel-13 FDD MTC, 8 HARQ processes were supported in CE Mode A and 2 HARQ processes were supported in CE Mode B. In order to support higher data rate, 10 HARQ processes were supported in downlink in CE Mode A for Rel-14 HD-FDD UEs. The maximum number of TBs scheduled by one DCI is limited by the maximum HARQ processes number. If the number of TBs is large, multi-TBs scheduling saves DCI signaling overhead. However, the transmission efficiency may be influenced by the feedback mechanism. For CE Mode B, there are only 2 HARQ processes. The maximum number of TBs shall be configured as 2. For CE Mode A, the maximum number of TBs scheduled by one DCI can be configured from set {2, 4, 6, 8, 10}

When scheduling multiple TBs is configured for a UE, single TB scheduling also should be supported to fully adapt to different use cases. Dynamic switching between multi-TBs scheduling and single-TB scheduling can be considered. Or, for multi-TBs scheduling, the minimum number of TBs can be equal to 1.

- Scheduling pattern

Mixed scheduling and non-mixed scheduling can be considered for scheduling multiple TBs. Mixed scheduling means the new transmission TB and the retransmission TB can be scheduled by one DCI. Non-mixed scheduling means the scheduled TBs are either new transmission or retransmission.

The mixed scheduling allows to transmit a new TB immediately, which can improve the transmission efficiency compared with non-mixed scheduling. For example, one of 2 HARQ processes is on the retransmission state. For non-mixed scheduling, the new transmission can start only after finishing retransmission. But for mixed transmission, the new transmission can be scheduled together with retransmission. Mixed scheduling saves the number of PDCCHs and improve the transmission efficiency. However, the DCI design of mixed scheduling would be more complicated than that of non-mixed scheduling.

For CE Mode B, considering the maximum number of HARQ processes is 2, the indication may be simple and signaling overhead may be small, the mixed scheduling can be considered to improve the transmission efficiency. For CE Mode A, the tradeoff between performance and DCI overhead should be considered.

- Resource assignment

For multiple TBs, if frequency domain location of each TB can be independently indicated, it can bring additional frequency diversity gain and scheduling flexibility. However, more DCI overhead is needed. If multiple TBs are at the same frequency location, frequency location filed can be reused for all the TBs scheduled by one DCI.

Time domain location of multiple TBs may be located in continuous or discontinuous valid subframe. The extra overhead may be needed to indicate the PDSCH location for the discontinuous case. Only supporting continuous time domain location for multiple TBs scheduled by one DCI can save the DCI overhead.

- Modulation and coding scheme

For mixed scheduling or non-mixed scheduling, new transmission and retransmission may have different BLER requirement, which may influence the DCI field design of resource assignment, MCS, and repetition number.

The TBS is determined by the DCI fields of MCS and Resource block assignment. MCS field is also related to the modulation order determination. For multi-TBs scheduling, if the TBS for each TB is indicated respectively, the DCI overhead would be much higher.

Besides the aspects discussed above, there are some other issues related to the DCI design, such as the redundancy version and TPC. For mixed scheduling, different TB may need different redundancy version and TPC filed according to its transmission state (new transmission or retransmission). For non-mixed scheduling, same redundancy version and same TPC can be used for all TBs because of the same transmission state.

For multi-TBs scheduling, a unified solution can be applied for DL and UL. But for CE Mode A and Mode B, the above issues should be considered separately.

Proposal 2: For DCI design of scheduling multiple TBs, details such as the number of TBs, scheduling pattern, resource assignment and MCS should be studied.

- A unified solution can be applied for DL and UL multi-TBs scheduling.

- DCI design for CE Mode A and Mode B should be considered separately.

  - Feedback mechanism

The feedback of multi-TBs PUSCH scheduling can be indicated by next UL Grant. Feedback mechanism of PDSCH can be divided into two types: bundled feedback and independent feedback. As shown in Figure 2, bundled feedback means the feedback of multiple TBs is transmitted on one subframe with one or multiple bits while independent feedback means the feedback of multiple TBs is separately transmitted on different subframe for each TB.

Figure 2 Illustration of bundled feedback and independent feedback

The bundled feedback mechanism is suitable for non-mixed scheduling. The independent feedback mechanism can be used for mixed scheduling. These two types of feedback mechanism can be considered for scheduling multiple DL TBs in Rel-16.

For half duplex UE, there is a gap between PDSCH and PUCCH in time domain. If independent feedback mechanism is configured for half duplex UE, the resource occupied by the gap will be large. So the bundled feedback mechanism is the better choice for half duplex UEs and the independent feedback mechanism is more suitable for full duplex UEs.

Proposal 3: For scheduling multiple DL TBs for unicast, bundled feedback and independent feedback can be considered.

- Feedback for half duplex UEs and full duplex UEs should be considered separately.

- Issues on scheduling enhancement with DCI for SC-PTM

For MTC, SC-PTM has been supported from Rel-14. Different from unicast, since feedback is not needed for SC-PTM, scheduling multiple TBs for SC-PTM only needs to study the DCI design. For DCI design for multi-TBs scheduling for SC-PTM, similar to that of unicast the number of TBs, and frequency/time domain resource assignment should be considered. Due to lack of retransmission, only non-mixed scheduling is considered for SC-PTM case. For scheduling multiple TBs for SC-PTM, DCI fields such as resource block assignment, MCS, repetition number and DCI subframe repetition number can be reused. Besides, the number of TBs also needs to be indicated.

Proposal 4: For scheduling multiple TBs with one DCI for SC-PTM, the number of TBs needs to be indicated.

- Conclusions

In this contribution, we have discussed the scheduling enhancement for MTC. We make the following observations and proposals:

Observation 1: Compared with the legacy SPS, scheduling multiple TBs without DCI shows no obvious gain.

Proposal 1: Compared with scheduling multiple DL/UL transport blocks without DCI, study on scheduling multiple DL/UL transport blocks with DCI should be prioritized.

Proposal 2: For DCI design of scheduling multiple TBs, details such as the number of TBs, scheduling pattern, resource assignment and MCS should be studied.

- A unified solution can be applied for DL and UL multi-TBs scheduling.

- DCI design for CE Mode A and Mode B should be considered separately.

Proposal 3: For scheduling multiple DL TBs for unicast, bundled feedback and independent feedback can be considered.

- Feedback for half duplex UEs and full duplex UEs should be considered separately.

Proposal 4: For scheduling multiple TBs with one DCI for SC-PTM, the number of TBs needs to be indicated.

References

- 3GPP, RP-181450, New WID on Rel-16 LTE-MTC, Ericsson, RAN #80