3GPP TSG RAN WG1 Meeting #94								    R1-1809085

Gothenburg, Sweden, Aug 20th – 24th, 2018

Agenda Item:	7.2.2.1

Source:	InterDigital Inc.

Title:	Simulation and Calibration for NR-U Outdoor Sub-7GHz

Document for:	Discussion and Decision

# Introduction

In RAN1 #93 meeting [1], it was agreed to perform simulation calibration for the following outdoor sub-7GHz scenarios.

Agreement:

For sub7 GHz outdoor scenario, adopting the following

    - Macro deployment with ISD=200×A meters

  - Each operator randomly drops 1 micro-layer TRP within each macro cell sector with minimum distance between micro-layer TRPs equals 57.9×A meters

  - Independent dropping between two operators

    - Use 10 meters as the inter-operator micro-layer TRP minimum distance

    - For the inter-operator, micro-layer TRP maximum distance

      - Outdoor scenario 1: 30

      - Outdoor scenario 2: No limit as long as the TRP is within the macro cell

  - UE randomly dropped within macro cell sector with a minimum serving cell RSSI of -82dBm

  - All UEs dropped outdoor

  - Try A>=1 and find the A that satisfies serving cell received power distribution satisfies (10+X)% to (15+X)%] UEs below -72dBm

  - Other parameters follow the table below

| Parameters | Outdoor Sub-7GHz |
| --- | --- |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline, 80MHz optional |
| Number of carriers | 1 |
| Number of users per operator | 5 per gNB per 20MHz |
| SCS | To be reported together simulation results |
| Channel Model | NR UMi street canyon |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Min serving cell rx power for UE dropping | -82dBm |
| UE receiver | MMSE-IRC as the baseline receiver |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array config | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ Optional Tx/Rx: (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | Use 36.889 Table A.1.1. Note: Results based on the mixed traffic models can be used to determine the design. |
| UE/STA to UE/STA link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |
| gNB to gNB link pathloss model | Directly use UMi street canyon pathloss model with proper d_3D with UMi street canyon LOS probability |

In this contribution, we present our simulation results for above outdoor sub-7GHz scenarios.

# Outdoor Sub-7GHz Simulation Calibration

Figure 1 and Figure 2 show the serving cell RSSI for scenarios 1 and 2, respectively, for a veriety of A values. As seen serving cell RSSI statistics are very similar for both scenarios 1 and 2. Results for both scenarios indicate that for RSSI<-72dBm, when the value of A increases the value of the serving cell RSSI percentile increases.  Moreoever, for the range of 1.2<A<1.4 the desired serving cell RSSI percentile of %10-%15 is achieved.

| Figure 1. Serving cell RSSI for scenario 1 | Figure 2. Serving cell RSSI for scenario 2 |
| --- | --- |

Figure 3 and Figure 4 provide max AP-AP RSSI for scenarios 1 and 2, respectively. For scenario 1, it is more likely that the closest AP is the one of the other operator in the same sector, which is located 10~30m away.  Therefore the max AP-AP RSSI for different A values are almost identical.  However, for scenario 2 the closest AP is more likely to be further away as site distance A increases.  Hence, as the value of A increases  the max AP-AP RSSI percetile increases for RSSI<-72dBm.  A=1.6 provides around 13% of APs with RSSI under -72dBm.

| Figure 3. Max AP-AP RSSI for scenario 1 | Figure 4. Max AP-AP RSSI for scenario 2 |
| --- | --- |

Observation 1: Based on our simulation results, A=1.2 yields %10-%15 of serving cell RSSI<-72dBm for both scenario 1 and scenario 2.

Proposal 1:  We believe it is best to choose one A value for both scenarios. We recommend A=1.2 and X=0 (corresponding to %10-%15 of serving cell RSSI<-72dBm) for both scenario 1 and scenario 2.

# Summary

Our simulation results for both scenarios are provided which show for RSSI<-72dBm, when the value of A increases the value of the serving cell RSSI percentile increases.  Moreoever, for the range of 1.2<A<1.4 the desired serving cell RSSI percentile of %10-%15 is achieved.

Observation 1: Based on our simulation results, A=1.2 yields %10-%15 of serving cell RSSI<-72dBm for both scenario 1 and scenario 2.

Proposal 1:  We believe it is best to choose one A value for both scenarios. We recommend A=1.2 and X=0 (corresponding to %10-%15 of serving cell RSSI<-72dBm) for both scenario 1 and scenario 2.

# References

- Chairman’s notes, 3GPP TSG RAN WG1#93, Busan, Korea, May 21st – 25th 2018