3GPP TSG-RAN WG1 Meeting #94	R1-1809264

Gothenburg, Sweden, August 20th  – 24th , 2018

Source:	Ericsson

Title:	IMT-2020 self-evaluation: Mobility evaluations for LTE

Agenda Item:	7.2.7.2

Document for:	Discussion

# Introduction

In this contribution, we show evaluation results for LTE regarding the IMT-2020 mobility requirements. After a brief review of the requirements and evaluation methodology, system-level SINR distributions and link-spectral efficiency results are provided. The results indicate that that the IMT-2020 requirements can be met.

# The Requirement and the evaluation methodology

Table 1 list IMT-2020 requirements [1] on spectral efficiency in mobility scenarios. Each test environment further contains configurations with different carrier frequencies, 4, 30 and 70GHz for Indoor Hotspot, 4GHz and 30GHz for Dense Urban, and 700MHz and 4GHz for Rural. For each test environment, the requirement needs to be reached for at least one configuration.

The evaluation methodology in [2] involves first generating median (50th percentile) uplink SINR from system-level simulations of the test environments above. Link-level simulations for the corresponding test environments are then used to verify that the required normalized traffic channel link data rates can be reached at the median SINR values.

The requirements are for the uplink, but proponents are encouraged to evaluate also the downlink.

Table 1	Traffic channel link data rates normalized by bandwidth (from [1])

| Test environment | Normalized traffic channel link data rate (Bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5 | 10 |
| Dense Urban – eMBB | 1.12 | 30 |
| Rural – eMBB | 0.8 | 120 |
|  | 0.45 | 500 |

# System evaluations – median SINR values

This section presents SINR distributions and median SINR values for the evaluated test environments.  Models and assumptions are according to the ITU guidelines [2]. It should be noted that a rather basic system configuration is assumed, not utilizing the full antenna configurations allowed, and with power control and scheduling parameters not specifically targeting high median SINR values, A selection of parameters is presented in

Table 2	Models and assumptions

| Parameter | Indoor hotspot | Dense urban | Rural |
| --- | --- | --- | --- |
| Frequency | 4GHz | 4GHz | 700MHz |
| BS antenna | 8x8x2 | 8x8x2 | 8x4x2 |
| UE antenna | Isotropic | Isotropic | Isotropic |
| Beamforming and transmission scheme | Wideband eigen beamforming, SIMO | Wideband eigen beamforming, SIMO | Wideband eigen beamforming, SIMO |
| Power control | SNR target 15dB alpha = 1.0 | SNR target 10dB alpha = 1.0 | SNR target 10dB alpha = 1.0 |
| BW allocation | Full system BW | Full system BW | 10 resource blocks |

SINR distributions for downlink and uplink for the different test scenarios, and for propagation model A and B, are presented in Figure 1 - Figure 4. Downlink and uplink median SINR values are summarized in Table 3.

Figure 1. Downlink and uplink SINR distributions for the indoor hotspot scenario with one sector per site

Figure 2. Downlink and uplink SINR distributions for the indoor hotspot scenario with three sectors per site

Figure 3. Downlink and uplink SINR distributions for the dense urban scenario

Figure 4. Downlink and uplink SINR distributions for the rural scenario

Table 3	Downlink and uplink median SINR values

| Test environment | Median downlink SINR [model A / B] | Median uplink SINR [model A/B] |
| --- | --- | --- |
| Indoor Hotspot, 1 sector/site | 14.5 / 14.1dB | 14.2 / 13.8dB |
| Indoor Hotspot, 3 sectors/site | 11.0 / 10.7 dB | 14.8 / 14.5dB |
| Dense Urban | 13.0 / 12.8dB | 8.6 / 8.8dB |
| Rural | 14.9 / 15.3dB | 8.6 / 8.6dB |

# Link results – required SNR values

In the presented link level evaluations, due to the high doppler and low SNR working point, a conservative approach was taken to link adaptation; potential gains of following fast channel variations and SU-MIMO using dense CSI feedback are not reflected. A fixed bandwidth corresponding to roughly 10 MHz was assumed but as requirements are set in the spectral efficiency domain, this may be of secondary importance. The required SINR for reaching the spectral efficiency target should consequently be from the point in the curves where the throughput matches the target spectral efficiency. Reference signals, channel codes, etc. were implemented according to the approved RAN1 specification (with overhead accounted for). Higher layer protocols, including MAC, PDCP, RLC were not included in the evaluations. More detailed simulation assumptions are provided in Section 4.1.

## Models and Assumptions

TDL models [2] are used for the different scenarios according to Table 4 where delay spreads and K-factors for line-of-sight (LOS) channels are chosen according to [4].

Table 4 Test environment, carrier frequency, and corresponding channel model parameters as used in the evaluations

| Test environment | Carrier frequency [GHz] | TDL channel | Delay spread [ns] | K-factor [dB] |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB, NLOS | 4 | TDL-i (TDL-A) | 39 | - |
| Indoor Hotspot – eMBB, LOS | 4 | TDL-iv (TDL-D) | 20 | 13.3 |
| Dense Urban – eMBB, NLOS | 4 | TDL-iii (TDL-C) | 363 | - |
| Dense Urban – eMBB , LOS | 4 | TDL-v (TDL-E) | 93 | 22 |
| Rural – eMBB, NLOS | 0.7 | TDL-iii (TDL-C) | 37 | - |
| Rural – eMBB , LOS | 0.7 | TDL-v (TDL-E) | 32 | 22 |
| Rural – eMBB , NLOS | 4 | TDL-iii (TDL-C) | 37 | - |
| Rural – eMBB, LOS | 4 | TDL-v (TDL-E) | 32 | 22 |

The layer-one configurations reflect a FDD deployment, where  control channel overhead for UL/DL and DMRS overhead is accounted for. For TDD, guard-symbols should be added to enable DL/UL switching.  It is here assumed that not more than an additional 2 guard-symbols per DL/UL switching is needed for this purpose.

Table 5	Simulation parameters for 700 MHz evaluations

| Numerology, waveform | Subcarrier spacing 15kHz, CP-OFDM |
| --- | --- |
| Codec | LTE turbo codec including HARQ. |
| Transmission rank | 1, fixed |
| DMRS | 2 symbol DMRS (front loaded and one additional) with configuration type 1, no FDM with data and full power utilization in DMRS symbols. |
| Antenna | Tx1, Rx2 |
| Other overhead | 1 symbol control overhead per 14 symbols, one directional data allocation |
| Channel estimator | Practical, slot based |
| Allocation | 14 symbol slots, with 52 RB allocated |

## Numerical evaluations

In this section evaluation results of layer-one throughput are presented for the different Test environments (Indoor Hotspot/Dense Urban/Rural scenario) LOS and NLOS channels and for carrier frequencies selected from {700 MHz, 4GHz}, depending on the scenario. The required SNR values to meet the normalized data rate targets are summarized in Table 3.

As noted before, it is sufficient that one configurations per Test environment fulfils the IMT2020 requirements. Thus, here we focus on the best-performing configuration per Test environment for without and with DL/UL switching over-head; in Table 9, the Required SNR is noted for cases without and with frequent DL/UL switching over-head and for LOS as well as NLOS.

Table 9 Required SNR values

| Test environment | Requirement | Required uplink SNR (NLOS/LOS) | Required downlink SNR (NLOS/LOS) |
| --- | --- | --- | --- |
| Indoor Hotspot – eMBB | 1.5bps/Hz at 10km/h | 11.2/7 dB | 10/3.3 dB |
| Dense Urban – eMBB | 1.12bps/Hz at 30km/h | 8.4/4.2 dB | 7/2.4 dB |
| Rural – eMBB | 0.8bps/Hz at 120km/h | 6.3/1.8 dB | 6.1/-1 dB |
|  | 0.45bps/Hz at 500km/h | 4.1/-0.8 dB | 13/3.5 dB |

Figure 5	Uplink (left) and downlink (right) layer 1 throughput as a function of SNR for the Indoor Hotspot scenario at 4GHz.

Figure 6 Uplink (left) and downlink (right) layer 1 throughput as a function of SNR for the Dense Urban scenario at 4.0GHz.

Figure 7	Uplink (left) and downlink (right) layer 1 throughput as a function of SNR for the Rural scenario at 700 MHz and at 120 km/h.

Figure 8 Uplink (left) and downlink (right) layer 1 throughput as a function of SNR for the Rural scenario at 700 MHz and at 500 km/h.

# Summary

The required SINR levels for reaching the spectral efficiency requirements and the and achieved median SINR values are given by Table 10 below.

Table 10	Required and achieved SNR values

| Test environment | Requirement | Required downlink SNR (NLOS/LOS) | Achieved median downlink SINR [model A/B] | Required uplink SNR (NLOS/LOS) | Achieved median uplink SINR [model A/B] |
| --- | --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB, 1sector/site | 1.5bps/Hz at 10km/h | 10/3.3 dB | 14.5dB / 14.1dB | 11.2/7 dB | 14.2dB / 13.8dB |
| Indoor Hotspot – eMBB, 3 sectors/site | 1.5bps/Hz at 10km/h | 10/3.3 dB | 11.0dB / 10.7 dB | 11.2/7 dB | 14.8dB / 14.5dB |
| Dense Urban – eMBB | 1.12bps/Hz at 30km/h | 7/2.4 dB | 13.0dB / 12.8dB | 8.4/4.2 dB | 8.6dB / 8.8dB |
| Rural – eMBB | 0.8bps/Hz at 120km/h | 6.1/-1 dB | 14.9dB / 15.3dB | 6.3/1.8 dB | 8.6dB / 8.6dB |
|  | 0.45bps/Hz at 500km/h | 13/3.5 dB | 14.9dB / 15.3dB | 4.1/-0.8 dB | 8.6dB / 8.6dB |

Observation: The achieved median SINR values are higher than the required SNR values. Hence, the IMT-2020 mobility requirements are fulfilled.

# References

[1] ITU-R Report M.2410, “IMT-2020 Minimum Requirements related to technical performance for IMT-2020 radio interfaces”, November 2017

[2] ITU-R Report M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, December 2017

[3] R1-1720952, “IMT-2020 self-evaluation: Mobility evaluations for NR”, 3GPP TSG-RAN WG1 #91, Ericsson

[4] R1-1805643 “Summary of discussion on mobility evaluation method and parameters”, Huawei, 3GPP TSG RAN WG1 Meeting #92Bis, Sanya, China, April 16 – 20, 2018