3GPP TSG-RAN WG1 Meeting #94	R1-1809271

Gothenburg, Sweden, August 20th  – 24th , 2018

Source:	Ericsson

Title:		IMT-2020 self-evaluation: Peak data rate and peak spectrum efficiency evaluations for LTE

Agenda Item:	7.2.7.2

Document for:	Discussion

# Introduction

In this paper we show evaluation results of LTE w.r.t. peak data rate, peak spectrum efficiency, and bandwidth. The ITU [1] targets w.r.t. above metrics are summarized in the table below.

Table 1: ITU and 3GPP targets on peak data rate, peak spectral efficiency, and bandwidth

| Performance Measure | Requirement |
| --- | --- |
| Peak data rate | DL: 20 Gb/s UL: 10 Gb/s |
| Peak spectral efficiency | DL: 30 bps/Hz UL: 15 bps/Hz |
| Bandwidth | At least 100MHz, up to 1GHz for higher frequency bands |

# Bandwidth

LTE supports carrier aggregation of up to 32 carriers of 20MHz each, so the maximum supported bandwidth is 32 x 20MHz = 640MHz.

Observation 1: LTE supports a maximum bandwidth of 640MHz, which is higher than the ITU requirement of 100MHz. LTE fulfils the bandwidth requirement.

# Peak data rate and spectral efficiency

Downlink and uplink peak data rates and spectral efficiencies are calculated according toTable 2. For the downlink MBSFN subframes are used, and it is taken into account that some subframes carry P/S-SS. The final result is an average over the subframe types, weighted by their occurrence. Eight-layer MIMO based on DMRS is assumed. A minimum CRS overhead still remains. It is assumed that 1024 QAM is used and the highest code rates is 0.93. A single-symbol control region is also assumed. For the uplink four-layer MIMO is used with 256QAM and a maximum code rate of 0.93. Each carrier uses two RBs for PUCCH.

Observation 2: The downlink reaches a peak data rate of 25Gb/s and the uplink reaches 13.5Gb/s. These are above the IMT-2020 targets of 20 and 10Gb/s respectively. LTE fulfils the IMT-2020 targets on peak data rate.

Observation 3: The downlink reaches a peak spectral efficiency of 39bps/Hz and the uplink reaches 21bps/Hz. These are above the IMT-2020 targets of 30 and 15bps/Hz respectively. LTE fulfils the IMT-2020 targets on peak spectral efficiency.

Table 2: Downlink and uplink peak datarates and peak spectral efficiencies.

|  | MBSFN subframe | P/S SS subframe | Ordinary subframe | Downlink Average |  | Uplink |
| --- | --- | --- | --- | --- | --- | --- |
| Nominal BW [MHz] | 640 | 640 | 640 |  |  | 640 |
| Relative spectrum allocation | 0,9 | 0,846 | 0,9 |  |  | 0,882 |
| Occupied BW [MHz] | 576 | 541,44 | 576 |  |  | 564,48 |
| Subcarrier spacing [kHz] | 15 | 15 | 15 |  |  | 15 |
| Number of subcarriers | 38 400 | 36 096 | 38 400 |  |  | 37 632 |
| Number of subcarriers per RB | 12 | 12 | 12 |  |  | 12 |
| Number of RBs | 3200 | 3008 | 3200 |  |  | 3136 |
| Subframe duration [ms] | 1,00 | 1,00 | 1,00 |  |  | 1,00 |
| Number of symbols per subframe | 14 | 14 | 14 |  |  | 14 |
| Symbol duration [us] | 71,4 | 71,4 | 71,4 |  |  | 71,4 |
| Cyclic prefic [us] | 4,76 | 4,76 | 4,76 |  |  | 4,76 |
| Number of REs per RB | 168 | 168 | 168 |  |  | 168 |
| Number of REs per subframe / slot | 537600 | 505344 | 537600 |  |  | 526848 |
| Symbol rate [Msps] | 537,6 | 505,344 | 537,6 |  |  | 526,848 |
| Number of MIMO streams | 8 | 8 | 8 |  |  | 4 |
| Number of CRS per RB (excl CCH) | 0 | 6 | 6 |  |  | 0 |
| CRS OH (excl CCH region) | 0,000 | 0,036 | 0,036 |  |  | 0,000 |
| Number of DMRS per stream and RB | 3 | 3 | 3 |  |  | 6 |
| Number of DMRS per RB | 24 | 24 | 24 |  |  | 24 |
| DMRS OH per stream | 0,018 | 0,018 | 0,018 |  |  | 0,036 |
| DMRS OH | 0,143 | 0,143 | 0,143 |  |  | 0,143 |
| Number of CCH REs / RB | 12 | 12 | 12 |  |  | 0 |
| CCH OH | 0,071 | 0,071 | 0,071 |  |  | 0,000 |
| Number of data REs per RB | 132 | 126 | 126 |  |  | 144 |
| RS+CCH OH (disjunctive) | 0,214 | 0,250 | 0,250 |  |  | 0,143 |
| Data RE rate [MREs/s] | 422,4 | 379,008 | 403,2 |  |  | 451,584 |
| Modulation order | 10 | 10 | 10 |  |  | 8 |
| Code rate | 0,931 | 0,931 | 0,931 |  |  | 0,931 |
| Number of subframes per frame | 6 | 2 | 2 | 10 |  |  |
| Peak datarate [Mbps] | 25 066 | 25 066 | 25 066 | 25 066 |  | 13 454 |
| Peak spectral efficiency [bps/Hz] | 39,17 | 39,17 | 39,17 | 39,17 |  | 21,02 |

# Conclusion

In this contribution, we evaluate LTE w.r.t. supported bandwidth, peak data rate, and peak spectral efficiency. It is shown that LTE fulfills ITU and 3GPP targets w.r.t. to these three metrics.

Observation 1: LTE supports a maximum bandwidth of 640MHz, which is higher than the ITU requirement of 100MHz. LTE fulfils the bandwidth requirement.

Observation 2: The downlink reaches a peak data rate of 25Gb/s and the uplink reaches 13.5Gb/s. These are above the IMT-2020 targets of 20 and 10Gb/s respectively. LTE fulfils the IMT-2020 targets on peak data rate.

Observation 3: The downlink reaches a peak spectral efficiency of 39bps/Hz and the uplink reaches 21bps/Hz. These are above the IMT-2020 targets of 30 and 15bps/Hz respectively. LTE fulfils the IMT-2020 targets on peak spectral efficiency.

# References

ITU-R Report M.2410, “IMT-2020 Minimum Requirements related to technical performance for IMT-2020 radio interfaces”, November 2017