3GPP TSG RAN WG1 Meeting #94	R1-1809281

# Gothenburg, Sweden, August 20-24, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to indication of UE-to-eNodeB COT sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following was agreed in RAN1#92: - COT Sharing indication (1 bit: indicating if subframe n+X is an applicable subframe for UL to DL sharing) - X is configured by the eNB as part of AUL RRC configuration and 1 < X < 5 The corresponding RRC parameter subframeOffsetCOT-Sharing has also been captured into 36.331. However, the related part is missing from RAN1 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding decription for determination of COT sharing indication into 37.213 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The symbol in which UE-to-eNodeB COT sharing is undefined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <SOME UNCHANGED SUBSECTIONS ARE OMITTED IN THIS FILE >

### 4.1.3	Channel access procedure for transmissions including PDCCH and not including PDSCH

If a PUSCH transmission in subframe n indicates COT sharing, an eNB may transmit a transmission including PDCCH but not including PDSCH on the same carrier in subframe n + X, where X is given by subframeOffsetCOT-Sharing, immediately after sensing the channel to be idle for at least a sensing interval , if the duration of the PDCCH is less than or equal to two OFDM symbols length and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received.  consists of a duration immediately followed by one slot duration  and includes an idle slot duration at start of . The channel is considered to be idle for  if it is sensed to be idle during the slot durations of .

<SOME UNCHANGED TEXT IS OMITTED IN THIS FILE