3GPP TSG RAN WG1 Meeting #94	R1-1809355

Gothenburg, Sweden, August 20th- 24th, 2018

Agenda Item:	6.1.6.1

Source:	Huawei, HiSilicon

Title:	Text proposal to section 7.1 of TS 36.213 on the configurable HARQ process numbers

Document for:	Discussion and decision

# Introduction

This contribution provides a text proposal to section 7.1 of TS 36.213 to reflect one agreement for LTE HRLLC which was not captured in the latest version [1].

# Discussion

Regarding the DL HARQ processes numbers for LTE HRLLC, the following agreement was achieved:

Agreement:

- The number of HARQ processes for PDSCH repetition applicable when k > 1 can be configured. The value range is 1 to the maximum number of DL HARQ processes.

When the UE is configured with blindSubframePDSCH-Repetitions or blindSlotSubslotPDSCH-Repetitions and when the number of DL transmissions is greater than 1, the HARQ process indicated in the DL assignment shall not exceed the configured number. This clarification shall be captured accordingly in section 7.1 of UE procedure for receiving PDSCH [1]; otherwise, UE may not discriminate whether the indicated HARQ process number is an error case.

Proposal 1: Endorse the following text proposal:

------------------------------------------------Start of Text Proposal to 36.213-----------------------------------------

<Unchanged parts are omitted>

## 7.1	UE procedure for receiving the physical downlink shared channel

Except the subframes indicated by the higher layer parameter mbsfn-SubframeConfigList or by mbsfn-SubframeConfigList-v1250 or by mbsfn-SubframeConfigList-v14xy or by laa-SCellSubframeConfig of serving cell , a UE shall

-	upon detection of a PDCCH of the serving cell with DCI format 1, 1A, 1B, 1C, 1D, 2, 2A, 2B, 2C, or 2D intended for the UE in a subframe, or

-	upon detection of an EPDCCH of the serving cell with DCI format 1, 1A, 1B, 1D, 2, 2A, 2B, 2C, or 2D intended for the UE in a subframe, or

-	upon detection of a PDCCH of the serving cell with DCI format 7-1A, 7-1B, 7-1C, 7-1D, 7-1E, 7-1F, 7-1G intended for the UE in the first slot/subslot of a subframe

-	upon detection of a SPDCCH of the serving cell with DCI format 7-1A, 7-1B, 7-1C, 7-1D, 7-1E, 7-1F, 7-1G intended for the UE in a slot/subslot

decode the corresponding PDSCH in the same subframe/slot/subslot with the restriction of the number of transport blocks defined in the higher layers, unless specified otherwise.

For a given serving cell, if the UE is configured with higher layer parameter blindSubframePDSCH-Repetitions, the UE shall upon detection of a PDCCH/EPDCCH with DCI format 1A with CRC scrambled by C-RNTI in UE-specific search space of subframe n, intended for the UE, decode, starting from subframe n, the corresponding PDSCH transmission in k consecutive DL subframe(s) according to the PDCCH information, where the value of k is determined by the repetition number field in the corresponding DCI. For k>1,

- the UE shall assume the truncated modulation and coding scheme field is interpreted such that the modulation and coding scheme corresponding to DCI format 1A is determined from MCS indices 0 through 15 if the higher layer parameter MCS-restrictionSubframePDSCH-Repetitions is set to ‘1’.

- The UE is not expected that the HARQ process number is equal or greater than numberOfProcesses-SubframePDSCH-Repetitions.

- The UE shall assume the“PDSCH RE Mapping and Quasi-Co-Location Indicator”field is applied to all the k PDSCH data transmissions signaled via PDCCH/EPDCCH with DCI format 2D.

- The UE shall assume all the k PDSCH data transmissions are received in the same resource blocks.

- For TDD cell, if the UE is configured with higher layer parameter subframeAssignment, the k consecutive DL subframes include the k DL subframes or special subframes according to the DL-reference UL/DL configuration.

For a given serving cell, if the UE is configured with higher layer parameter blindSlotSubslotPDSCH-Repetitions, the UE shall upon detection of a PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G with CRC scrambled by C-RNTI in slot/sublot n, intended for the UE, decode, starting from slot/subslot n, the corresponding PDSCH transmission in k consecutive DL slot(s)/subslot(s) according to the PDCCH/SPDCCH information, where the value of k is determined by the repetition number field in the corresponding DCI. For k>1,

- the UE is not expected to receive the PDSCH data transmissions with more than two transmission layers.

- if the k consecutive DL slots/subslots cross two consecutive subframes with different downlink transmission modes, the UE is not expected to receive the PDSCH data transmissions after the former subframe.

- for DCI format 7-1F/7-1G, the UE shall assume the value of the DMRS position indicator field (defined in 3GPP TS 36.212 [4]) is set to ‘0’.

- the UE shall assume the modulation and coding scheme corresponding to DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G is determined from MCS indices 0 through 15 if the higher layer parameter MCS-restrictionSlotSubslotPDSCH-Repetitions is set to ‘1’.

- The UE is not expected that the HARQ process number is equal or greater than numberOfProcesses-SlotSubslotPDSCH-Repetitions.

- the UE shall assume the “PDSCH RE Mapping and Quasi-Co-Location Indicator” field is applied to all the k PDSCH data transmissions signaled via PDCCH/SPDCCH with DCI format 7-1G.

- the UE shall assume all the k PDSCH data transmissions are received in the same resource blocks.

- For FDD cell,

  - the UE may assume that the same precoder applies to all scheduled k PDSCH transmissions.

  - subslot 0 of a subframe is not counted as a DL subslot for k PDSCH transmissions if the CFI value is 2 or 3.

- For TDD cell, the UE may assume that the same precoder applies to those of the k scheduled PDSCH transmissions occur between two adjacent special subframes.

<Unchanged parts are omitted>

------------------------------------------------End of Text Proposal to 36.213-----------------------------------------

# Reference

- 3GPP TS 36.213, (E-UTRA); Physical layer procedure (Release 15); V15.2.0 (2018-06).