3GPP TSG-RAN WG1 #94	R1-1809458

August 20th – 24th, 2018

Gothenburg, Sweden

Agenda item:	7.2.6.2

Source: 	Qualcomm Incorporated

Title: 	UL inter UE Tx prioritization/multiplexing

Document for:	Discussion/Decision

# Abstract

This is a revision of the RAN1#92b contribution [1].

# Introduction

RAN#80[2] provides the following SID on Physical Layer Enhancements for NR URLLC:

“The objective of this study item is to investigate enhancements to URLLC (Ultra Reliable Low Latency Communications), considering both FR1 and FR2 as well as TDD and FDD, with the already existing solutions for NR as the baseline. The study is focusing on the following items:

Establishing the baseline performance achievable with Release 15 URLLC considering the prioritized URLLC use cases identified in the justification section. Besides the baseline Release 15 URLLC performance, the study will investigate the necessary improvement for the prioritized URLLC use cases in the justification section and how to meet the requirements for those use cases in Release 16 with higher requirements, such as:

- Higher reliability (up to 1E-6 level), higher availability, time synchronization down to the order of a few µs where the value can be 1 or a few us depending on frequency range, short latency in the order of 0.5 to 1 ms, depending on the use cases (factory automation, transport industry and Electrical power distribution)

- Relevant development in other work and study items to be taken into account.

URLLC L1 improvements (RAN1) for further improved reliability/latency and for other requirements related to the use cases identified,

- PDCCH enhancements. Study focus on Compact DCI, PDCCH repetition, increased PDCCH monitoring capability

- UCI enhancements. Study focus on Enhanced HARQ feedback methods (increased number of HARQ transmission possibilities within a slot), CSI feedback enhancements

- PUSCH Enhancements. Study focus on mini-slot level hopping & retransmission/repetition enhancements.

- Enhancements to scheduling/HARQ/CSI processing timeline (UE and gNB), (for existing TTI durations)

Enhanced multiplexing considering different latency and reliability requirements (RAN1):

UL inter UE Tx prioritization/multiplexing

Enhanced UL configured grant (grant free) transmissions, with study focusing on improved configured grant operation, example methods such as explicit HARQ-ACK, ensuring K repetitions and mini-slot repetitions within a slot. (RAN1/RAN2)

The following items have been identified to have relationship URLLC but are covered in other study items:

- Multi-TRP transmission

- Mobility improvements for higher reliability

- Beam Management

”

This contribution focuses on the aspects of UL inter UE Tx prioritization/multiplexing. Rel-15 agreements on URLLC and eMBB multiplexing are provided in Appendix A. In particular, a majority of companies agreed to support inter-UE UL multiplexing in the RAN1#92b meeting, although no consensus was reached:

Proposals: support of handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

Yes: QC, IDC, ZTE, vivo, Sharp, OPPO, DCM, CMCC, E///, Fujitsu, Mistubishi, LGE, Panasonic, Sequans, MotM, Lenovo, Nokia, NSB (18)

No: HW, HiSi, CATT, Intel, MTK (5)

Conclusion:

- There is no concensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

# Dynamic resource sharing of eMBB and URLLC on UL

Dynamic multiplexing of eMBB and URLLC transmissions in both time and frequency domains on the downlink was agreed in Rel-15 (see Appendix A). This systems design optimizes the outage capacity of minislot-based URLLC transmissions by exploiting the statistical multiplexing gain over wide bandwidth, and maximizes the capacity of slot-based eMBB transmissions by fully utilizing time and frequency resources. The same design principles are applicable to uplink URLLC and eMBB transmissions. Wideband resources allow more URLLC uplink transmissions to be dynamically FDM’ed in the same minislot, reducing the latency overhead at the UE side and increasing the number of URLLC UEs that can be connected to the network. In contrast, if uplink resources are statically or semi-statically reserved for URLLC, either the overall system utilization is low when too much bandwidth is reserved to URLLC, or the URLLC capacity is significantly reduced when too little bandwidth is reserved.

See the system-level simulation (SLS) results in Section 5 to support the dynamic resource sharing.

# Uplink pre-emption indication (ULPI)

The URLLC traffic on the uplink needs to be scheduled as soon as practically feasible at the minislot level due to the low latency requirement. URLLC transmissions need wideband resources to exploit trunking efficiency and require a fair amount of system resources to achieve high reliability. Due to the different scheduling granularity of eMBB and URLLC transmissions, gNB may reallocate eMBB resources to later URLLC transmissions by suspending ongoing eMBB transmissions. One option is gNB sends an indication to eMBB UEs to suspend transmissions during the scheduled URLLC PUSCH. This is referred to as uplink pre-emption indication (ULPI). Unlike downlink pre-emption indication that can takes place after the completion of eMBB transmissions, ULPI should be sent before the scheduled URLLC PUSCH to allow eMBB UEs to take actions. The monitoring periodicity of ULPI can be aligned with that of URLLC UL scheduling DCIs and should be minislot-based. See Figure 1 for an illustration.

Figure 1. When URLLC PUSCH is scheduled by DCIs in a minislot, eMBB UEs receive uplink pre-emption indication (PI) from gNB to suspend their transmissions during the scheduled URLLC PUSCH.

By preventing eMBB from interfering URLLC transmission on the uplink, the reliability performance of URLLC is improved, less resources are consumed, and the end-to-end HARQ transmission latency is reduced. As a result, more URLLC traffic/UEs can be admitted into the network, leading to higher URLLC capacity. Table I in Section 5 shows the SLS results that the URLLC outage capacity under eMBB suspension via ULPI is higher than that under various eMBB power-control setpoints. Compared to semi-static power control, ULPI allows eMBB transmissions to be suspended only when URLLC PUSCH is scheduled. eMBB UEs can return to normal high-spectral-efficiency operations in the absence of URLLC transmissions. In addition, dynamically changing the transmission power while keeping the phase continuity is much more challenging in practice to achieve than suspend-and-resuming eMBB transmissions over a preemption period.

Observation 1: Uplink pre-emption indication maximizes the URLLC performance by muting eMBB transmissions that interfere with the URLLC ones. Compared to semi-static power control, the eMBB performance is improved since eMBB transmissions are only suspended when URLLC transmission is present.

## ULPI monitoring and processing timeline

eMBB UEs need to monitor ULPI on the minislot level before the scheduled URLLC transmissions. In practical deployments, ULPI-enabled eMBB UEs can dynamically avoid interfering URLLC UEs in the same cell. eMBB UEs not capable of monitoring ULPI may not be able to dynamically multiplex their transmissions with the URLLC ones, losing the full statistical multiplexing gains.

The monitoring and reaction to ULPI takes less processing time than monitoring UL grants and preparing for uplink data transmissions by URLLC UEs, where data preparation is not performed in the former. Hence, the grant processing timeline N2 for URLLC should be sufficient for ULPI processing timeline for eMBB. In Rel-16, ULPI processing can be further reduced from URLLC grant processing timeline N2.

Observation 2: The monitoring and reaction to ULPI, per UE capability, takes less processing time than the processing time N2 during which URLLC UEs monitor UL grants and prepare uplink data transmissions.

## Timing-advance (TA) alignments across UEs

ULPI can be efficiently signalled to eMBB UEs that have ongoing transmissions via GC-PDCCH. In Rel-15, a similar scheme of uplink transmission cancellation signalled by GC-PDCCH is already supported in SFI, where UEs refrain from configured-granted and SRS transmissions when they fall on flexible symbols. In a case where eMBB UEs have different timing advances in large-cell URLLC deployments and may not receive GC-PDCCH in sync, further technical discussions can address it in the same way as SFI monitoring.

Observation 3: The timing alignment of UEs with different TAs for ULPI processing can be done in the same way as SFI which is signalled by GC-PDCCH.

Proposal 1: NR should support uplink pre-emption indication for eMBB and URLLC dynamic multiplexing on the uplink. Monitoring of ULPI by eMBB UEs, per UE capability, should be minislot-based to align with UL URLLC transmissions. ULPI needs to be a current indication scheme and is received by eMBB UEs before the scheduled URLLC PUSCHs.

# The eMBB-and-URLLC multiplexing via uplink power control and ULPI

An alternative proposal to ULPI is reducing the eMBB-to-URLLC intra-cell interference by either boosting the transmission power of URLLC UEs or reducing the transmission power of eMBB UEs, which are discussed below.

## Boosting transmission power of URLLC UEs

The issues with boosting the transmission power of URLLC UEs are as follows.

- The outage cell capacity of the URLLC service is dominated by cell-edge URLLC UEs, which may not have the power headroom for power boosting.

- Increasing transmission power goes against the long-standing practice of controlling the interference-over-thermal (IoT) to achieve stability in cellular networks. In the interference-limited regime where there may or may not be URLLC UEs in neighboring cells, increasing transmission power provides little improvement of system capacity at the cost of high power consumption.

Observation 4: Boosting transmission power of URLLC UEs is infeasible for cell-edge UEs with limited power headroom, and is ineffective in the interference-limited regime for all URLLC UEs.

## Reducing transmission power of eMBB UEs

When URLLC UEs have random traffic, the exact timing of the URLLC traffic is unpredictable. The options of reducing eMBB transmission power include:

- Static or semi-static power reduction. No new eMBB UE capability is needed. But the eMBB performance will be significantly impacted in the long run because power reduction is not needed when there is no URLLC traffic present.

- Dynamic power reduction in the presence of URLLC transmissions. A gNB provides on-demand power control command and the timing of scheduled URLLC transmissions in DCI to the eMBB UEs. ULPI is a special case of dynamic power control where the eMBB transmission power is set to zero during colliding portion of an eMBB PUSCH. More importantly, fully dynamic power control during an eMBB PUSCH is infeasible when phase continuity cannot be maintained across the collided period. New eMBB UE capability is needed here.

In Table I below, the SLS results show the outage URLLC capacity is significantly degraded even when eMBB UEs in the same cell perform power control. The performance degradation of URLLC is mitigated only if eMBB UEs set a very low target received data SNR, which will result in prolonged low throughput and spectral efficiency for eMBB UEs even in the absence of URLLC traffic.

Observation 5: For eMBB UEs, semi-static transmission power reduction to maintain satisfactory URLLC performance significantly impacts the eMBB performance. Fully dynamic transmission power reduction to portions of eMBB PUSCHs that collide with URLLC PUSCHs is infeasible because phase continuity cannot be maintained.

## System-level simulation results

To study the effectiveness of uplink power control and the need of dynamic multiplexing between eMBB and URLLC transmissions on the uplink, we provide the system-level simulation (SLS) results in the following. The detailed SLS assumptions are given in Section 8. We consider one URLLC serving cell with 21 URLLC UEs and one eMBB UE, surrounded by 20 eMBB cells with ISD=200m. eMBB UEs in the neighbouring cells have full-buffer traffic and are scheduled dynamically, and the inter-cell interference is captured in the simulations. In the URLLC serving cell, the eMBB UE has the best path gain among all intra-cell UEs and is not power-limited, and intra-cell interference is captured by using a linear MMSE receiver at the gNB. The eMBB and URLLC UEs in all cells apply open-loop power control with partial pathloss compensation. All eMBB UEs in the neighbouring cells and all URLLC UEs in the serving cell set the target data received SNR to 20dB above thermal. The eMBB UE in the serving cell is power-controlled at different operating points to study the impact of intracell interference from eMBB to URLLC.

Table I shows the cell capacity of URLLC UEs on the uplink, subject to different available frequency resources to URLLC and different target received data SNRs for the eMBB UE in the serving cell. Target received data SNR of “-inf dB” means that all eMBB transmissions are suspended, at least during scheduled URLLC transmissions, in the serving cell. The cell capacity corresponds to the largest traffic load at which all URLLC UEs meet the QoS requirements of 1ms latency and 1e-5 reliability in the serving cell (see an exemplified capacity calculation in [Section 3.2.2, 3]). We observe that the URLLC capacity and the associated spectral efficiency increase with frequency resources across all received data SNR targets of the jamming eMBB transmissions. The associated resource utilization of URLLC transmissions, shown in Table II, is relatively low even when the URLLC traffic load is at the capacity-achieving level. The simulation results confirm our observation that URLLC needs wideband resources, and dynamic multiplexing between eMBB and URLLC is needed to maximize total system resource utilization on the uplink. It is also observed that power control does not solve the problem for URLLC and eMBB dynamic multiplexing. In the BW = 20MHz case, even with P0 = 14dB at the eMBB interferer, the URLLC capacity can drop by a factor of three. FDM-ing URLLC and eMBB is also not a good idea as the capacity drops superlinearly as the URLLC frequency resources are reduced.

In addition, lowering target received data SNR for UEs in good channel condition is essentially performing channel inversion, which significantly handicaps the capacity of the corresponding eMBB system, with and without the presence of URLLC transmissions.

Table I: The URLLC cell capacity under different URLLC frequency resources and target received data SNR values of the eMBB UE in the serving cell.

| Target received data snr of in-cell eMBB UE | URLLC resources 20MHz | URLLC resources 10MHz | URLLC resources 5MHz |
| --- | --- | --- | --- |
| -inf dB (eMBB transmissions are preempted) | 16.13Mbps | 5.38Mbps | 1.08Mbps |
| 2dB | 15.05Mbps | 5.38Mbps | 1.08Mbps |
| 8dB | 11.83Mbps | 4.3Mbps | negligible |
| 14dB | 5.38Mbps | 2.15Mbps | negligible |
| 20dB | 2.15Mbps | negligible | negligible |

Table II: The resource utilization of URLLC transmissions at the maximum URLLC capacity under different URLLC frequency resources and target received data SNR values of the eMBB UE in the serving cell.

| Target received data snr of incell eMBB UE | URLLC resources 20MHz | URLLC resources 10MHz | URLLC resources 5MHz |
| --- | --- | --- | --- |
| -inf dB (eMBB transmissions are preempted) | 63.9% | 41.7% | 17.3% |
| 2dB | 63.8% | 44.5% | 18.6% |
| 8dB | 58.8% | 41.3% | negligible |
| 14dB | 39.4% | 28.1% | negligible |
| 20dB | 25.4% | negligible | negligible |

Observation 6: In intra-cell eMBB and URLLC multiplexing on the uplink, semi-static power control of eMBB UEs significantly degrades the URLLC performance, unless the target received data SNR of eMBB is very low, resulting in significantly degraded eMBB performance.

# Conclusion

Observation 1: Uplink pre-emption indication maximizes the URLLC performance by muting eMBB transmissions that interfere with the URLLC ones. Compared to semi-static power control, the eMBB performance is improved since eMBB transmissions are only suspended when URLLC transmission is present.

Observation 2: The monitoring and reaction to ULPI, per UE capability, takes less processing time than the processing time N2 during which URLLC UEs monitor UL grants and prepare uplink data transmissions.

Observation 3: The timing alignment of UEs with different TAs for ULPI processing can be done in the same way as SFI which is signalled by GC-PDCCH.

Observation 4: Boosting transmission power of URLLC UEs is infeasible for cell-edge UEs with limited power headroom, and is ineffective in the interference-limited regime for all URLLC UEs.

Observation 5: For eMBB UEs, semi-static transmission power reduction to maintain satisfactory URLLC performance significantly impacts the eMBB performance. Fully dynamic transmission power reduction to portions of eMBB PUSCHs that collide with URLLC PUSCHs is infeasible because phase continuity cannot be maintained.

Observation 6: In intra-cell eMBB and URLLC multiplexing on the uplink, semi-static power control of eMBB UEs significantly degrades the URLLC performance, unless the target received data SNR of eMBB is very low, resulting in significantly degraded eMBB performance.

Proposal 1: NR should support uplink pre-emption indication for eMBB and URLLC dynamic multiplexing on the uplink. Monitoring of ULPI by eMBB UEs, per UE capability, should be minislot-based to align with UL URLLC transmissions. ULPI needs to be a current indication scheme and is received by eMBB UEs before the scheduled URLLC PUSCHs.

# References

- R1-1804820, “eMBB and URLLC dynamic multiplexing and preemption indication on the uplink,” Qualcomm Technologies, Inc., 3GPP TSG-RAN WG1 #92b, Sanya, China, April 2018.

- RP-181477, “New SID on Physical Layer Enhancements for NR URLLC,” Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN#80, La Jolla, US, June 2018.

- R1-1610188, “URLLC system capacity and URLLC/eMBB multiplexing efficiency analysis,” Qualcomm, 3GPP TSG-RAN WG1 #86-BIS, Lisbon, Portugal, October 2016.

# System-level simulation assumptions

| Parameters | Urban Macro |
| --- | --- |
| Layout | Single macro layer. Hex. Grid, 21 cells wrap around |
| Inter-BS distance | 200m |
| Carrier frequency | 2GHz |
| System bandwidth | 5, 10, 20MHz (FDD UL) |
| Channel model | 3D UMa |
| Transmission power | BS: 49dBm PA scaled with simulation bandwidth. UE: 23dBm |
| Antenna config | 2 Tx / 2 Rx (X-pol) |
| BS antenna height | 35m |
| BS antenna element gain+connector loss | 8dBi |
| BS/UE receiver noise figure | 5/9 dB |
| Open-loop power control | Target received data SNR=20dB, partial pathloss compensation alpha=0.9. Same for all eMBB and URLLC UEs. The eMBB UE in the serving cell has other target received data SNR values for evaluation. |
| Traffic model | eMBB: full-buffer. URLLC: Poisson with 32-byte packets (FTP3) |
| UE distribution | 21 URLLC UEs and 1 eMBB UE (closet to BS) in the serving cell. Uniformly random drop in a cell with 80% indoor and 20% outdoor. 20 eMBB neighboring cells, each has one eMBB UE. |
| Scheduling algorithm | URLLC: delay-based subband 2x2 SU-MIMO |
| Inter/intra-cell interference | Fully captured with beamforming. Linear-MMSE receiver in the URLLC serving cell |
| Tone spacing/cyclic prefix | 30KHz/NCP |
| Minislot/RTT durations | 2-symbol minislot, 6-symbol RTT |
| HARQ | Incremental redundancy |
| Target reliability | Tx missed deadline + Rx HARQ failure <= 1e-5 |
| Hard latency bound | 1ms |
| Channel estimation | Demod chanEst error is captured in link sim results |
| Control | Overhead is not captured in capacity analysis |

# Appendix A

In RAN#71, the technology study item for 5G new RAT (NR) has been approved. URLLC (ultra-reliable and low latency communication) requirements were discussed in the RAN plenary in June 2016. Between RAN1#86bis and RAN1#93, the following agreements were made regarding URLLC and eMBB multiplexing and mini-slot designs.

Agreements:

- For DL, dynamic resource sharing between URLLC and eMBB is supported by transmitting URLLC scheduled traffic

URLLC transmission may occur in resources scheduled for ongoing eMBB traffic

Agreements:

- For DL, support indication of time and/or frequency region of impacted eMBB resources to respective eMBB UE(s)

  - FFS: Details of  the granularity for impacted region used in the indication

    - e.g., PRB (group)/symbol (group)/mini-slot (group)/CB (group)/TB/Slot

  - The indication is transmitted at one of the following (will be down selected later)

    - during current eMBB TTI

    - after current eMBB TTI

    - during  and after current eMBB TTI

  - The indication is one of the following (will be down selected later)

    - explicit

    - implicit

    - explicit and implicit

Agreements:

- Indication of URLLC transmission overlapping the resources scheduled for an eMBB UE in downlink can be dynamically signaled to the eMBB UE to facilitate demodulation and decoding

  - FFS details

Agreements:

- Indication can be dynamically signaled to a UE, whose assigned downlink resources have  partially been preempted by another downlink transmission, to increase the likelihood of successful demodulation and decoding  of the TB(s) transmitted within the above mentioned assigned resource

- The indication may be used to increase the likelihood of successful demodulation and decoding of the transport block based on the pre-empted transmission and/or subsequent (re)-transmissions of the same TB

Agreements:

- No new physical channel specific for indication of DL resources being preempted by another DL transmission is introduced

  - FFS whether the indication is based on NR-PDCCH or a group common PDCCH

  - FFS location of the indication

  - FFS timing of the indication

Agreements:

      - The minimum periodicity for UE to monitor group common DCI for DL preemption indication is down-selected between

          - Option 1: one slot

          - Option 2: less than a slot

Agreements:

      - The time duration of the reference downlink resource for pre-emption indication equals to the monitoring periodicity of the group-common DCI carrying the pre-emption indication

        - In TDD, at least the semi-statically configured UL symbols are excluded from the reference downlink resource

          - Note: This means the reference downlink resource only includes the DL or unknown symbols given by semi-static configuration within the semi-statically configured time duration of the reference downlink resource.

        - FFS for the handling of reserved resource especially at RE level

Agreements:

      - For minimum monitoring periodicity of pre-emption indication:

        - At least slot level monitoring periodicity of preemption indication is supported

        - FFS to additionally support other cases (e.g. non-slot level monitoring)

Agreements:

      - For slot level monitoring periodicity, UE is not required to monitor preemption indication for a slot in which PDSCH is not scheduled

      - UE is not required to monitor preemption indication in DRX slots

      - UE is not required to monitor preemption indication for the deactivated DL BWP

      - Note: not necessarily all of the above bullets will have spec impacts

Agreements:

- The HARQ timeline for a PDSCH transmission is not affected by preemption indication.

Agreements:

      - No concensus to introduce an explicit RRC configuration for frequency region of the reference downlink resource for pre-emption indication in Rel-15

      - (working assumption) the frequency region of the reference downlink resource for pre-emption indication is the active DL BWP

Agreements:

- A fixed payload size (excluding CRC and potential reserved bits) of the group-common DCI carrying the downlink pre-emption indication (PI), in the format of a bitmap is used to indicate preempted resources within the semi-statically configured DL reference resource

  - The bitmap indicates for one or more frequency domain parts (N>=1) and/or one or more time domain parts (M>=1)

    - There is no RRC configuration involved in determining the frequency or time-domain parts

  - The following combinations are supported and predefined {M, N} = {14, 1}, {7, 2}

  - A combination of {M,N} from this set of possible {M,N} is indicated 1bit by RRC configuration for a UE

Agreements:

- Within a PUCCH group, UE can be configured to monitor group common PDCCH for pre-emption indication for a Scell on a different serving cell

      - One DCI can contain one or more pre-emption indication field(s) corresponding one or more serving cells

        - Each field (14bits bitmap) for one serving cell

      - RRC configures the PI field location in the DCI format that is applied to that cell

Agreements:

- Supported periodicities for slot level preemption monitoring are

  - 1, 2, TBD1, TBD2 slots

Agreements:

- No concensus to support mini-slot level monitoring periodicity of preemption indication in RAN1#91

Agreements:

- Confirm the following working assumption in RAN1#90bis

  - The frequency region of the reference downlink resource for pre-emption indication is the active DL BWP

Agreements:

- Configuration of UE monitoring of preemption indication is per DL BWP

Agreements:

- For the bitmap indication, the time-frequency blocks of the reference DL resource determined by {M, N} ({M, N}={14, 1}, {7, 2} ) are indexed in frequency first manner

  - Note: The reference DL resource is partitioned with M time domain parts and N frequency domain parts.

  - Note: Current TS38.213 needs to be updated according to the above agreement.

Agreements:

- When a PI is detected, the time location of the corresponding reference DL resource (RDR) is determined by:

  - The RDR starts at the 1st symbol of the previous CORESET for PI monitoring and ends right before the current CORESET at which the PI is detected.

Agreements:

- The UE is not expected to take into account a PI detected in a BWP for a PDSCH scheduled in a different BWP of the same serving cell.

Agreements:

- Support 4 as additional PI monitoring periodicity value as part of the RRC configuration for the UE

  - No other additional values in Rel-15

Agreements:

- To adopt the TP in section 3 of R1-1801155 (for Section 11.2 of 38.213)

Agreements:

- Study the options to support dynamic resource sharing between eMBB UL and URLLC UL from different UEs (comparing with existing techniques)

            - Option 1: eMBB UE cancels UL transmission when an indication is detected. Details to be discussed/clarified

              - UE processing timeline for cancelation

              - UE monitoring periodicity

              - Group common or UE specific signalling (including the possibility to use eMBB scheduling DCI)

              - reliability of indication

              - Any impact due to timing advance

            - Option 2: UL power control. URLLC UE transmits over the same resource with eMBB UE transmission. The transmission power for URLLC UL is boosted and/or transmission power for eMBB UL is reduced. Details need to be discussed/clarified

              - Performance impact to eMBB/URLLC transmission

              - How to signal the URLLC transmission power boosting

              - How to signal the eMBB transmission power reduction after UL grant

              - UE monitoring periodicity

              - Processing timeline

              - Feasibility of changing eMBB Tx power during the transmission

              - reliability of indication

              - Any impact due to timing advance

            - Other options including gNB receiver interference cancelation schemes are not precluded

            - Aspects to be included in the study

              - Processing timeline for grant-based procedure for URLLC in UL

              - Applicability of the options to TDD and/or FDD can be studied

              - Cases for GB-based & GF-based

Proposals: support of handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

Yes: QC, IDC, ZTE, vivo, Sharp, OPPO, DCM, CMCC, E///, Fujitsu, Mistubishi, LGE, Panasonic, Sequans, MotM, Lenovo, Nokia, NSB (18)

No: HW, HiSi, CATT, Intel, MTK (5)

Conclusion:

- There is no concensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)