3GPP TSG RAN WG1 Meeting #94	R1-1809477

Gothenburg, Sweden

Aug. 20th – Aug. 24th, 2018

Agenda item:    7.2.2.3.1

Source:             Qualcomm Incorporated

Title:                  DL signals and channels for NR-U

Document for:  Discussion and Decision

# Introduction

In RAN#77 a new SI [1] for NR based access to unlicensed spectrum was approved which was then revised in RAN#80 [5]. The objectives of the SI include the following:

- Study NR-based operation in unlicensed spectrum (RAN1, RAN2, RAN4) including

  - Physical channels inheriting the choices of duplex mode, waveform, carrier bandwidth, subcarrier spacing, frame structure, and physical layer design made as part of the NR study and avoiding unnecessary divergence with decisions made in the NR WI

    - Consider unlicensed bands below 7GHz

    - Consider similar forward compatibility principles made in the NR WI

  - Initial access, channel access. Scheduling/HARQ, and mobility including connected/inactive/idle mode operation and radio-link monitoring/failure

  - Coexistence methods within NR-based and between NR-based operation in unlicensed and LTE-based LAA and with other incumbent RATs in accordance with regulatory requirements in e.g., 5GHz, 6GHz bands

    - Coexistence methods already defined for 5GHz band in LTE-based LAA context should be assumed as the baseline for 5GHz operation. Enhancements in 5GHz over these methods should not be precluded. NR-based operation in unlicensed spectrum should not impact deployed Wi-Fi services (data, video and voice services) more than an additional Wi-Fi network on the same carrier;

In some regions in the world, a node need to perform a Listen-Before-Talk (LBT) procedure for medium access in unlicensed band to achieve fair coexistence between different nodes, including cellular nodes and WiFi nodes. With the LBT operation, a node cannot guarantee its transmission as in NR.  In addition, the node cannot guarantee the medium access right before the symbol or slot boundary when the medium sensing is asynchronous. These areas introduce the design changes of NR-U on top of NR.

In some unlicensed bands (e.g., 5GHz), there are transmission PSD limitations. For example, the regulation on PSD limitation with 5GHz band is listed below [2]:

- 5150-5350 MHz with TPC: 10dBm/MHz

- 5250-5350 MHz without TPC: 7dBm/MHz

- 5150-5250 MHz without TPC: 10dBm/MHz

- 5470-5725 MHz with TPC: 17dBm/MHz

- 5470-5725 MHz without TPC: 14dBm/MHz

Under the PSD limitation, in order for a node to transmit with higher power, it needs to occupy wider bandwidth. In order to fully utilize the Tx power of a node with PSD limit, the interlace waveform design has been adopted in eLAA UL and MF UL and is also widely discussed in NR-U contributions. In downlink, the interlace design can also bring benefit as the wider bandwidth transmission may introduce reduced coverage and additional inter-cell interference.

# Waveform Design Options for NR-U in SUB-7GHZ Band

To minimize changes with respect to NR, it is desirable to build the waveforms in NR unlicensed on top of NR waveforms.

## SSB Transmission in SUB-7GHZ Band

In RAN1#93 the following was agreed with respect to SSB:

- NR-U should have a signal that contains at least SS/PBCH block burst set transmission

  - FFS: Other channels and signals transmitted together as part of the signal

- The design of this signal should consider the following characteristics specific to unlicensed band operation

  - There are no gaps within the time span the signal is transmitted at least within a beam

    - FFS: Whether any gaps are needed for beam switching and, if needed, their duration

  - The occupied channel bandwidth is satisfied (although this may not be a requirement)

  - Strive to minimize the channel occupancy time of the signal

  - Characteristics that may facilitate fast channel access

The section lists some of the design considerations for SSB transmission in the sub-7Ghz unlicensed band.

## 2.1.1 SSB Numerology in SUB-7GHZ Band

In NR, the initial acquisition and initial access only define 15Khz and 30Khz.  For SSB transmission, due to the PSD limit, the performance of an SSB with 30Khz SCS yields similar performance as an SSB with 60Khz SCS with low delay spread channel. This is shown in Figure 1 and Figure 2.

With 60Khz SCS for SSB, the SSB detection complexity is increased due to the increased SSB transmission bandwidth compared to 30Khz SCS. The additional signalling change may also be required in PBCH when the 60Khz SCS is added for the initial acquisition. On the other hand, the 60Khz SCS for both SSB and data allows easier channel multiplexing from gNB’s transmission. It also allows less time span for SSB burst set transmission, which is more beneficial for DRS transmission with shorter duration for low LBT category (1ms can fit 8 SSBs with 60Khz SCS) as well as the factory automation applications in the unlicensed spectrum.

Figure 1. SSB detection performance with 60Khz SCS

Figure 2. SSB detection performance with 30Khz SCS

Based on the above simulations, we can make the following observations: It is beneficial to support both 30Khz and 60Khz SCS for SSB transmission to consider different use cases. To avoid blind search on receiver processing, a node is provisioned to use a given SCS for SSB acquisition.

Observation 1: SCS for SSB in NR-U SUB-7GHZ BAND should consider the trade-off on the performance, implementation complexity, signaling change on top of NR as well as the LBT and factory automation application. It is beneficial to support both 30Khz SCS and 60Khz SCS for SSB transmissions.

Proposal 1: Introduction of SSB transmission with 60Khz SCS in NR-U on top of 30Khz SCS. A node is provisioned to use a given SCS for SSB acquisition.

### 2.1.2 SSB Multiplexing with RMSI in SUB-7GHZ Band

In RAN1#93 the following was agreed with respect to the initial DL/UL BWP in NR-U:

- Initial active DL/UL BWP is approximately 20MHz for 5GHz band

  - The final value will be quantized to number of PRBs

- Initial active DL/UL BWP is approximately 20MHz for 6GHz band if similar channelization as 5GHz band is used for 6GHz band

- FFS: Initial active DL/UL BWP for other applicable bands, including 60GHz

With 30Khz SCS, the SSB does not occupy the entire initial BWP and it is beneficial to allow RMSI to be multiplexed with SSB in frequency domain. In current NR, only pattern 1 is allowed for FR1, therefore, it is beneficial to introduce FDM between CORESET/PDSCH of RMSI and SSB.

Figure 3 SSB and RMSI multiplexing patter in NR

Proposal 2: Allow FDM between RMSI CORSET/PDSCH and SSB.

### 2.1.3 Impact of LBT on SSB Transmission in SUB-7GHZ Band

### 2.1.3.1 Impact of LBT on SSB Pattern in SUB-7GHZ Band

NR has defined transmission of multiple SSBs along with the SSB index signalling. The transmission of SSBs within a SS burst set is confined to a 5 ms window regardless of SS burst set periodicity. Within this 5 ms window, the max number of possible candidate SS block locations is L where for carrier frequency range up to 3 GHz, L=4; for carrier frequency range from 3 GHz to 6 GHz, L=8; for carrier frequency range from 6 GHz to 52.6 GHz, L=64.

To minimize the change on top of existing NR implementation, the SSB pattern defined in NR for 30Khz can be directly used for NR-U in sub-7GHz band. One example pattern on NR SSB can be shown in Figure 4. When multiple SSBs need to be transmitted, gNB can fill the gap symbols between SSBs with other signalling, such as RMSI, paging, OSI, CSI-RS, etc. Moreover, additional PSS/SSS/PBCH symbols can be transmitted in the gap symbols to increase the one shot detection performance. Note that one shot SSB detection performance is important in NR-U as gNB may not be able to transmit SSB in next DMTC window when LBT does not succeed. When the additional PSS symbols are transmitted, they can use different PSS sequences to avoid the timing confusion. Such enhanced SSB transmission is adopted in MulteFire.

Figure 4 Example of SSB transmission pattern in NR

It is also recommended to use the same SSB pattern for NSA and SA mode to unify the transmit and receiver processing.

Proposal 3: Reuse NR SSB pattern for 30Khz SCS SSB for NSA and SA operation.

### 2.1.3.2 Impact of LBT on SSB Transmission Opportunity in SUB-7GHZ Band

The multiple SSB transmission introduced in NR not only allows beam sweeping, it is also friendly when a gNB cannot gain medium access at the beginning of DMTC window, the gNB may still transmit remaining SSBs. The concept of truncated SSB transmission is illustrated in Figure 5.

Figure 5. Truncated SSB transmission in NR-U

In some cases, gNB may not be able to access the medium for a few slots and the SSB transmission truncation may not guarantee SSB transmissions as needed. In addition, gNB may want to transmit multiple SSBs in sub-7Ghz band for different applications, such as massive MIMO use cases.

In RAN1#93 the following was agreed with respect to the initial access procedure in NR-U:

- Modifications to initial access procedures considering limitations on access to the channel based on LBT

  - Develop techniques to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure

Both floating SSB and cyclic extended SSB transmission can be considered along with the additional signalling on the SSB timing and they are illustrated in Figure 6 and Figure 7. The floating SSB always start with natural SSB index order from LBT access while cyclic extended SSB follows NR SSB index with cyclic extension depending on LBT outcome. We have slight preference on cyclic extended SSB transmission over the floating SSB transmission as the additional signalling to indicate the SSB timing is expected to be less.

Figure 6. Floating SSB transmission in NR-U

Figure 7. Cyclic extended SSB transmission in NR-U

Proposal 4: Consider cyclic extended SSB transmission in NR-U

## Interlace Waveform in SUB-7GHz Band DL

Due to the PSD limitation, in order for a node to transmit with higher power, it needs to occupy wider bandwidth. While the interlace waveform design has been adopted in eLAA UL and MF UL and is also widely discussed in NR-U contributions, in downlink, the general assumption is that gNB could use wideband transmission for better power utilization.

Take the 20Mhz channel as an example. NR defines the nominal RBG size as in Table 1. Assume 30Khz SCS is used, the RBG size is 4 RBs, i.e., 1.44MHz. With PSD limit of 10dBm/Mhz, gNB needs to span the entire 20Mhz to achieve full power 23dBm for transmission. In this case, the PSD is -5.23dBm/RE.

With the introduction of interlace transmission in DL, gNB could transmit on a given interlace for link budget limited UE. In this case the PSD is -0.8dBm/RE on the scheduled interlace. That is, UE is able to achieve better channel estimation with interlace DL transmission, even with RBG based channel estimation.

Table 1. Nominal RBG size P defined in NR

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

One may argue that instead of introducing interlace waveform in DL, gNB could use resource allocation type 1 along with distributed resource allocation to achieve wideband transmission without spanning the entire bandwidth. However, the distributed resource allocation may not be able to guarantee uniform RB distribution with respect to the PSD limit unit (i.e., 1MHz) hence will likely result into power under utilization compared to the carefully designed interlace waveform.

In addition to the increased link budget with the introduction of interlace waveform in DL, the interlace transmission can also help to increase the reuse among different nodes. When both gNBs transmit data on different interlace, they do not cause inter-cell interference. One use case is that even when gNB observes high energy from its neighbour nodes, if it identifies un-used interlace, it can still transmit without affecting the on-going transmission. The information on the un-used interlace may be obtained in the channel usage indicator (CUI) signal.

Proposal 5: Consider interlace waveform for NR-U DL in SUB-7GHZ BAND for better coverage and interference coordination

## Channel Usage Indicator (CUI)

In RAN1#93 the following was agreed with respect to the initial access procedure in NR-U:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

In this section, we discuss on introducing Channel Usage Indicator (CUI) signal to support more accurate channel reuse and SINR trade-off. More details are listed in our companion paper [4].

For the waveform for the CUI, it is preferred to reuse existing NR waveform, instead of having a completely new design. Some potential waveform design options include reusing PDCCH design, or reusing PUCCH design. Between the two options, we believe PDCCH waveform is more suitable due to the following reasons:

The CUI needs to carry some payload information (NAV information and a flag to distinguish transmitter or receiver), so PUCCH formats with 1-2 bits payloads are not helpful.

The CUI waveform needs to be transmitted from both gNB and UE. Since NR UE already support both DFT-s-FDM waveform and OFDMA waveform, it is more natural to use OFDMA waveform for CUI

The CORESET configuration that includes parameters such as bandwidth, time domain length, interleaving or not, different aggregation levels for the DCI is more flexible than the PUCCH waveforms. Thus it is easier to find a good configuration to fit the purpose and match the performance requirement

If PDCCH waveform is picked for CUI transmission, we will need to define a control resource set (CORESET) structure for carrying the CUI. Since the CUI needs to be detected across operators, some coordination is needed on the CORESET configuration.

Proposal 6: Reuse NR PDCCH waveform for CUI transmission from both gNB and UE.

## DL Wake-up Signal (WUS)

In RAN1#93 the following was agreed with respect to the initial access procedure in NR-U:

- Benefits of using a signal that facilitates its detection with low complexity can be investigated including all/part of the following scenarios/use cases:

  - UE power saving

  - Improved coexistence

  - Spatial reuse at least within the same operator network

  - Serving cell transmission burst acquisition

  - FFS: further usage scenarios

In this section, we discuss on introducing wake-up signal for UE power saving.

In NR study item phase, the introduction of wake-up signal has been considered. The main benefit of introducing such mechanism is for UE side power saving. Though eventually, WUS was not further considered in NR licensed design, in NR-unlicensed, we see the importance of WUS increases. This is mainly because, even though a UE is configured to monitor the PDCCH in a slot, if the gNB failed to pass LBT for the PDCCH transmission, the UE monitoring of the whole PDCCH region is wasted. It will be more energy efficient if the UE has some side information on when the gNB is transmitting.

The use cases of WUS in NR-unlicensed include:

- PDCCH monitoring power saving.

- More starting positions for TxOP without significantly impacting power consumption

- Preamble for channel usage indicator transmission

Figure 8. WUS use case A: Wake up signal for PDCCH monitoring

For use case A, WUS helps to skip the PDCCH monitoring for better power saving. The PDCCH monitoring involves multiple computation steps including FFT, channel estimation, de-mapping, and Polar code decoding of multiple decoding candidates. Better battery life can be achieved if these processing steps are conditioned on a simple WUS detection.

Figure 9. WUS use case B: Wake up signal for flexible TxOP starting position indication

In LTE LAA, the TxOP can start at slot level. To support more starting positions within a NR slot, the UE needs to be configured to monitor PDCCH more frequently, which translates to higher power consumption. If a WUS is introduced before the actual TxOP starts, the PDCCH based processing can be skipped when WUS is not detected, which allows more flexible starting positions to be supported without significantly impacting the power consumption.

Figure 10. WUS use case C: Wake up signal for CUI detection

When channel usage indicator is used across different NR-U nodes for better reuse, the CUI transmission can be asynchronous as different NR-U nodes may not be time aligned, especially when they are from different operators. Then to detect the CUI from other nodes, we will need to recover the timing for CUI transmission, assuming CUI is using OFDM waveform. The WUS can serve that purpose. The WUS can be detected in asynchronous fashion and the WUS can provide the timing needed for the CUI detection.

For the waveform for WUS, we need to keep in mind that the waveform should support low power consumption detection. We can consider NR-PSS/SSS type waveform to reuse the PSS/SSS detection mechanism already available in NR UE. However, the NR-PSS/SSS waveform is relatively narrow band, and in some unlicensed bands, the PSD limitation enforced by regulators may not allow the NR-PSS/SSS type waveform to be transmitted with full power.

Alternatively, wideband waveform that support auto-correlation based detection can be supported. The wideband waveform supports full power transmission, and the auto-correlation based detection typically can be supported with lower power consumption

Proposal 7. Further study the design and usage of wake-up signal in NR

For the waveform for WUS, we will need a design that supports low complexity detection, time domain detection (if asynchronous detection is needed), and reasonably good coverage. The following two options can be considered:

- Narrow band waveform like what is used for PSS

- Wideband waveform formed by repeating short sequence, like the waveform used by WiFi STF

For the narrow band option, if a similar sequence like PSS is used, we can most likely reuse the design of PSS detection. In hardware implementation, the same circuit and algorithm for PSS detection can be reused as well. The waveform is relatively narrow band, so the complexity of cross-correlation based time domain detection is relatively low. A downside of this design is the transmission under PSD limitation. In that case, due to the narrow band nature of the signal, the transmit power of the waveform can be limited which will limit the coverage of the waveform.

On the other hand, if wideband repeated short sequence is used, since the signal is wideband, the transmission under PSD limitation will not be a problem. However, since the sampling rate of the waveform is high, cross-correlation based time domain detection will be of high complexity. Instead, typically an auto-correlation based detector is used which has much lower complexity but comes at expense of degraded performance when compared with a cross-correlation based solution.

Both options have their pros and cons. We propose to study them both and pick one.

Proposal 8. For the waveform for WUS, consider PSS/SSS like waveform and wideband repeated short sequence type waveform.

In addition, if the WUS can be designed to be technology neutral, it can be understood by nodes with different RAT to achieve better coexistence.

Proposal 9. A technology neutral WUS can be considered for improved coexistence between different RAT.

# Summary

In this section, we summarize the DL waveform design options for NR-U in SUB-7GHZ BAND along with the performance evaluation. We have the following observations and proposals.

Observation 1: SCS for SSB in NR-U SUB-7GHZ BAND should consider the trade-off on the performance, implementation complexity, signaling change on top of NR as well as the LBT and factory automation application. It is beneficial to support both 30Khz SCS and 60Khz SCS for SSB transmissions.

Proposal 1: Introduction of SSB transmission with 60Khz SCS in NR-U on top of 30Khz SCS. A node is provisioned to use a given SCS for SSB acquisition.

Proposal 2: Allow FDM between RMSI CORSET/PDSCH and SSB.

Proposal 3: Reuse NR SSB pattern for 30Khz SCS SSB for NSA and SA operation.

Proposal 4: Consider cyclic extended SSB transmission in NR-U

Proposal 5: Consider interlace waveform for NR-U DL in SUB-7GHZ BAND for better coverage and interference coordination

Proposal 6: Reuse NR PDCCH waveform for CUI transmission from both gNB and UE.

Proposal 7. Further study the design and usage of wake-up signal in NR

Proposal 8. For the waveform for WUS, consider PSS/SSS like waveform and wideband repeated short sequence type waveform.

Proposal 9. A technology neutral WUS can be considered for improved coexistence between different RAT.

# References

[1] RP-172021, Revised SID on NR-based Access to Unlicensed Spectrum, Qualcomm Incorporated, RAN#77

[2] ETSI EN 301 893 V2.1.1

[3] R1-1809476, “Frame Structure for NR-U Operation”, RAN1 #94, Qualcomm Incorporated

[4] R1-1809479, “Channel access procedures for NR unlicensed”, RAN1 #94, Qualcomm Incorporated

[5] RP-181339, “Revised SID on NR-based Access to Unlicensed Spectrum”, Qualcomm Incorporated, RAN#80, La Jolla, USA, June 11-14, 2018.