3GPP TSG-RAN WG1 Meeting #94	R1-18xxxxx

Gothenburg, Sweden, August 20st – 24th 2018

Agenda Item:	7.1.3.2

Source:	Ericsson

Title:	Summary of Maintenance Issues on Short PUCCH and UCI Multiplexing

Document for:	Discussion and Decision

# Introduction

This contribution summarizes the maintenance issues on UL control channel with the focus on the short PUCCH design as well as UCI multiplexing due to overlapping of PUCCH resources.

It is important to note that some of the maintenance issues are already addressed in the proposed update of TS 38.213.

Recommendation: - Review the TPs in [1] and [2] corresponding to subclauses 9.1 and 9.2 of TS 38.213, respectively, for possible agreement.

# Proposed corrections to TS 38.211

## Sequence and Group hopping

HW raises the issue that the higher-layer parameter hoppingId, used to determine the group or sequence hopping pattern for a PUCCH transmission is an optional parameter in TS38.331. If the hoppingId is not configured, the default value is also needed. The following is proposed to consider  as the default value.

Proposal 1: Adopt the following TP in TS 38.211, subclause 6.3.2.2.1 and 6.3.2.2.2:

--------------------------------- Start of text proposal to TS 38.211 ------------------------------------------ 6.3.2.2.1 Group and sequence hopping The sequence group and the sequence number within the group depends on the higher-layer parameter pucch-GroupHopping: - if pucch-GroupHopping equals 'neither' where is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, . - if pucch-GroupHopping equals 'enable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, . - if pucch-GroupHopping equals 'disable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, .. <Unchanged text is omitted> 6.3.2.2.2 Cyclic shift hopping <Unchanged text is omitted> The function is given by where the pseudo-random sequence is defined by subclause 5.2.1. The pseudo-random sequence generator shall be initialized with , where is given by the higher-layer parameter hoppingId, if the higher-layer parameter hoppingId is configured; otherwise, .. --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

## Extended CP for PUCCH format 2

HW suggests corrections in TS 38.211 were s for usage of PUCCH format 2 for extended CP and the following is proposed:

Proposal 2: Adopt the following TP in TS 38.211, subclause 6.3.2.2.2 and 6.4.1.3.2.1:

--------------------------------- Start of text proposal to TS 38.211 ------------------------------------------ 6.3.2.2.2 Cyclic shift hopping <Unchanged text is omitted> The function is given by where the pseudo-random sequence is defined by subclause 5.2.1. The pseudo-random sequence generator shall be initialized with , where is given by the higher-layer parameter hoppingId. <Unchanged text is omitted> 6.4.1.3.2.1 Sequence generation The reference-signal sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2 and is given by clause 6.3.2.5.2. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within the radio frame, and NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and by NIDcell otherwise. <Unchanged text is omitted> --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

## Scrambling for PUCCH formats 2/3/4

MTK discusses that for PUCCH format 2/3/4, cinit is required to generate scrambling sequence, and is a function of nRNTI and nID, as shown in the below (Section 6.3.2.5.1 of 38.211). Determination of nRNTI is missing.

It is suggested to adopt C-RNTI for this purpose, similarly to LTE as the following is proposed:

Proposal 3: C-RNTI is used for nRNTI of cinit to generate the scrambling sequences for PUCCH format 2/3/4.

## DMRS for PUCCH format 2

In Samsung discusses that for DM-RS generation based on PUCCH format 2 as described in section 6.4.1.3.2.1 of TS 38.211, the pseudo-random sequence  to generate DM-RS is initialized with cinit using NID0 as a parameter where NID0∈0,1,…,65535 is provided by higher-layer parameter scramblingID0 in DMRS-UplinkConfig IE if provided. When two “DMRS-UplinkConfig”s are provided by dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, respectively, one issue to clarify is how a UE selects a scramblingID0 from the two “scramblingID0”s. In TS 38.212, section 7.3.1.1.2, a procedure to determine the bitwidth for antenna ports in DCI format 0_1 is specified if a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. There are two alternatives. The first is to choose one of the two parameters. That is, to obtain scramblingID0 either from dmrs-UplinkForPUSCH-MappingTypeA or from dmrs-UplinkForPUSCH-MappingTypeB. The second is to determine scramblingID0 depending on the starting symbol position for the PUCCH with PUCCH format 2. For example, if a starting symbol is symbol 0 (or the first UL symbol), scramblingID0 in dmrs-UplinkForPUSCH-MappingTypeA is selected; otherwise, scramblingID0 in dmrs-UplinkForPUSCH-MappingTypeB is selected. The first alternative is simple while the second alternative follows same principles as for TypeA and TypeB PUSCH. The following is proposed by Samsung to clarify the behavior, emphasizing that either alternative can be adopted as this is not a critical issue.

Proposal 4: For PUCCH format 2 when a UE is provided both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, scramblingID0 is provided by either (a) one of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB (down-select), or (b) dmrs-UplinkForPUSCH-MappingTypeA if the first PUCCH symbol is the first UL symbol in the slot; otherwise, dmrs-UplinkForPUSCH-MappingTypeB.

## Frequency hopping

In previous meeting, the following agreement was made:

Agreements:

- In 6.3.2.4.1 of TS38.211,

  - No intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “disabled”.

  - Intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “enabled”.

  - Add the following:

    - The channel over which a symbol on the antenna port used for UL transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to the same frequency hop, when frequency hop is enabled, irrespective of whether the frequency hop distance is zero or not.

Regarding the corresponding text in TS 38.211, [Panasonic], proposes the following modifications to clarify that the description is from the Tx side (i.e. UE).

Proposal 5: In TS38.211 Section 6.3.2.4.1, adopt the following text proposals.

--------------------------------- Start of text proposal to TS 38.211 ------------------------------------------ 6.3.2.4.1 Sequence modulation <Unchanged text is omitted> In case of intra-slot frequency hopping enabled, regardless of whether the frequency-hop distance is zero or not, UE is allowed to transmit signal whose the phase of the transmitted signal in the two hops is may be uncorrelated.” <Unchanged text is omitted> --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

Moreover, DCM claims that the current description in TS 38.211 is not enough. Similar ambiguity of OCC configuration remains at 6.4.1.3.1.1 in 38.211 for DMRS in PUCCH format 1; when the higher-layer parameter intraSlotFrequencyHopping is provided and secondHopPRB is the same as startingPRB, it is unclear which OCC configuration of “intra-slot hopping” or “no intra-slot hopping” is used. To avoid the ambiguity between gNB and UE, the same sentence as that at 6.3.2.4.1 in 38.211 needs to be added where DCM proposes the following TP.

Proposal 6: Apply the following TP for section 6.4.1.3.1 in TS 38.211.

-------------------------------- Start of text proposal to TS 38.211 ------------------------------------------ ##### 6.4.1.3.1 Demodulation reference signal for PUCCH format 1 ###### 6.4.1.3.1.1 Sequence generation The reference signal sequence is defined by where is given by Table 6.4.1.3.1.1-1. Intra-slot frequency hopping shall be assumed when the higher-layer parameter intraSlotFrequencyHopping is provided, regardless of whether the frequency-hop distance is zero or not, otherwise no intra-slot frequency hopping shall be assumed. The sequence is given by clause 5.2.2. <Unchanged text is omitted> --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

Finally, both Panasonic and DCM propose for the replacement of the corresponding text where Panasonic suggests (Proposal 7a) to be moved to section 6.3.2.1 to be applicable for all the PUCCH formats while DCM suggests (Proposal 7b) the description to be moved to section 4.4.1 to be applicable for any UL transmission while.

Proposal 7a: The above text proposal in TS38.211 Section 6.3.4.1, is moved to Section 6.3.2.1 in order to apply all intra-slot frequency hopping of PUCCH.

Proposal 7b: Adopt the following TP in TS 38.211 section 4.4.1.

------------------------------- Start of text proposal to TS 38.211 ------------------------------------------ ### 4.4.1 Antenna ports An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. When intra-slot frequency hopping is not configured in higher layer parameter for a channel, the channel over which a symbol on the antenna port used is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. When intra-slot frequency hopping is configured in higher layer parameter for a channel, the channel over which a symbol on the antenna port used is conveyed can be inferred from the channel within a hop over which another symbol on the same antenna port is conveyed. Unchanged text is omitted> --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

## Editorial corrections

ZTE discusses that a typo is in Subclause 6.4.1.3.2.1 of TS 38.211 is identified where  needs to be modified to .

Proposal 8: In Subclause 6.4.1.3.2 of TS 38.211, replace  with .

# Proposed corrections to TS 38.213

## Pseudo code for UCI multiplexing

In RAN1#93 meeting, the following agreement was made to replace the original Step 1 agreed to under the UCI multiplexing agenda.

Agreements for new step1:

- For step 1, while there are overlapping PUCCH resources in a slot,

  - The UE determines the PUCCH resource (resource A) that overlaps with at least another PUCCH resource with the priority order of earliest followed by largest duration (if applicable)

    - In case of multiple candidates for resource A, the UE can pick one (implementation-specific)

  - The UE determines a set of PUCCH resources (resource set X) overlapping with PUCCH resource A.

  - The UE determines a PUCCH resource and corresponding UCI for multiplexing the PUCCH resource A and PUCCH resources in set X in one shot.

  - The determined PUCCH resource and the corresponding UCI replace resource set X and resource A

- For step 1, when there is no more overlapping PUCCH resources in the slot, the UE determines “the timeline requirements for overlapping UL channels”. If the requirement is met, the UE proceeds to Step 2.

- Note: the above is per PUCCH group

The above agreement is to replace Step 1 in the agreements under 7.1.3.2.3

UCI multiplexing in Step 1 for determining non-overlapping PUCCH resources is described in subclause 9.2.5 in TS 38.213 by a pseudo code.  Couple of companies (CATT, LG, Spread spectrum, Nokia) proposed changes to the current pseudo code. It is not clear whether changes are needed or not. Therefore, discussion is necessary to reach to a common understanding.

Proposal 9: Discuss whether the following change to the pseudo code for UCI multiplexing described in subclause 9.2.5 in TS 38.213 is needed.

9.2.5 UE procedure for reporting multiple UCI types --------------------------------- End of text proposal to TS 38.211 ------------------------------------------ <Unchanged text is omitted> while if any of resources Q(j-o) overlaps with resource else Unchanged text is omitted> --------------------------------- End of text proposal to TS 38.211 ------------------------------------------

## Overlapping with SR PUCCH resource

Few companies have discussed the ambiguity cases that may happen due to overlapping of SR resource to CSI resources.

- One SR resource overlapping with two non-overlapping CSI resources

It was agreed that when SR overlaps with CSI, SR shall be multiplexed with CSI and transmitted on CSI resource. For the case that one SR resource overlaps with more than one non-overlapping CSI PUCCH resources showed in the figure below Vivo discusses the following alternatives where the first one is also proposed by MTK discussing the same issue:

When one SR PUCCH resource overlaps with two non-overlapping CSI PUCCH resources in a slot,

- Alt 1: SR is multiplexed and transmitted with CSI on the first CSI PUCCH resource.

- Alt 2: SR is multiplexed and transmitted on each CSI PUCCH resource.

- Alt 3: Choose the smaller code rate PUCCH resource to multiplex SR and CSI. If these two CSI PUCCH resources are configured with a same code rate, the higher priority CSI PUCCH resource is selected.

- Alt 4: This is an error case due to poor configuration. No need for specification.

Example for one SR PUCCH overlaps with two CSI PUCCH

Proposal 10: Discuss whether clarification on UE behavior is needed when a SR resource overlaps with two non-overlapping CSI resources.

- SR ID in overlapping case

CATT discusses that following the current multiplexing rule for HARQ-ACK and SR using PUCCH Format 1, HARQ-ACK will be transmitted on both SR1 resource and SR2 resource for Case 1 in figure below  HARQ-ACK is Resource A but for Case 2, HARQ-ACK is only transmitted on SR1 resource since Resource A = SR1.To avoid redundant transmission and also for unified behavior regardless of starting position of HARQ-ACK, we prefer that HARQ-ACK is only transmitted on the first SR resource.

HARQ-ACK/CSI overlapping with two SR PUCCHs corresponding to different SR configurations

For HARQ-ACK/CSI using PUCCH Format 2/3/4, the UE will multiplex both SR on the HARQ-ACK/CSI resource for both Case 1 and Case 2 in Figure 3. As agreed, the UE multiplexes  = 2 bits on the HARQ-ACK/CSI resource. Since only one positive SR can be sent per RAN1 agreement, it should be left to UE implementation to determine which of SR1 or SR2 is transmitted if both SR1 and SR2 are positive.

Proposal 11: Discuss whether the following clarification on UR behavior is needed.

- For HARQ-ACK/CSI overlapping with PUCCH resources corresponding to different SR configurations

- When HARQ-ACK and SR use PUCCH Format 1 and the SR resources do not overlap, HARQ-ACK is transmitted on the first SR PUCCH resource

- When HARQ-ACK/CSI uses PUCCH Format 2/3/4 PUCCH resource and the SR PUCCH resources do not overlap,  SR bits are transmitted on the HARQ-ACK/CSI resource. In case of multiple positive SR indications, it is up to UE implementation which SR configuration is indicated as positive in the PUCCH transmission.

## Overlapping with HARQ-ACK PUCCH resource for DL SPS

### DL SPS HARQ-ACK PUCCH and CSI PUCCH

Intel discusses that when UE is not provided high layer parameter multi-CSI-PUCCH-ResourceList, and when SPS HARQ-ACK PUCCH resource overlaps with two non-overlapping CSI PUCCHs, it is unclear which PUCCH resource is utilized to carrying the combined UCI.

Multiplexing of SPS HARQ-ACK/SR and CSI

Figure above illustrates the multiplexing of SPS HARQ-ACK/SR with multiple CSI reports on PUCCH. To simplify the multiplexing rule, one solution is to multiplex the SPS HARQ-ACK/SR on CSI with highest priority and drop the CSIs with lower priority. In other words, when UE is not provided high layer parameter multi-CSI-PUCCH-ResourceList, and when SPS HARQ-ACK PUCCH resource overlaps with two non-overlapping CSI PUCCHs, CSI report and corresponding PUCCH resource with highest priority is selected.

Proposal 12: Discuss whether the following clarification on UE behavior is needed.

- When UE is not provided high layer parameter multi-CSI-PUCCH-ResourceList, and when SPS HARQ-ACK PUCCH resource overlaps with two non-overlapping CSI PUCCHs, CSI report and corresponding PUCCH resource with highest priority is selected.

### Overlap between DL SPS HARQ-ACK PUCCH and other configured resources

OPPO discusses that when ACK/NACK for SPS PDSCH is multiplexed with SPS-PUSCH/SP-CSI/P-CSI/SR in slot n, if both following conditions are satisfied:

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than symbol N1+X after the last symbol of SPS-PDSCH

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the beginning of SPS-PDSCH.

Otherwise, UE behavior in slot n is not specified.

ACK/NACK for SPS PDSCH + semi-statically configured PUCCH or PUSCH

Proposal 13: Discuss whether the following clarification on UE behavior is needed.

- When ACK/NACK for SPS PDSCH overlaps with SPS-PUSCH/SP-CSI/P-CSI/SR in slot n, the conditions for UCI multiplexing are:

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than symbol N1+X after the last symbol of SPS-PDSCH(s)

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the beginning of SPS-PDSCH.

### Overlap between any combination of SR, SPS-PUSCH, SP-CSI and P-CSI

OPPO discusses that when the UCIs/UL data are multiplexed in one PUCCH or PUSCH in slot n, using the existing UCI multiplexing rule, if following condition is satisfied:

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the beginning of the last PUCCH(s)/PUSCH(s) among all the overlapping channels in the last transmission.

Otherwise, UE behavior in slot n is not specified.

SR + SPS-PUSCH/SP-CSI/P-CSI or SPS-PUSCH + SP-CSI/P-CSI

However, it was discussed in the previous meeting that the timeline requirement for UCI multiplexing is relevant to the resources that overlap with at least one dynamically scheduled resource,

Proposal 13: Discuss whether the following clarifications for UE behavior is needed.

When SR overlaps with SPS-PUSCH/SP-CSI/P-CSI, or SP-CSI/P-CSI overlaps with SPS-PUSCH in slot n, the conditions for UCI multiplexing are:

- If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the beginning of the last PUCCH(s)/PUSCH(s) among all the overlapping channels in the last transmission.

# References

R1-1809707	Specification Update on HARQ-ACK Codebooks Incorporating Proposals in RAN1#94 Samsung

R1-1809707	Specification Update on UL Control Incorporating Proposals in RAN1#94	Samsung

R1-1808103	Remaining issues for physical uplink control channel	Huawei, HiSilicon

R1-1808207	Remaining issues on NR PUCCH	ZTE

R1-1808225	Remaining issues on PUCCH	vivo

R1-1808260	Remaining issues of PUCCH	MediaTek Inc.

R1-1808313	Corrections on Subclause 9.2 UCI reporting in PUCCH in TS 38.213	Fujitsu

R1-1808363	Remaining issues on PUCCH	NEC

R1-1808379	Corrections to UCI feedback procedures	CATT

R1-1808491	Correction on PUCCH and UCI multiplexing for NR	LG Electronics

R1-1808620	Remaining issues on physical uplink control channel	Panasonic Corporation

R1-1808673	Remaining details on NR PUCCH	Intel Corporation

R1-1808754	Remaining Issues on PUCCH	Samsung

R1-1808802	Remaining issues on UCI multiplexing	Spreadtrum Communications

R1-1808812	Remaining issues about PUCCH	ETRI

R1-1808893	Remaining issues for UL control	Guangdong OPPO Mobile Telecom

R1-1808966	Remaining details on NR Physical UL Control Channel	Nokia, Nokia Shanghai Bell

R1-1809109	Remaining issues on PUCCH	Sharp

R1-1809142	Maintenance for physical uplink control channel	NTT DOCOMO, INC.

R1-1809321	Remaining issues on PUCCH	WILUS Inc.

R1-1809406	Maintenance issues of physical uplink control channel	Ericsson

R1-1809427	CRs for PUCCH	Qualcomm Incorporated