3GPP TSG-RAN WG1 Meeting #94	R1- 1809989

Gothenburg, Sweden, August 20th – 24th, 2018

Agenda item:	7.1.7

Source: 	Qualcomm Incorporated

Title: 	UE UL beam capability

Document for:	Discussion/Decision

The proposal is to include UE UL beam capability in the following rows into the UE capability list.

| 2-20a | Configured spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS |  | Yes | Only one configured spatial relation for UL signals can be supported | Type 1 | N.A. | N.A. |  | Note: Only applicable for FR2 |  | candidate value set: {4, 8, 16, 32, 64, 96} |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-20b | Active spatial relations | Maximum number of active spatial relations with regarding to PUCCH and SRS for PUSCH, per BWP per CC NOTE: ‘active spatial relation’ refers to the finally selected spatial relation for each PUCCH resource or SRS resource for PUSCH via MAC-CE or RRC. | 2-20a | Yes | Only one active spatial relation for UL control and data can be supported | Type 1 | N.A. | N.A. |  | Note: Only applicable for FR2 FFS whether DL RS in the active TCI states and the active spatial relation info can be different if 2-20b is set to 1 and if different whether or not new UE behavior is needed |  | Candidate value set: {1, 2, 4, 8, 14} |  |
| 2-20c | Additional active spatial relation for PUCCH | Support one additional active spatial relation for PUCCH | 2-20b | Yes | No additional active spatial relation for control | Type 1 | N.A. | N.A. |  | Note: Only applicable if 2-20b is set to 1 |  | Mandatory with capability signaling | Mandatory with capability signaling |