3GPP TSG RAN WG1 Meeting #96 R1-1902233 Athens, Greece, February 25th – March 1st, 2019 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.213; CR; rev; Current version:; 15.4.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on PDCCH monitoring for SCell. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-25. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; TS38.213 v15.4.0 defines a default UE behavior on PDCCH monitoring when there is ambiguty on TCI state for the configured CORESET as follows: - For a CORESET other than a CORESET with index 0, if a UE has not been provided with a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has received initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure. Assuming that PDCCH DMRS is quasi co-located with the SS/PBCH block identified during the initial access procedure the above text is generally not applicable for the SCell since the PDCCH DMRS in the CORESET in SCell is quasi co-located with the SS/PBCH transmitted in SCell. When the TCI state is not explictly informed to the UE, the UE may keep doing PDCCH monitoring with mismatched QCL assumption. Therefore, a UE behavior on PDCCH monitoring for the case of ambiguous TCI state for the CORESET in SCell should be clarified.. Summary of change:; Describe UE behavior on PDCCH monitoring when there is ambiguty on TCI state for the configured CORESET for SCell.. Consequences if not approved:; Incomplete specifications for PDCCH monitoring If the UE is implemented according to the change but the NW is not, - When more than one TCI states are configured but one of the TCI states has not yet been activated for a CORESET in SCell, the NW may transmit PDCCH via the CORESET, however the UE may not decode the PDCCH. The NW may consider the UE’s link quality is poor, so that will trigger another step of operation such as link adaptation for PDCCH, BFR, RACH, etc. As a consequence, the system performance is degraded. If the NW is implemented according to the change but the UE is not, - When more than one TCI states are configured but one of the TCI states has not yet been activated for a CORESET in SCell, the NW may not transmit PDCCH via the CORESET, however the UE may try to decode the PDCCH. As a consequence, UE power consumption is increased. In addition since DCI is not detectable, the radio link failure can be triggered by UE.. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

For the PCell and PSCell and for a CORESET other than a CORESET with index 0, if a UE has not been provided with a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has received initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure.

For the PCell and PSCell and for a CORESET with index 0, if a UE has not received a MAC CE activation command for a TCI state, as described in [11, TS 38.321], the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with a SS/PBCH block the UE identified either during initial access or, if any, from a most recent random access procedure not initiated by a PDCCH order that triggers a non-contention based random access procedure.

For a SCell and for any CORESET, if a UE has not been provided with a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has received initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE does not expect to monitor PDCCH candidates in the CORESET.

*** Unchanged text is omitted ***