3GPP TSG RAN WG1 Meeting #96               	                         R1-1902803

Athens, Greece, February 25th –March 1st, 2019

Agenda item:	7.2.4.5

Source: 	NTT DOCOMO, INC.

Title:	Updated views on LTE and NR sidelink coexistence

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on co-existence of LTE V2X sidelink (SL) and NR V2X SL.

Discussion

In the context of NR-LTE co-existence discussion, the following aspects are considered.

- For FDM solutions

  - Inter-band co-existence or intra-band co-existence including the extent of frequency separation

  - Semi-static power sharing and dynamic power sharing

  - Tx-Tx, Tx-Rx, and Rx-Rx

- For TDM solutions

  - Short-term coordination and long-term coordination within a UE

  - Tx-Tx, Tx-Rx, and Rx-Rx

- Handling of NW involvement

We describe our views on the above remaining aspects.

FDM

At the 3GPP RAN1#1901 ad-hoc meeting, the following agreements on FDM solutions were made.

Agreements: - Inter-band FDM Solutions for coexistence - For static power assignment of Pc,max for each carrier - Synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink. - This FDM solution is feasible for resolution of Tx/Tx coexistence conflicts - If the band separation is large enough (based on RAN4 indication), then this FDM solution for coexistence is feasible for Tx/Rx coexistence - If the band separation is NOT large enough, then this FDM solution is not feasible for resolution of Tx/Rx coexistence conflicts - For dynamic power sharing between carriers, - FFS details of FDM solutions and whether they are feasible

According to the above agreements in RAN1#1901 ad-hoc meeting, primary issue on in-device FDM seems only on power sharing between LTE SL and NR SL. For semi-static power sharing, the frequency separation affects the feasibility of Tx/Rx coexistence, which is, however, considered to be out of RAN1 expertise.  Regarding dynamic power sharing, if short-term coordination where dynamic information exchange between NR and LTE within the UE is performed as is the case with short-term scale TDM solutions, we consider dynamic power sharing would be also feasible and NR Uu EN-DC power sharing mechanism can be reused. However, some prioritization rules (e.g., prioritizing LTE over NR) can be revisited since service types would be important rather than RAT types. In addition, specification impact, e.g., how to support different numerologies, and potential workload in other WGs should be assessed.

Observation 1: As is the case with dynamic power sharing for EN-DC, FDM solution with dynamic power sharing for V2X in-device co-existence is considered to be feasible.

Observation 2: NR Uu EN-DC power sharing can be considered as the baseline solution for FDM solution with dynamic power sharing. V2X SL specific manner, e.g., prioritization rule between LTE and NR, can be revisited.

TDM

At the 3GPP RAN1#1901 ad-hoc meeting, the following agreements on TDM solutions were made.

Agreements: - For long term time scale TDM solutions for in-device coexistence between LTE and NR V2X: - For a UE with coexistence impact, non-overlapping (in time domain) resource pools are (pre-)configured for NR V2X and LTE V2X sidelinks - No information is exchanged between LTE and NR sidelinks within the UE - Long term time scale TDM solution is feasible from RAN1 point of view - Note: although feasible, it is expected that such a solution may have impact on latency, reliability and data rate requirements for some applications - No additional modifications to LTE specifications are needed Agreements: Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X, - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement

According to the agreements made in RAN1#1901 ad-hoc meeting, major issues were resolved for long-term coordination. For short-term TDM solutions, when TX/TX or TX/RX for NR and LTE with SPS scheduling happens, it was agreed that one RAT is prioritized over the other RAT. How to prioritize, e.g., UE implementation, remains unresolved. For each occurrence of Tx/Tx overlap, it would be beneficial to support (pre)configuration of prioritization rule in the specification based on RATs or service types such as URLLC or non-URLLC.  Detailed prioritization rule can be considered during WI phase. For the case of Tx/Rx overlap, in addition, it should be considered whether determination of priority for Rx operation is feasible or not. There would be some cases where determination of priority for Rx operation is feasible. For such cases, it would be beneficial to set priority rules in the specification. However, there are also other cases where determination of priority for Rx operation is not feasible. For example, UE does not know if the currently receiving data should be prioritized or not until UE completes the decoding. In this case, prioritization rule may not work and  UE implementation based approach should be considered. If RAN (WG) is tasked to set prioritization rule, RAN (WG) should consider it during work item phase.

Observation 3:

- For short-term TDM solutions for in-device coexistence

  - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another based on (pre-)configured prioritization rule. Details can be considered during WI phase.

  - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another by UE implementation. However, if RAN (WG) is tasked to set prioritization rule, RAN (WG) would consider it during work item phase.

NW involvement

There is an argument that it is useful to report a resource pool of a RAT, e.g., NR, to the NW of the other RAT (LTE). This reporting mechanism is beneficial particularly when NR NW is not available and NR is used only for side-link communication. In this case, eNB can schedule LTE SL resource such that it is not overlapped with NR SL resource. On the other hand, we can also assume that the LTE NW would know (pre)configured NR SL resource pool information in advance.

Observation 4: Reporting information on SL resource pool of a RAT to the other RAT NW would be beneficial. However, the usage scenarios should be further investigated during WI phase.

Conclusion

In this contribution, we had discussion on LTE and NR V2X SL coexistence. Based on the discussion, we made the following observations:

Observation 1: As is the case with dynamic power sharing for EN-DC, FDM solution with dynamic power sharing for V2X in-device co-existence is considered to be feasible.

Observation 2: NR Uu EN-DC power sharing can be considered as the baseline solution for FDM solution with dynamic power sharing. V2X SL specific manner, e.g., prioritization rule between LTE and NR, can be revisited.

Observation 3:

- For short-term TDM solutions for in-device coexistence

  - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another based on (pre-)configured prioritization rule. Details can be considered during WI phase.

  - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another by UE implementation. However, if RAN (WG) is tasked to set prioritization rule, RAN (WG) would consider it during work item phase.

Observation 4: Reporting information on SL resource pool of a RAT to the other RAT NW would be beneficial. However, the usage scenarios should be further investigated during WI phase.

Proposal: Capture the above observations in TR38.885.

- Reference

[1] Chairman’s note, RAN1 ad-hoc#1901, January 2019