3GPP TSG RAN WG1 Meeting #96	R1-1903103

Athens, Greece, 25th February - 1st March, 2019

Agenda Item:	7.2.8.6

Source:	Huawei, HiSilicon

Title:	Evaluation results of full power transmission for UL MIMO with multiple PAs

Document for:	Discussion and Decision

Introduction

In the last meeting, the main scope for enhancement to allow full power UL transmission was agreed [1]:

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs. The support of this feature is indicated by the UE as part of UE capability signalling. For power class 3:

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

  - FFS: detailed power scaling description

  - Note: Full Tx power means UE delivers total power of 23dBm for PC3

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

  - FFS: detailed design

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

FFS: Whether all three capabilities will be specified or a subset will be specified

FFS: UE capability signalling/reporting details

Note: Two or more of the above capabilities could be merged depending on the further details

Agreement

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

- FFS: Whether this applies for the entire codebook or subset of codebook

Several options or combination of the options from the last two meetings listed for further discussion:

- Option 1: Refinement/adjustment of UL codebook is supported

  - 1-1: Support a new codebookSubset for non-coherent and partial-coherent transmission capable UEs

  - 1-2: Introduce additional scaling factor for uplink codebook

- Option 2: UE transparently apply a small cyclic or linear delay

- Option 3: Power control mechanism to be modified to support UL full power transmission without precluding the use of full rated PA(s)

  - Note: Full rated PA refers to a PA having power not lower than that of the power class

- Option 4: Up to UE implementation with UE capability signalling of full power transmission in UL

- Option 5: For the precoders with 0 entries, the linear value PPUSCH,b,f,ci,j,qd,l of a PUSCH transmission power is scaled by a ratio Rel-16. The value of Rel-16 is selected up to UE implementation within the range of [Rel-15, 1],  where Rel-15 is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of configured antenna ports for the PUSCH transmission scheme as defined in NR Rel-15 specification.

  - UE is required to maintain consistent Rel-16 value on different occasions of PUSCH transmissions with the same precoder for PUSCH

In this paper, we compared potential solutions.

Performance on potential solutions

In order to let partially-coherent / non-coherent UE achieves the same maximum transmit power as fully-coherent UE, full power transmission should be supported for partially-coherent / non-coherent UE especially for the coverage limited cases. In [2], we analyse the agreed options and provide two alternatives for addressing the full power issue for PUSCH transmission:

- Alt-1: Combining Option 2 and Option 1-1

- Alt-2: Combining Option 3 and Option 1-2

For Alt-1, non-coherent and partial coherent codewords are replaced by coherent codewords, and small delay CDD transmission is applied when these coherent codewords are indicated. For Alt-2, part of the antennas are used to support full power utilization for PUSCH transmission.

Based on the above discussion, we provide the performance comparison between the two alternatives as mentioned above. The simulation assumes that UE has 2Tx with non-coherent capability and PUSCH transmission is fixed to rank-1, and more detailed simulation assumptions can be found in the appendix. And we also provide the evaluations for the case considering the modelling of UE’s one antenna blockage.

In Figure 1 which is based on the case without antenna blocking, we can see that both of Alt-1 and Alt-2 provide substantial gain over current Rel-15 non full power PUSCH transmission mechanism since UE can use more power than Rel-15 for PUSCH transmission via either Alt-1 or Alt-2.

In the case that both of the antennas are not blocked, Alt-2 with full power transmission in single antenna is slightly better than Alt-1 with small delay CDD on two antennas. The reason is that all the transmission power is concentrated to the optimal antenna which is selected by gNB based on the SRS measurement for Alt-2, while for Alt-1, the transmission power is scaled equally to all the antennas to support full power transmission.

Figure 1. Performance of the candidate alternatives without blocking.

Observation 1: Alt-2 with full power transmission in single antenna is slightly better performance than Alt-1 with small delay CDD on two antennas, and both them much better than non-full power case.

However, as shown in Figure 2, in the case that one antenna is blocked, there is obvious performance loss of Alt-1 compared to Alt-2. The reason is that each of the antennas cannot utilize the maximum power in Alt-1, when one antenna is blocked, only half of the maximum power can be achieved. Meanwhile, for Alt-2, the antennas that are not blocked for the PUSCH transmission can be selected for PUSCH transmission and full power on the selected antennas can still be utilized. Therefore, Alt-2 outperforms Alt-1. Based on the evaluation results and discussion, at least for the UE with full power related PAs, Alt.2 should be supported to enhance UL coverage.

Figure 2. Performance of the candidate alternatives with blocking.

Observation 2: Alt-2 with full power transmission in single antenna is much better performance than Alt-1 with small delay CDD on two antennas in the blockage cases, i.e., 2dB difference.

Summary of discussions

Based on the above discussion, we have the following observations:

Observation 1: Alt-2 with full power transmission in single antenna is slightly better performance than Alt-1 with small delay CDD on two antennas, and both them much better than non-full power case.

Observation 2: Alt-2 with full power transmission in single antenna is much better performance than Alt-1 with small delay CDD on two antennas in the blockage cases, i.e., 2dB difference.

References

3GPP, “RAN1#95 Meeting Chairman’s Notes”, Spokane, USA, November 12th – 16th, 2018.

R1-1900019, Enhancement on UL MIMO with multiple PAs to allow full power transmission, Huawei, HiSilicon.

- Appendix A. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2GHz |
| Subcarrier spacing | 15kHz |
| Delay spread | 100ns |
| System band | 10MHz |
| Channel model | TDL |
| UE speed | 3km/h |
| UE antenna configurations | 2Tx |
| UE antenna pattern | Omnidirectional |
| BS antenna configurations | 2Rx |
| Link Adaptation | Enable |
| Receiver | MMSE |